3GPP TSG-RAN4 Meeting #105	R4-2218671

# Toulouse, France, Nov 14 - Nov 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | - | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on RRC re-establishment and timing requirement for eMTC UE in IoT-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2022-11-1 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has agreed to introduce RRC re-establishment and timing requirement for eMTC UE for LTE IoT-NTN in R4-2214350. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new sections for requirements for LTE IoT NTN eMTC UE RRC re-establishment and timing requirement in both GSO and NGSO scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will not be defined for LTE IoT-NTN UE RRC re-establishment and timing requirement in both GSO and NGSO scenario. The WI can’t be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7A(New) 7.24A(New) 7.27A (New) 7.28A (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change#1>

6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access

6.7A.1	Introduction

RRC connection re-establishment is initiated when a Cat-M1 UE either configured with CEModeA or CEModeB in RRC connected mode looses RRC connection due to any of these reasons: radio link failure or radio link problem. The RRC re-establishment procedure is specified in clause 5.3.7 in TS 36.331 [2]. FFS: If the UE is configured with ‘t-Service-r17’ [2] in the serving cell, for LEO RRC connection re-establishment shall be initiated before ‘t-Service-r17’ [2].

The requirements in this clause are applicable for RRC connection re-establishment to E-UTRAN, which is served by satellite access node (SAN).

6.7A.2	Requirements

In RRC connected mode the UE shall be capable of sending RRCConnectionReestablishmentRequest message within Tre-establish_delay seconds from the moment it detects a loss in RRC connection.  The total RRC connection delay (Tre-establish_delay) shall be less than:

Tre-establish_delay =  TUL_grant + TUE_re-establish_delay

TUL_grant: It is the time required to acquire and process uplink grant from the target cell. The uplink grant is required to transmit RRCConnectionReestablishmentRequest message.

The UE re-establishment delay (TUE_re-establish_delay) is specified in clause 6.7A.2.1 for a UE configured with CEModeA and in clause 6.7A.2.2 for a UE configured with CEModeB.

6.7A.2.1	UE Re-establishment delay requirement for CEModeA

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 36.331 [2] is detected by the UE to the moment the UE has transmitted all repetitions of the PRACH preamble to the target cell. The UE re-establishment delay (TUE_re-establish_delay) requirement for a UE configured with CEModeA shall be less than:

TUE_re-establish_delay=50 ms+i=1NfreqKsatellite,i*Tsearch+TSI_EUTRA-M1-CEModeA+TPRACH

-	Tsearch is the time required by the UE to search the target cell. Tsearch =100 ms if the target cell is known by the UE. Otherwise, Tsearch is specified in relevant intra-frequency cell identification requirements as described in Clause 8.13.2.1 for a UE configured with CEModeA. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

In the above requirement, a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal to or longer than the time duration required for the cell identification. Otherwise, it is unknown.

- 	Ksatellite,i: It is the scaling factor for measurements correspond to multiple NGSO satellites to be monitored on i-th frequency for RRC re-establishment; Ksatellite,i  = [TBD]

-	TSI-EUTRA-M1-CEModeA: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [2] for the target cell for a UE configured with CEModeA. TSI-EUTRA-M1-CEModeA includes the time to acquire the MIB and all the relevant SIBs of the target cell.

-	TPRACH is the interruption uncertainty in acquiring the first available PRACH occasion in the target cell. The actual value of TPRACH shall depend upon the PRACH configuration used in the target cell and the PRACH coverage enhancement level used by the UE for sending the random access to the target cell.

-	Nfreq: It is the total number of frequencies to be monitored for RRC re-establishment; Nfreq = 1 if the target cell is known.

There is no requirement if the target cell does not contain the UE context.

Editor’s note: FFS to consider addition component, Tsi-to-epoch, to accommodate for the time needed between the UE acquires the NTN SIB.

6.7A.2.2	UE Re-establishment delay requirement for CEModeB

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 36.331 [2] is detected by the UE to the moment the UE has transmitted all repetitions of PRACH preamble to the target cell. The UE re-establishment delay (TUE_re-establish_delay) requirement for a UE configured with CEModeB shall be less than:

TUE_re-establish_delay=50 ms+i=1NfreqKsatellite,i*Tsearch+TSIEUTRA-M1-CEModeB+TPRACH

-	Tsearch is the time required by the UE to search the target cell. Tsearch =100 ms if the target cell is known by the UE. Otherwise, Tsearch is specified in relevant intra-frequency cell identification requirements as described in Clause 8.13.3.1 for a UE configured with CEModeB. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

In the above requirement, a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal to or longer than the time duration required for the cell identification. Otherwise, it is unknown.

-	Ksatellite,i: It is the scaling factor for measurements correspond to multiple NGSO satellites to be monitored on i-th frequency for RRC re-establishment; Ksatellite,i  = [TBD]

-	TSI-EUTRA-M1-CEModeB: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [2] for the target cell for a UE configured with CEModeB; TSI-EUTRA-M1-CEModeB includes the time to acquire the MIB and all the relevant SIBs of the target cell.

-	TPRACH is the interruption uncertainty in acquiring the first available PRACH occasion in the target cell. The actual value of TPRACH shall depend upon the PRACH configuration used in the target cell and the PRACH coverage enhancement level used by the UE for sending the random access to the target cell.

-	Nfreq: It is the total number of frequencies to be monitored for RRC re-establishment; Nfreq = 1 if the target cell is known.

There is no requirement if the target cell does not contain the UE context.

Editor’s note: FFS to consider addition component, Tsi-to-epoch, to accommodate for the time needed between the UE acquires the NTN SIB.

<End of change#1>

<Start of change#2>

7.24A	UE transmit timing for Category M1 for Satellite Access

7.24A.1	Introduction

The Category M1 UE shall have the capability to follow the frame timing change of the connected eNode B. The uplink frame transmission takes placeNTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell.

The UE shall use the serving cell as the reference cell for deriving the UE transmit timing. UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements.

7.24A.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te_NTN_M1 where the timing error limit value Te_NTN_M1 is specified in Table 7.24A.2-1. This requirement applies when it is the first transmission in a DRX cycle, eDRX_CONN cycle, or the first transmission in a repetition period (R>1) for PUCCH, PUSCH, and SRS, or the first transmission after an uplink transmission gap in a repetition period (R>1) for PUCCH or PUSCH, or it is the PRACH transmission, or it is the transmission on PUR, or the first transmission in each segment of PUSCH/PUCCH for eMTC in a repetition period. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving cell minusNTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving cell. NTA_Ref for PRACH is defined as 0. NTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE(in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA_Ref for other channels is not changed until the next timing advance is received.

Table 7.24A.2-1: Te_NTN_M1 Timing Error Limit

| CE Mode | Te_NTN_M1_ |
| --- | --- |
| A | [41]*TS |
| B | [65]*Ts |
| NOTE 1: TS is the basic timing unit defined in TS 36.211. NOTE 2: This requirement applies regardless of the downlink carrier bandwidth. |  |

When it is not the first transmission in a DRX or eDRX_CONN cycle or there is no DRX or no eDRX_CONN cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, or at the start of a transmission segment boundary, be capable of changing the transmission timing according to the received downlink frame of the serving cell except when the timing advance in clause 7.3 is applied such that the UE transmission timing error shall be less than or equal to Te_NTN_M1 where the timing error limit value Te_NTN_M1 is specified in Table 7.24A.2-1.

When no repetition period is configured, or the configured repetition period is R=1, or at the start of a transmission segment boundary, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission,  in one adjustment shall be Tq_NTN_M1 seconds.

2)	The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission,  shall be 7*TS per second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission,  shall be Tq_NTN_M1 per 200ms.

Where the maximum autonomous time adjustment step Tq_NTN_M1 is specified in Table 7.24A.2-2.

Table 7.24A.2-2: Tq_NTN_M1 Maximum Autonomous Time Adjustment Step

| CE Mode | Tq_NTN_M1_ |
| --- | --- |
| A | 17.5*TS |
| B | 17.5*Ts |
| NOTE 1: TS is the basic timing unit defined in TS 36.211. NOTE 2: This requirement applies regardless of the downlink carrier bandwidth. |  |

For satellite access, when a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary as defined above.

<End of change#2>

<Start of change#3>

7.27A	UE timer accuracy for category M1 for Satellite Access

7.27A.1	Introduction

UE timers are used in different protocol entities to control the UE behaviour.

7.27A.2	Requirements

The requirements defined in clause 7.21.2 also apply for this section.

7.28A	Timing Advance for Category M1 for Satellite Access

7.28A.1	Introduction

The timing advance is initiated from E-UTRAN served by SAN, with MAC message that implies and adjustment of the timing advance, see TS 36.321 [17] clause 5.2.

7.28A.2	Requirements

7.28A.2.1	Timing Advance adjustment delay

The UE shall adjust the timing of its uplink transmission timing at sub-frame n+6+Koffset for a timing advance command received in sub-frame n. The value of Koffset is defined in clause 4.2.3 in TS 36.213 [3]

The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure.

7.28A.2.2	Timing Advance adjustment accuracy

The requirements defined in clause 7.3.2.2 also apply for this section.

<End of change#3>