3GPP TSG-RAN WG4 Meeting #105	R4-2218731

E-Meeting, 14 Nov. 2022 – 18 Nov. 2022

Title:	Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility

Response to:	R1-2210727

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN WG4

To:	RAN WG1

Cc:	RAN WG2

Contact Person:

Name:

E-mail Address :

Attachments:	None

1. Overall description:

RAN4 thanks RAN1 for the LS R1-2210727, and would like to provide the following feedback.

Question1: As mentioned in this agreement, while RAN1 assumes Rel-17 ICBM CSI measurement as starting point for L1 intra-frequency measurement for Rel-18 L1/L2 mobility, RAN1 wonders if the restriction on e.g., SFN offset alignment, BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP, and Rx timing difference, etc, described in 9.13.2 of TS38.133 for intra-frequency L1 non-serving measurement can be relaxed or not.

RAN4 Reply:

RAN4 has concluded to define SSB based intra-frequency L1-RSRP measurement requirements at least with the following three restrictions:

- SFN offset alignment in the same frequency layer,

- SSB for L1-RSRP measurement is within serving cell active BWP,

- RTD of serving cell and neighbour cell is within one CP.

RAN4 will further discuss whether to define SSB based intra-frequency L1-RSRP measurement requirements for the case that RTD of serving cell and neighbour cell is larger than one CP.

Question 2: As mentioned in this agreement, RAN1 would like to ask RAN4 to confirm our understanding that the supported scenarios not included in intra-frequency are regarded as inter-frequency for L1 measurement, which includes at least the following scenarios:

- The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

- The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

RAN4 Reply:

RAN4 would like to confirm that the scenarios not included in intra-frequency are regarded as inter-frequency for L1 measurement.

For SSB L1-RSRP measurement, intra-frequency or inter-frequency is defined with reference to the centre frequency and SCS of serving cell SSB but not BWP. RAN4 has defined SSB based intra-frequency L1 measurement as follows.

- For SSB L1-RSRP measurement, follow the definition of L3 measurement: - A measurement is defined as a SSB based intra-frequency L1 measurement provided the center frequency and SCS of the SSB of the neighbor cell is the same as SSB of the serving cell indicated in ServingCellConfigCommon - Note: RAN4 will revisit the definition based on RAN1/2 conclusion.

Regarding the two scenarios in Q2, RAN4’s view is as follows. Note: RAN4 assumes measured RS here refers to SSB.

  - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

[RAN4]: If SSB of neighbour cell has the same centre frequency and SCS as serving cell, then this scenario is intra-frequency L1-RSRP measurement, regardless whether the RS is covered by the active BWP or not. But RAN4 will not define requirements for such a case due to SSB for intra-frequency L1-RSRP measurement is not within active BWPs.

  - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

[RAN4]: RAN4 assumes “configured BWP” here includes initial BWP. If the SSB is not within any configured BWPs, it definitely will not align with the SSB for serving cell. Therefore, this scenario is inter-frequency L1-RSRP measurement.

2. Actions:

To RAN4:

RAN4 kindly requests RAN1 to consider the response above into their future specification work.

3. Date of Next RAN1 Meetings:

RAN WG4 Meeting #106	    Feb. 27 – Mar.03, 2023 		Athens, Greece

RAN WG4 Meeting #106bis-e	    Apr. 17 – Apr.26, 2023		Online