3GPP TSG-RAN WG4 Meeting # 105-e	R4-2218757

# Toulouse, 14th-18th, Nov, 2022

Source: 	Samsung

Title: 	Further consideration on Fs assumption for non-collocated EN-DC, NR-CA deployment

Agenda Item:	8.11.2

Document for:	Discussion

- Introduction

In RAN#95-e meeting, a new WI is approved which focused on non-collocated EN-DC/NR-CA deployment scenario, while further revised in RAN#97 as [1], the detailed background of non-collocated deployment was provided in [2].

During RAN4#104-bis-e meeting, the UE architecture and the RF requirements for Type-2 intra-band NR-CA non-collocated deployment had been confirmed [3]. Meanwhile, UE architecture for Type-3a/b and Type-4a/b had also been confirmed and the corresponding requirements are FFS [4]. In addition, we also provided some analysis on the frequency separation along with the architectures for collocated and non-collocated deployment [5].

Particularly, in this paper, we further discuss the frequency separation assumption for the work (limited to CA/EN-DC for bands 42, n77/n78) under this WI.

- 2. Discussion

In last meeting, we made some initial analysis on the frequency separation limitation of UE along with the different architectures for both collocated and non-collocated deployment under this WI (work is limited to CA/EN-DC for bands 42, n77/n78), companies show interest hence moderator KDDI insert the “Frequency separation between 2cc” column into final architecture table for further discussion, reproduce as below.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

As following we further analyse UL Fs and DL Fs separately.

- UL Fs

In terms of non-contiguous intra-band UL CA, IE intraBandFreqSeparationUL-AggBW-GapBW-r16 was introduced in 38.306 to allow UE to indicate the frequency separation class to NW, the benefit is that NW could avoid configuring the Fs larger than the UE’ capability, correspondingly Table 5.3A.5-2 was introduced in 38.101-1, reproduce as below. Take 2-CC case CA_n77(2A) as example, 1Lo+1PA architecture generally is able to achieve frequency separation class I (100MHz) and class II (200MHz); 2Lo+2PA architecture is able to achieve any frequency separation, that is no frequency separation limitation imposed on 2Lo+2PA architecture. In addition, 600MHz for class III was selected with the assumption that no operator’s holding spectrum on ＞3.3GHz licensed band exceeding 600MHz, hence essentially class III corresponds to no frequency separation limitation.

| intraBandFreqSeparationUL-AggBW-GapBW-r16 Indicates the UL frequency separation class between lower edge of lowest CC and upper edge of highest CC of Intra-band UL non-contiguous CA, i.e. including both the aggregated bandwidth and the gap bandwidth. 3 frequency separation classes are introduced and the values are as follow: - class I: Non-contiguous CA separation class ≤ 100MHz - class II: 100MHz < Non-contiguous CA separation class≤ 200MHz - class III: 200MHz < Non-contiguous CA separation class <600MHz | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Table 5.3A.5-2: NR intra-band non-contiguous UL CA frequency separation classes

| NR NC UL CA frequency separation class | Maximum allowed frequency separation |
| --- | --- |
| I | 100 MHz |
| II | 200 MHz |
| III | [600MHz] |

In terms of non-contiguous inter-band EN-DC with overlapping bands, take 2-CC case DC_42_n77 as example, dual UL has not been defined yet.

- DL Fs

Usually there would be no limitation on DL frequency separation since single LNA is always full-band supported.

With above consideration, it is suggested to replace the “Frequency separation between 2cc” column with a general assumption text that the frequency separation assumption is ≤ 600MHz under this WI (work is limited to CA/EN-DC for bands 42, n77/n78)

Proposal 1: It is suggested to replace the “Frequency separation between 2cc” column with a general text (note to architecture table) that the assumption for frequency separation is ≤ 600MHz under this WI (work is limited to CA/EN-DC for bands 42, n77/n78)

In addition, it is identified that the description for intraBandFreqSeparationUL-AggBW-GapBW-r16 in Ts 38.306 is not fully aligned with the latest RAN4 spec. The background is, indeed originally in RAN4#96-e RAN4 agreed the CR [6] with the following table:

Table 5.3A.5-2: NR non-contiguous CA frequency separation classes

| NR NC CA frequency separation class | frequency separation |
| --- | --- |
| I | 20 MHz ≤ BWChannel_NC_CA ≤ 100 MHz |
| II | 100 MHz < BWChannel_NC_CA ≤ 200 MHz |
| III | 200MHz<BWChannel_NC_CA≤ [600MHz] |

However, in RAN4#97-e meeting there is a proposal [7] from Apple to change the table as below, and the corresponding CR [8] was agreed in RAN4#98-e meeting to revise the FR1 UL NC CA frequency separation class definition in Table 5.3A.5-2 (38.101-1) from specifying a frequency range to only defining the upper limits, as well as to add text to clarify the frequency separation class specified in Table 5.3A.5-2 can be indicated when the signalling is absent for dualPA-Architecture IE. The consideration is that in the original table, it might be confused that if a UE support class II (100~200), whether it is able to support (20~100) or not. However, it appears that we didn't inform RAN2 this change at that time, so RAN2 is still using the table with ~100, 100~200, 200~600.

Table 5.3A.5-2: NR intra-band non-contiguous UL CA frequency separation classes

| NR NC UL CA frequency separation class | Maximum allowed frequency separation |
| --- | --- |
| I | 100 MHz |
| II | 200 MHz |
| III | [600 MHz] |

Furthermore, in the description for class III of intraBandFreqSeparationUL-AggBW-GapBW-r16, “=” 600MHz is not included, while “=” 600MHz is desirable for the spectrum allocation of Japanese operators,

With above consideration, it is suggested to send Ls to RAN2 to inform the RAN4 change on Table 5.3A.5-2 of 38.101-1, as well as adding “=” 600MHz for frequency separation class III from Rel-16.

Proposal 2: It is suggested to send Ls to RAN2 to inform the RAN4 change on frequency separation class definition, as well as to add “=600MHz” for frequency separation class III for intraBandFreqSeparationUL-AggBW-GapBW-r16 in TS 38.306 from Rel-16 if there is no NBC issue.

- 3. Conclusion

Proposal 1: It is suggested to replace the “Frequency separation between 2cc” column with a general text (note to architecture table) that the assumption for frequency separation is ≤ 600MHz under this WI (work is limited to CA/EN-DC for bands 42, n77/n78)

Proposal 2: It is suggested to send Ls to RAN2 to inform the RAN4 change on frequency separation class definition, as well as to add “=600MHz” for frequency separation class III for intraBandFreqSeparationUL-AggBW-GapBW-r16 in TS 38.306 from Rel-16 if there is no NBC issue.

- 4. Reference

[1] RP-222309 New WID: Support of intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN#97.

[2] R4-2212011 On the UE RF aspects for non-collocated EN-DC,NR-CA, Samsung, RAN4#104e

[3] R4-2217733 WF_NonCol_intraB_for NR-CA Type-2 UE

[4] R4-2217734 WF_NonCol_intraB_for New Type UE, KDDI, RAN4#104-bis-e

[5] R4-2215736 Views on UE RF aspect for non-collocated EN-DC, NR-CA deployment, Samsung, RAN4#104-bis-e

[6] R4-2011725, CR for intra-band UL CA non-contiguous CA requirement, Huawei, HiSilicon, RAN4#96-e

[7] R4-2014909, FR1 intra-band UL NCCA frequency separation and power class, Apple, RAN4#97-e

[8] R4-2103149, CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements, Apple, RAN4#97-e