3GPP TSG-RAN WG4 Meeting #105	R4-2218908

Toulouse France, 14 – 18 November, 2022

Source:	NEC

Title:	Discussion on OBW measurements for FR2-2

Agenda item:	6.3.3.2

Document for:	Discussion

Introduction.

This contribution discusses the issues on OBW measurements for FR2-2 and provides proposals on it.

Discussion

RAN4 has agreed to apply FR2-1 OTA occupied bandwidth requirements to FR2-2 [1]. For OTA occupied bandwidth requirement measurements, it was agreed to discuss the occupied bandwidth measurement step size in the performance phase [2]. However, OTA occupied bandwidth measurement step size for FR2-2 has not been discussed and RAN4 does not have the agreements on it yet.

In the current specifications [3], span and number of measurement points for OBW measurements for FR2 are specified as below.

Table 6.7.2.4.2-2: Span and number of measurement points for OBW measurements for FR2

| Bandwidth | BS channel bandwidth BWChannel (MHz) |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 | > 50 |
| Span (MHz) |  |  |  |  |  |
| Minimum number of measurement points |  |  |  |  |  |

It is clear that above table is only applicable to FR2-1. RAN4 needs another table for span and number of measurement points for OBW measurements.

Observation 1: Current table for span and number of measurement points for OBW measurements for FR2 is not applicable to FR2-2.

Observation 2: RAN4 needs a table for span and number of measurement points for OBW measurements for FR2-2.

For span, twice of channel bandwidth is adopted for both FR1 and FR2-1. We do not see any reason to change this for FR2-2. We propose to keep the span to be twice of the channel bandwidth for FR2-2.

Proposal 1: 	Span for OBW measurement shall be twice of channel bandwidth for FR2-2.

Minimum number of measurement points for FR2-1 is specified to increase in proportion to the channel bandwidth and to make the measurement interval 100 kHz. If we keep this if FR2-2, number of measurement points will be as many as 20,000 points for 2,000 MHz channel bandwidth and more for aggregated channel bandwidth cases. We think it is too much and it makes the testing time unnecessarily long. Considering the fact that minimum number of measurement points are specified up to 400 MHz channel bandwidth in FR2-1, we propose to have the same numbers for 100 and 400 MHz channel bandwidth. For the channel bandwidth wider than 400 MHz, we proposed to reduce the number of points as shown in the table below. It is for the minimum number of measurement points. If some vendors want to test with more measurement points, it shall be allowed of course.

Table 1: Proposed span and number of measurement points for OBW measurements for FR2-2

| Bandwidth | BS channel bandwidth BWChannel (MHz) |  |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 | 400 | 800 | 1600 | 2000 | 400 ≧ | > 400 |
| Span (MHz) | 2×BWChannel |  |  |  |  | 2×BWChannel_CA |  |
| Minimum number of measurement points | 2×BWChannel200 kHz |  | 2×BWChannel800 kHz |  |  | 2×BWChannel_CA200 kHz | 2×BWChannel_CA800 kHz |

Proposal 2: 	Minimum number of measurement points for OBW measurement shall be 2×BWChannel200 kHz  for 100 MHz and 400 MHz channel bandwidth for FR2-2.

Proposal 3: 	Minimum number of measurement points for OBW measurement shall be 2×BWChannel800 kHz  for 800 MHz, 1600 MHz, and 2000 MHz channel bandwidth for FR2-2.

Corresponding CR is submitted in [4].

Conclusion

We have discusses the span and number of measurement points for OBW measurements for FR2-2 and made the following proposals.

Proposal 1: Span for OBW measurement shall be twice of channel bandwidth for FR2-2.

Proposal 2: Minimum number of measurement points for OBW measurement shall be 2×BWChannel200 kHz  for 100 MHz and 400 MHz channel bandwidth for FR2-2.

Proposal 3: Minimum number of measurement points for OBW measurement shall be 2×BWChannel800 kHz  for 800 MHz, 1600 MHz, and 2000 MHz channel bandwidth for FR2-2.

Reference

[1] R4-2106113, “WF on BS RF TX requirements for 52.6 – 71 GHz”, Nokia, Nokia Shanghai Bell

[2] R4-2108638, “WF on BS TX RF requirements for 52.6 – 71 GHz”, Nokia, Nokia Shanghai Bell

[3] TS 38.141-2 v17.7.0

[4] R4-2218909, “CR to 38.141-2: OTA OBW requirements for FR2-2”, NEC