3GPP TSG-RAN WG4 Meeting # 105									    		                   R4-2219006

Electronic Meeting, 14– 18 Nov, 2022, Toulouse

Agenda item:			8.23.2

Source:	Samsung

Title:	Simulation assumptions and preliminary co-existence study for above 10GHz NTN co-existence study

Document for:	Discussion

- Introduction

According to the Rel-18 NR NTN enhancement WID, RAN4 has been mandated to study and identify NTN example band in above 10 GHz.

The following assumptions are taken a baseline for this work:

- GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered

- ESIM scenarios for NGSO in Ka band are not considered in this WI.

- Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified

- Regarding mobile VSAT, three types of terminals and scenarios exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study.

- FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2

- The ITU-R harmonized Ka band will serve as reference

- Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification.

For co-existence studies, following considerations should be taken into account:

- Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow.

- For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands.

In the RAN4 #104-bis-e meeting, agreements on some assumptions have been achieved. And the remaining issues have been discussed in this paper.

- Discussion

Co-existence simulation scenarios

In order to determine the ACLR of NTN Cell and ACS of NTN UE,  coexistance studies are necessarily by using FR2 NT system parameters and requirements. RAN4 is responsible for defining these metrics, otherwise, the work of RAN4 is incomplete. The necessity of these parameters is that RAN4 needs to consider the forward compatibility of the frequency band, and the appropriate parameter values can make the subsequent work of other services in this frequency band easier and more reasonable in the future. On the other hand, RAN4 should also considers the coexistance between NTN system. However, the parameters and the requirements for the TN system in 20GHz are not determined yet, this situation makes the coexistance study hardly to start. Considering the propagtion characteristics, using the paramters and requirements of FR2 is a better choice.

Proposal 1:  In order to determine the ACLR of NTN Cell and ACS of NTN UE,  coexistance studies are necessarily by using FR2 TN system parameters and requirements.

It is easier to continue to use the same channel bandwidth assumption with the S-band coexistance study. And TN 400MHz & NTN 400MHz can be the second priority for the coexistance study because this is the largest channel bandwidth assumption for the TN and NTN systems.

Proposal 2: TN 200MHz & NTN 200MHz are proposed to be considered as the coexistence study assumption for the channel bandwidth, TN 400MHz & NTN 400MHz can be the second priority for the coexistance study.

The antenna height of the TN micro base station is lower than that of the macro base station, therefore, the interference from far away NTN UEs would be blocked by the building in the urban area. However, if the NTN terminal is deployed around the micro base station or the micro base station is covered by the NTN beam, the interference still cannot be ignored.

Proposal 3: Consider Dense Urban secnario of TN system with the mirco BSs as the second priority for the coexistance study.

Simulation parameters

NTN parameters

UE parameters

Proposal 4:  Propose Table 1, 2 and 3 as the NTN UE parameters for the coexistence study. Some parameters should be further discussed.

Table 1 NTN Fixed VSAT UE parameters

| Characteristics |  |
| --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 39.7 dBi |
| Noise figure | 1.2 dB |
| Antenna temperature | 150 K |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 43.2 dBi |
|  | NOTE: VSAT terminal characteristics could be implemented with phased array antenna |

Table 2 NTN movable VSAT UE parameters

| Characteristics | ESIM (TR 38.821) | ESIM (ITU) |
| --- | --- | --- |
| Antenna type and configuration | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) | Antenna pattern: S.580-6 |
| Polarisation | Linear: +/-45°X-pol |  |
| Rx Antenna gain | TBD dBi per element | 45 dBi |
| Antenna temperature | TBD K |  |
| Noise figure | TBD dB |  |
| Tx transmit power | [TBD W (TBD dBm)] | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain | TBD dBi per element | 45 dBi |

For the horizontal boresight, it is suggested to use random value between 0-360 for one snapshot during the simulation. Due to the height of the satellite orbit is much larger than the TN network diameter, the NTN UE boresight can be simplified as the same value. Otherwise, the boresight of the NTN UEs should be calculated based on the angular relation between satellite and UEs, but the complex of the simulation would increase a lot.

Table 3 Horizontal boresight and Vertical tilt assumptions of NTN UEs

|  | GEO | LEO |
| --- | --- | --- |
| Antenna vertical tilt | 20 degree above horizontal | 30 degree above horizontal |
| Antenna horizontal boresight | Following Serving satellite or random between (0-360) | Following Serving satellite or random between (0-360) |

TN parameters

Proposal 5: The TN parameters of Macro base station and Dense Urban Micro base station are proposed according to the TR38.803. Macro base station and Dense Urban Micro base station are considered.

Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling

Proposal 6: Refers to TR 38.863 to determine the Satellite and UE Antenna and beam forming pattern modelling.

TN BS and UE antenna and beam forming pattern modelling

Proposal 7: Base station and UE antenna pattern modelling are proposed according to the TR 38.803.

Co-existence study

Simulation assumptions basic on the assumptions above and the annex. NTN UL interfere with TN UL considered below, which we think is one of the important scenarios for this topic. Some undetermined parameters assuptions for the coexistence study can be found in the annex, basic on our understanding. Therefore, the simulation results are very perlimiary version, and will be updated with the disscusion for the scenarios and assuptions.

- Simulation results

Figure.1 Throughput loss – ACLR curves for Situation 3 and 4

|  | ACLR requirement |
| --- | --- |
| Situation 1 – NTN UEs deployed on the circular of the TN network edge with 20, 60 90 degrees up-tilt | No specific requirement for the ACLR for NTN UEs |
| Situation 2 – NTN UEs deployed inside the circular of the TN network with 20, 60 90 degrees up-tilt | No specific requirement for the ACLR for NTN UEs |
| Situation 3 - NTN UEs deployed on the circular of the TN network edge, all NTN UEs pointing to the TN BSs | 30 |
| Situation 4 - NTN UEs deployed inside the circular of the TN network edge, all NTN UEs pointing to the TN BSs | 44 |
| NOTE: Situation 3 and 4 are considering very specific deployment scenarios, e.g., airport, land-base ESIM. |  |

Observiation 1: The NTN UL interfere with TN UL situation 1&2 have no specific ACLR requriement due to the low probability that NTN UEs point to the BSs directly and the high propagtion loss.

Observation 2: The NTN UL interfere with TN UL situation 3&4 shown that in some specific deployment scenarios, 30-44dB ACLR are needed to protect the TN uplink.

- Conclusion

It is proposed to use the simulation assumptions in this paper for Ka-band NTN co-existence study.

Proposal 1: In order to determine the ACLR of NTN Cell and ACS of NTN UE,  coexistance studies are necessarily by using FR2 NT system parameters and requirements.

Proposal 2: TN 200MHz & NTN 200MHz are proposed to be considered as the coexistence study assumption for the channel bandwidth, TN 400MHz & NTN 400MHz can be the second priority for the coexistance study.

Proposal 3: Consider Dense Urban secnario of TN system with the mirco BSs as the second priority for the coexistance study.

Proposal 4:  Propose Table 1, 2 and 3 as the NTN UE parameters for the coexistence study. Some parameters should be further discussed.

Proposal 5: The TN parameters of Macro base station and Dense Urban Micro base station are proposed according to the TR38.803. Macro base station and Dense Urban Micro base station are considered.

Proposal 6: Refers to TR 38.863 to determine the Satellite and UE Antenna and beam forming pattern modelling.

Proposal 7: Base station and UE antenna pattern modelling are proposed according to the TR 38.803.

Observiation 1: The  situation 1&2 have no specific ACLR requriement due to the low probability that NTN UEs point to the BSs directly and the high propagtion loss.

Observation 2: The situation 3&4 shown that in some specific deployment scenarios, 30-44dB ACLR are needed to protect the TN uplink.

- Reference

[1] TR 38.803 Study on new radio access technology: Radio Frequency (RF) and co-existence aspects

[2] TR 38.863 Non-terrestrial networks (NTN) related RF and co-existence aspects

[3] 3GPP, R4-2217468 WF on [313] NR_NTN_enh_Part2

- Annex – Coexistance study assumptions

- Scenario NTN UL interfere with TN UL

- NTN Satellite parameters

| Satellite orbit |  | GEO |
| --- | --- | --- |
| Satellite altitude |  | 35786 km |
| Satellite EIRP density | Ka-band (i.e. 20 GHz for DL) | 40 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi |
| Channel bandwidth |  | 200 |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  | 0.1765 deg |
| Satellite beam diameter |  | 110 km |
| G/T | Ka-band (i.e. 30 GHz for UL) | 28 dB K-1 |
| Satellite Rx max Gain |  | 58.5 dBi |

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 30GHz(UL)/20GHz(DL) |  |
| The number of active UE (UL) | 9 UEs and 2RBs per UE for GEO and LEO1 |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 2.6.2 |  |
| NTN satellite Noise figure in dB | See Table 2.3.1-3 |  |
| Handover margin | 3dB |  |
| Note 1: UEs are equally splitted inside the channel bandwidth into ACIR 3 regions. Scheduled PRB position for UE1 per satellite beam should be also fully aligned to simulate the worst case for co-channel interference and this is also aligned with full buffer case. |  |  |

| Satellite | GEO |
| --- | --- |
| G/T (dB K-1) | 28 |
| G_Rx (dBi) | 58.5 |
| NF (dB) | 5.9 |

- NTN UE parameters

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | circular |
| Rx Antenna gain | 39.7 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 43.2 dBi |

| Carrier frequency in GHz | 30 |
| --- | --- |
| Transmission bandwidth in MHz | 200 |
| Environment | Urban macro |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | 200m |
| Number of micro BSs per macro cell |  |
| System loading and activity | See Annex 2 |
| Network location | See Annex 2 |
| DL subcarrier spacing | 120kHz |
| UL | OFDMA |
| DL power control | No |
| UL power control | See Section 2.7.1, 2.7.2 |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | Same as the number of BS beam |
| Number of scheduled UE per cell (UL) | Same as the number of BS beam |
| UE antenna height in meter | Same as 3D-UMi in TR 36.873 |
| UE TX power in dBm | -40 to 23 |
| UE antenna gain in dBi | 0 |
| Building penetration loss | In path loss model, TR 38.901 |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 11 |
| UE noise figure in dB | 9 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Link-level performance model | See Section 2.10 |
| UE distribution | Uniform |
| Evaluation metrics | See Section 2.9 |

|  | Urban Macro |
| --- | --- |
| ISD in meters | 200 |
| BS Antenna height in meters | 25 |
| UE Outdoor/indoor | 100% Outdoor |
| UE height in meter | 1.5 |

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 28dB |
|  | ACS | 24dB |
| UE | ACLR | 17dB |
|  | ACS | 22dB |

- TN BS and UE antenna and beam forming pattern modelling

Refer to TR 38.803

- Propagation model

38.901-UMa

- Transmission power control model

  - TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

  - NTN UL TPC

For calibration purpose, reuse the same TN TPC for NTN with SNR target 15dB.

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821. The CLx-ile value should be adapted for rural scenario.

  - DL TPC

For downlink scenario, no power control scheme is applied.

  - Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

- Performance metric

For NR,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

For NTN,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

- Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803 as the SINR-Throughput performance metrics