3GPP TSG-RAN4 Meeting #105	R4-2219024

# Toulouse, France, 14th Nov – 18th Nov, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0319 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on UE demodulation and CSI requirement for FeMIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-11-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This big CR merges endorsed draft CR to 38.101-4 in RAN4#104-bis-e and RAN4#105. The big CR R4- 2217433 was approved in RAN4#104bis-e meeting.The reason for change in endorsed draft CR is copied below - R4-2217434 RAN4 has agreed to define UE demodulation requirements for - HST Scheme A and Scheme B with test applicability rule - PMI requirment for multi-TRP - R4-2217435 RAN4 has agreed to define UE demod requirements for HST Scheme A - R4-2217436 Introduce PDSCH requirement for HST-SFN scheme B - R4-2217437 For FeMIMO performance, RAN4 has agreed to introduce PMI measurement requirement for multi-TRP - R4-2217438 RAN4 has agreed to define requirements for PDCCH with intra-slot repetition - R4-2217439 Reference measurement channels do not exist for multi-TRP PDCCH requirements - R4-2220194 After RAN4#104-bis-e big DraftCR was endorsed. Results were captured in “[ ]” or as TBD. Updates to requirements with HST SFN Scheme-A are needed based on updated simulation results. Updates for sub-clause numbering based on number in Big draftCR. - R4-2220195 Introduce PDSCH requirement for HST-SFN scheme B - R4-2219277 RAN4 has agreed to define requirements for PDCCH with intra-slot repetition |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of change in endorsed draft CR is copied as below: - R4-2217434 Added general and applicabilty information for agreed new FeMIMO demodulation requirements - R4-2217435 Introduced requirements and channel model for HST Scheme A - R4-2217436 For PDSCH requirement for HST-SFN scheme B, added clause 5.2.2.1.x, 5.2.2.2.x, 5.2.3.1.x, 5.2.3.2.x, B.3.x. - R4-2217437 For FeMIMO performance, RAN4 has agreed to introduce Single PMI with 8Tx TypeI-SinglePanel PMI measurement requirement for multi-TRP, including 2Rx/4Rx for both FDD and TDD - R4-2217438 Introduced new sections for requirements for FDD/TDD PDCCH with intra-slot repetition - R4-2217439 Introduced reference measurement channels for multi-TRP PDCCH performance - R4-2220194 Updated sub-clause numbers, Updated requirements based on simulation results - R4-2220195 For PDSCH requirement for HST-SFN scheme B, added clause 5.2.2.1.x2, 5.2.2.2.x2, 5.2.3.1.x2, 5.2.3.2.x2, B.3.x2. - R4-2219277 Introduced new sections for requirements for FDD/TDD PDCCH with intra-slot repetition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequences if not approved for endorsed draft CR are coppied as below. - R4-2217434 No testable requirements for new agreed FeMIMO demodualtion requirements - R4-2217435 UE demod requirements for HST Scheme A will not be defined - R4-2217436 There will be inconsistence between RAN4 agreements and the specification. - R4-2217437 Requirements for FeMIMO performance will be incomplete. - R4-2217438 Requirements will not be defined for FDD/TDD PDCCH with intra-slot repetition - R4-2217439 UE demod requirements for multi-TRP PDCCH requirements will not be defined. - R4-2220194 Requirements will not be complete for HST SFN schemeA - R4-2220195 There will be inconsistence between RAN4 agreements and the specification. - R4-2219277 Requirements will not be defined for FDD/TDD PDCCH with intra-slot repetition |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3, 5.1.1.4 5.2.2.1, 5.2.2.2, 5.2.3.1, 5.2.3.2, 5.2.2.1.x, 5.2.2.x, 5.2.3.1.x, 5.2.3.2.x 5.3.2.1.x ,5.3.2.2.x ,5.3.3.1.x ,5.3.3.2.x 6.3.2.1.X, 6.3.2.2.X, 6.3.3.1.X, 6.3.3.2.X A.3.2, B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## General

### 5.1.1	Applicability of requirements

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217434>

#### 5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 5.5A.1 | 1)Up to 16 DL carriers 2)Same numerology across carrier for data/control channel at a given time |  |
| Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1) |  |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |  |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 |  |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR1 FDD | PDSCH | Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |  |
| UE PDSCH processing capability #2 (pdsch-ProcessingType2) | FR1 FDD | PDSCH | Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |  |
| Pre-emption indication for DL (pre-EmptIndication-DL) | FR1 FDD | PDSCH | Clause 5.2.2.1.8 Clause 5.2.3.1.8 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.8 Clause 5.2.3.2.8 |  |  |
| Single DCI based SDM transmission for multi-TRxP (singleDCI-SDM-scheme-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 Clause 5.2.3.1.11 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.11 Clause 5.2.3.2.11 |  |  |
| Multi DCI based multi-TRxP support (multiDCI-MultiTRP-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.12 Clause 5.2.3.1.12 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.12 Clause 5.2.3.2.12 |  |  |
| Single DCI based FDM Scheme-A for multi-TRxP(supportFDM-SchemeA-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 Clause 5.2.3.1.13 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.13 Clause 5.2.3.2.13 |  |  |
| Single DCI based inter-slot TDM for multi-TRxP (supportInter-slotTDM-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |  |
| Maximum number of TCI states in Single-DCI based inter-slot TDM (maxNumberTCI-states-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  | The requirements apply only when maxNumberTCI-states-r16 = 2. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |  |  |
| DRX Adaptation (drx-Adaptation-r16) | FR1 FDD | PDCCH | Clause 5.3.2.1.3 | If the Test 1 in Clause 5.3.2.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.2.1.1. |  |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.3 | If the Test 1 in Clause 5.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.2.2.1. |  |
|  | FR1 FDD | PDCCH | Clause 5.3.3.1.3 | If the Test 1 in Clause 5.3.3.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.3.1.1. |  |
|  | FR1 TDD | PDCCH | Clause 5.3.3.2.3 | If the Test 1 in Clause 5.3.3.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.3.2.1. |  |
| Validating P/SP-CSI-RS reception (periodicAndSemi-PersistentCSI-RS-r16) | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 Clause 5.2A.2.3 Clause 5.2A.3.3 | The requirements apply only in case tested UE supporting operations in shared spectrum access and validation of P/SP-CSI-RS reception based on DCI |  |
| Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 | The requirements apply only in case tested UE supports one of UL channels for dynamic channel access mode and UL channel access for semi-static channel access mode |  |
| Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS ( CRS-IM-DSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 Clause 5.2.3.1.18 | UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 Clause 5.2.3.2.19 |  |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 1-1) Clause 5.2.3.1.18 (Test 1-1) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-1) Clause 5.2.3.2.19 (Test 1-1) |  |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-NWA-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 2-1) Clause 5.2.3.1.18 (Test 2-1) | If the Test 1-1 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 2-1 in clause 5.2.2.2.20. If the Test 1-1 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 2-1 in clause 5.2.3.2.19. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-1) Clause 5.2.3.2.19 (Test 2-1) |  |  |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-2) Clause 5.2.3.2.19 (Test 1-2) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |  |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-NWA-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-2) Clause 5.2.3.2.19 (Test 2-2) | If the Test 1-2 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 2-2 in clause 5.2.2.2.20. If the Test 1-2 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 2-2 in clause 5.2.3.2.19. |  |
| Support for SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH (sfn-SchemeA-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.X1 Clause 5.2.3.1.X1 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.X1 Clause 5.2.3.2.X1 |  |  |
| Support for SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH (sfn-SchemeB-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.X2 Clause 5.2.3.1.X2 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.X2 Clause 5.2.3.2.X2 |  |  |
| Support for PDCCH with intra-slot repetition (mTRP-PDCCH-Repetition-r17) | FR1 FDD | PDCCH | Clause 5.3.2.1.X1 Clause 5.3.3.1.X1 |  |  |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.X1 Clause 5.3.3.2.X1 |  |  |

#### 5.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

#### << Unchanged sections omitted >>

#### 5.1.1.5	Applicability of different requirements for HST

The applicability rules for different HST requirements in section 5 are specified in Table 5.1.1.5-1, Table 5.1.1.5-X.

Table 5.1.1.5-1: Applicability of requirements for HST

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-6) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-5) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-11) | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-7) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.1 (Test 1-6) | FR1 FDD | PDSCH | Clause 5.2.3.1.1 (Test 1-5) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.1 (Test 1-11) | FR1 TDD | PDSCH | Clause 5.2.3.2.1 (Test 1-7) |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-5 ) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-7 and 1-11) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.9 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.3.1.1 (Test 1-5) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.9 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.3.2.1 (Test 1-7 and 1-11) |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-7) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.1 (Test 1-7) | FR1 FDD | PDSCH | Clause 5.2.3.1.1 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.10 (Test 1-1 or 1-2) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-5 ) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.10 (Test 1-1 or 1-2) | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-7 and 1-11) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.10 (Test 1-1 or 1-2) | FR1 FDD | PDSCH | Clause 5.2.3.1.1 (Test 1-5) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.10 (Test 1-1 or 1-2) | FR1 TDD | PDSCH | Clause 5.2.3.2.1 (Test 1-7 and 1-11) |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.10 (Test 1-2) | FR1 FDD | PDSCH | Clause 5.2.2.1.10 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.10 (Test 1-2) | FR1 TDD | PDSCH | Clause 5.2.2.2.10 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.10 (Test 1-2) | FR1 FDD | PDSCH | Clause 5.2.3.1.10 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.10 (Test 1-2) | FR1 TDD | PDSCH | Clause 5.2.3.2.10 (Test 1-1) |  |

Table 5.1.1.5-X: Applicability of requirements for HST Enhanced SFN Transmission schemes

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.X1 | FR1 FDD | PDSCH | Clause 5.2.2.1.X2 |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.X1 | FR1 TDD | PDSCH | Clause 5.2.2.2.X2 |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.X1 | FR1 FDD | PDSCH | Clause 5.2.3.1.X2 |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.X1 | FR1 TDD | PDSCH | Clause 5.2.3.2.X2 |  |

#### 5.1.1.6	Applicability and test rules for PDSCH performance requirements with power imbalance for intra-band contiguous CA

<End of Change R4-2217434>

## 5.2	PDSCH demodulation requirements

### 5.2.2	2RX requirements

#### 5.2.2.1	FDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217435, R4-2220194>

##### 5.2.2.1.X1	Minimum requirements for HST-SFN Scheme A

The performance requirements are specified in Table 5.2.2.1.X1-3, with the addition of test parameters in Table 5.2.2.1.X1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.X1-1.

Table 5.2.2.1.X1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme A scenario defined in B.3.X1 | 1-1 |

Table 5.2.2.1.X1-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI States TCI state #0, TCI State #1. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with TCI States TCI state #1, TCI State #2. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with TCI States TCI state #2, TCI State #0. |  |  |  |  |

Table 5.2.2.1.X1-3: Minimum performance for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [R.PDSCH.1-8.5 FDD] | 10 / 15 | 16QAM, 0.48 | HST-SFN Scheme A | 2x2 | 70 | [11.8] |

<End of Change R4-2217435, R4-2220194>

## <Start Of Change R4-2217436, R4-2220195>

##### 5.2.2.1.x2	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.2.1.x2-3, with the addition of test parameters in Table 5.2.2.1.x2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.x2-1.

Table 5.2.2.1.x2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.x2 | 1-1 |

Table 5.2.2.1.x2-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |
| Note 1: SSB # (k mod 3), CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint#0 {TCI state #0, TCI State #1} is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1. Codepoint#1 {TCI state #1, TCI State #2} is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2. Codepoint#2 {TCI state #2, TCI State #0} is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3. The second indicated TCI state in each codepoint is not used for quasi co-location parameters {Doppler shift, Doppler spread}. |  |  |  |  |

Table 5.2.2.1.x2-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.5 FDD | 10/15 | 16QAM, 0.48 | HST-SFN-SchemeB | 2x2 | 70 | [11.3] |

<End of Change R4-2217436, R4-2220195>

#### 5.2.2.2	TDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217435, R4-2220194>

##### 5.2.2.2.X1	Minimum requirements for HST-SFN Scheme A

The performance requirements are specified in Table 5.2.2.2.X1-3, with the addition of test parameters in Table 5.2.2.2.X1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.X1-1.

Table 5.2.2.2.X1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme A scenario defined in B.3.X1 | 1-1 |

Table 5.2.2.2.X1-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI States TCI state #0, TCI State #1. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with TCI States TCI state #1, TCI State #2. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with TCI States TCI state #2, TCI State #0. |  |  |  |  |

Table 5.2.2.2.X1-3: Minimum performance for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40 / 30 | 16QAM, 0.48 | HST-SFN Scheme A | 2x2 | 70 | [12.9] |

<End of Change R4-2217435, R4-2220194>

## <Start Of Change R4-2217436, R4-2220195>

##### 5.2.2.2.x2	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.2.2.x2-3, with the addition of test parameters in Table 5.2.2.2.x2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.x2-1.

Table 5.2.2.2.x2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.x2 | 1-1 |

Table 5.2.2.2.x2-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Note 1: SSB # (k mod 3), CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint#0 {TCI state #0, TCI State #1} is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1. Codepoint#1 {TCI state #1, TCI State #2} is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2. Codepoint#2 {TCI state #2, TCI State #0} is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3. The second indicated TCI state in each codepoint is not used for quasi co-location parameters {Doppler shift, Doppler spread}. |  |  |  |  |

Table 5.2.2.2.x2-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40/30 | 16QAM, 0.48 | HST-SFN-SchemeB | 2x2 | 70 | [11.8] |

<End of Change R4-2217436, R4-2220195>

### 5.2.3	4RX requirements

#### 5.2.3.1	FDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217435, R4-2220194>

##### 5.2.3.1.X1	Minimum requirements for HST-SFN Scheme A

The performance requirements are specified in Table 5.2.3.1.X1-3, with the addition of test parameters in Table 5.2.3.1.X1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.X1-1.

Table 5.2.3.1.X1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme A scenario defined in B.3.X1 | 1-1 |

Table 5.2.2.1.X1-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI States TCI state #0, TCI State #1. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with TCI States TCI state #1, TCI State #2. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with TCI States TCI state #2, TCI State #0. |  |  |  |  |

Table 5.2.3.1.X1-3: Minimum performance for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.5 FDD | 10 / 15 | 16QAM, 0.48 | HST-SFN Scheme A | 2x4 | 70 | [9.1] |

<End of Change R4-2217435, R4-2220194>

## <Start Of Change R4-2217436, R4-2220195>

##### 5.2.3.1.x2	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.3.1.x2-3, with the addition of test parameters in Table 5.2.3.1.x2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.x2-1.

Table 5.2.3.1.x2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.x2 | 1-1 |

Table 5.2.3.1.x2-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |
| Note 1: SSB # (k mod 3), CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint#0 {TCI state #0, TCI State #1} is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1. Codepoint#1 {TCI state #1, TCI State #2} is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2. Codepoint#2 {TCI state #2, TCI State #0} is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3. The second indicated TCI state in each codepoint is not used for quasi co-location parameters {Doppler shift, Doppler spread}. |  |  |  |  |

Table 5.2.3.1.x2-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.5 FDD | 10/15 | 16QAM, 0.48 | HST-SFN-SchemeB | 2x4 | 70 | [8.4] |

<End of Change R4-2217436, R4-2220195>

#### 5.2.3.2	TDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217435, R4-2220194>

##### 5.2.3.2.X1	Minimum requirements for HST-SFN Scheme A

The performance requirements are specified in Table 5.2.3.2.X1-3, with the addition of test parameters in Table 5.2.3.2.X1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.X1-1.

Table 5.2.3.2.X1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme A scenario defined in B.3.X1 | 1-1 |

Table 5.2.3.2.X1-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI States TCI state #0, TCI State #1. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with TCI States TCI state #1, TCI State #2. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with TCI States TCI state #2, TCI State #0. |  |  |  |  |

Table 5.2.3.2.X1-3: Minimum performance for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40 / 30 | 16QAM, 0.48 | HST-SFN Scheme A | 2x4 | 70 | [10.3] |

<End of Change R4-2217435, R4-2220194>

## <Start Of Change R4-2217436, R4-2220195>

##### 5.2.3.2.x2	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.3.2.x2-3, with the addition of test parameters in Table 5.2.3.2.x2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.x2-1.

Table 5.2.3.2.x2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.x2 | 1-1 |

Table 5.2.3.2.x2-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Note 1: SSB # (k mod 3), CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint#0 {TCI state #0, TCI State #1} is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1. Codepoint#1 {TCI state #1, TCI State #2} is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2. Codepoint#2 {TCI state #2, TCI State #0} is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3. The second indicated TCI state in each codepoint is not used for quasi co-location parameters {Doppler shift, Doppler spread}. |  |  |  |  |

Table 5.2.3.2.x2-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40/30 | 16QAM, 0.48 | HST-SFN-SchemeB | 2x4 | 70 | [9.3] |

<End of Change R4-2217436, R4-2220195>

#### << Unchanged sections omitted >>

## 5.3	PDCCH demodulation requirements

#### << Unchanged sections omitted >>

### 5.3.2	2RX requirements

#### 5.3.2.1	FDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217438, 2219277>

##### 5.3.2.1.x	Minimum requirements for PDCCH with intra-slot repetition

The performance requirements are specified in Table 5.3.2.1.x-2, with the addition of test parameters in Table 5.3.2.1.x-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.2.1.x-1: Tests parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
|  |  | Repetition transmission schemes |  |  | FDM |  |
|  |  | CCE to REG mapping type |  |  | nonInterleaved |  |
|  |  | REG bundle size |  |  | 6 |  |
|  |  | Time offset/Frequency offset of the second TxRP from the first TxRP |  |  | timing offset = -0.5us, frequency offset = 200Hz |  |
|  |  | Frequency domain resource allocation for CORSET |  |  | Frequency non-overlapping |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Note: PDCCH is transmitted from both TRxP #1 and TRxP #2 |  |  |  |  |  |  |

Table 5.3.2.1.x-2: Minimum performance for PDCCH with 15kHz SCS (Note 2)

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 2x2, ULA Low | 1 | [2.7] |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of Change R4-2217438, R4-2219277>

#### << Unchanged sections omitted >>

#### 5.3.2.2	TDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217438, R4-2219277>

##### 5.3.2.2.x	Minimum requirements for PDCCH with intra-slot repetition

The performance requirements are specified in Table 5.3.2.2.x-2, with the addition of test parameters in Table 5.3.2.2.x-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.2.2.x-1: Tests parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
|  |  | Repetition transmission schemes |  |  | FDM |  |
|  |  | CCE to REG mapping type |  |  | nonInterleaved |  |
|  |  | REG bundle size |  |  | 6 |  |
|  |  | Time offset/Frequency offset of the second TxRP from the first TxRP |  |  | timing offset = -0.25us, frequency offset = 300Hz |  |
|  |  | Frequency domain resource allocation for CORSET |  |  | Frequency non-overlapping |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 40 |  |
|  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 5 and 6 21 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Note: PDCCH is transmitted from both TRxP #1 and TRxP #2 |  |  |  |  |  |  |

Table 5.3.2.2.x-2: Minimum performance for PDCCH with 30kHz SCS (Note 2)

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 40 | 48 | 2 | 2 | R.PDCCH. 2-2.2 TDD | TDLA30-10 | 2x2, ULA Low | 1 | [3.5] |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of Change R4-2217438, R4-2219277>

### 5.3.3	4RX requirements

#### 5.3.3.1	FDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217438, R4-2219277>

##### 5.3.3.1.x	Minimum requirements for PDCCH with intra-slot repetition

The performance requirements are specified in Table 5.3.3.1.x-2, with the addition of test parameters in Table 5.3.3.1.x-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.3.1.x-1: Tests parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
|  |  | Repetition transmission schemes |  |  | FDM |  |
|  |  | CCE to REG mapping type |  |  | nonInterleaved |  |
|  |  | REG bundle size |  |  | 6 |  |
|  |  | Time offset/Frequency offset of the second TxRP from the first TxRP |  |  | timing offset = -0.5us, frequency offset = 200Hz |  |
|  |  | Frequency domain resource allocation for CORSET |  |  | Frequency non-overlapping |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Note: PDCCH is transmitted from both TRxP #1 and TRxP #2 |  |  |  |  |  |  |

Table 5.3.3.1.x-2: Minimum performance for PDCCH with 15kHz SCS (Note 2)

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 2x4, ULA Low | 1 | [-1.2] |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of Change R4-2217438, R4-2219277>

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217438, R4-2219277>

##### 5.3.3.2.x	Minimum requirements for PDCCH with intra-slot repetition

The performance requirements are specified in Table 5.3.3.2.x-2, with the addition of test parameters in Table 5.3.3.2.x-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.2.2.x-1: Tests parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
|  |  | Repetition transmission schemes |  |  | FDM |  |
|  |  | CCE to REG mapping type |  |  | nonInterleaved |  |
|  |  | REG bundle size |  |  | 6 |  |
|  |  | Time offset/Frequency offset of the second TxRP from the first TxRP |  |  | timing offset = -0.25us, frequency offset = 300Hz |  |
|  |  | Frequency domain resource allocation for CORSET |  |  | Frequency non-overlapping |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 40 |  |
|  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 5 and 6 21 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Note: PDCCH is transmitted from both TRxP #1 and TRxP #2 |  |  |  |  |  |  |

Table 5.3.2.2.x-2: Minimum performance for PDCCH with 30kHz SCS

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 40 | 48 | 2 | 2 | R.PDCCH. 2-2.2 TDD | TDLA30-10 | 2x4, ULA Low | 1 | [-1.0] |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of Change R4-2217438, R4-2219277>

#### << Unchanged sections omitted >>

# 6	CSI reporting requirements (Conducted requirements)

## 6.1	General

#### << Unchanged sections omitted >>

### 6.3.2	2RX requirements

#### 6.3.2.1	FDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217437>

##### 6.3.2.1.X	Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme

For the parameters specified in Table 6.3.2.1.X-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.X-2.

Table 6.3.2.1.X-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 20 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |  | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | FDD |  |  |
| Bandwidth |  |  |  |  | MHz | 10 |  |  |
| Subcarrier spacing |  |  |  |  | kHz | 15 |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | 2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (9) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #9}, with K1=1 CMR group #2: {NZP CSI-RS resource #10}, with K2=1 CMR paring: {NZP CSI-RS resource #9, NZP CSI-RS resource #10} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  |  | typeI-SinglePanel |  |  |
|  |  | CodebookMode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |  |
|  |  | RI Restriction |  |  |  | 00000001 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 8 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.1-6.4 |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |  |

Table 6.3.2.1.X-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.6] |

<End of Change R4-2217437>

#### 6.3.2.2	TDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217437>

##### 6.3.2.2.X	Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme

For the parameters specified in Table 6.3.2.2.X-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.X-2.

Table 6.3.2.2.X-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 40 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |  | 20 for CSI-RS resources 5 and 6 21 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | TDD |  |  |
| Bandwidth |  |  |  |  | MHz | 40 |  |  |
| Subcarrier spacing |  |  |  |  | kHz | 30 |  |  |
| TDD DL-UL configurations |  |  |  |  |  | FR1.30-1 as specified in Annex A |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (9) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #9}, with K1=1 CMR group #2: {NZP CSI-RS resource #10}, with K2=1 CMR paring: {NZP CSI-RS resource #9, NZP CSI-RS resource #10} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  |  | typeI-SinglePanel |  |  |
|  |  | CodebookMode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |  |
|  |  | RI Restriction |  |  |  | 00000001 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 6.5 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.2-8.5 TDD |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |  |

Table 6.3.2.2.X-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.6] |

<End of Change R4-2217437>

### 6.3.3	4RX requirements

#### 6.3.3.1	FDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217437>

##### 6.3.3.1.X	Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme

For the parameters specified in Table 6.3.3.1.X-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.X-2.

Table 6.3.3.1.X-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 20 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |  | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | FDD |  |  |
| Bandwidth |  |  |  |  | MHz | 10 |  |  |
| Subcarrier spacing |  |  |  |  | kHz | 15 |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 4 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | 2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (9) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #9}, with K1=1 CMR group #2: {NZP CSI-RS resource #10}, with K2=1 CMR paring: {NZP CSI-RS resource #9, NZP CSI-RS resource #10} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  |  | typeI-SinglePanel |  |  |
|  |  | CodebookMode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |  |
|  |  | RI Restriction |  |  |  | 00000001 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 8 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.1-6.4 |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |  |

Table 6.3.3.1.X-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.6] |

<End of Change R4-2217437>

#### 6.3.3.2	TDD

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217437>

##### 6.3.3.2.X	Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme

For the parameters specified in Table 6.3.3.2.X-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.X-2.

Table 6.3.3.2.X-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 40 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |  | 20 for CSI-RS resources 5 and 6 21 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | TDD |  |  |
| Bandwidth |  |  |  |  | MHz | 40 |  |  |
| Subcarrier spacing |  |  |  |  | kHz | 30 |  |  |
| TDD DL-UL configurations |  |  |  |  |  | FR1.30-1 as specified in Annex A |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 4 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (9) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #9}, with K1=1 CMR group #2: {NZP CSI-RS resource #10}, with K2=1 CMR paring: {NZP CSI-RS resource #9, NZP CSI-RS resource #10} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  |  | typeI-SinglePanel |  |  |
|  |  | CodebookMode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |  |
|  |  | RI Restriction |  |  |  | 00000001 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 6.5 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.2-8.5 TDD |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |  |

Table 6.3.3.2.X-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.6] |

<End of Change R4-2217437>

#### << Unchanged sections omitted >>

# A.3	DL reference measurement channels

#### << Unchanged sections omitted >>

## A.3.2	Reference measurement channels for PDSCH performance requirements

For PDSCH reference channels if more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).

### A.3.2.1	FDD

#### A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217437>

Table A.3.2.1.1-6: PDSCH Reference Channel for FDD PMI reporting requirements

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-6.1 FDD | R.PDSCH.1-6.2 FDD | R.PDSCH.1-6.3 FDD | R.PDSCH.1-6.4 FDD |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 | 52 |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames | Slots | 15 | 15 | 15 | 15 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 13 | 13 | 20 | 13 |  |
| Modulation |  | 16QAM | 16QAM | 64QAM | 16QAM |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.55 | 0.48 |  |
| Number of MIMO layer |  | 1 | 2 | 2 | 2 |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 | 24 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A | N/A |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 12040 | 24072 | 40976 | 24072 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A | N/A |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A | N/A |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..,19} | CBs | 2 | 3 | 5 | 3 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A | N/A |  |
| For Slots i = 10 | Bits | 23712 | 47424 | 71136 | 44928 |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..9,11,…,19} | Bits | 24960 | 49920 | 74880 | 49920 |  |
| Max. Throughput averaged over 2 frames | Mbps | 9.030 | 18.054 | 30.732 | 18.054 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

<End of Change R4-2217437>

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217435>

Table A.3.2.1.1-8: PDSCH Reference Channel for FDD HST scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-8.1 FDD | R.PDSCH.1-8.2 FDD | R.PDSCH.1-8.3 FDD | R.PDSCH.1-8.4 FDD | R.PDSCH.1-8.5 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 | 52 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 17 | 13 | 17 | 13 |
| Modulation |  | 16QAM | 64QAM | 16QAM | 64QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.43 | 0.48 | 0.43 | 0.48 |
| Number of MIMO layers |  | 1 | 1 | 2 | 2 | 2 |
| Number of DMRS REs |  | 18 | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 12552 | 16896 | 25104 | 33816 | 25104 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 2 | 3 | 3 | 5 | 5 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,2,11,12 | Bits | 24960 | 37440 | 49920 | 74880 | 47424 |
| For Slots i = 3,…, 10, 13, …, 19 | Bits | 26208 | 39312 | 52416 | 78624 | 52416 |
| Max. Throughput averaged over 2 frames | Mbps | 11.924 | 16.0512 | 23.8488 | 32.1252 | 23.8488 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<End of Change R4-2217435>

#### << Unchanged sections omitted >>

### A.3.2.2	TDD

#### A.3.2.2.1	Reference measurement channels for SCS 15 kHz FR1

#### << Unchanged sections omitted >>

#### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217437>

Table A.3.2.2.2-8: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR1.30-1

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-8.1 TDD | R.PDSCH.2-8.2 TDD | R.PDSCH.2-8.3 TDD | R.PDSCH.2-8.4 TDD | R.PDSCH.2-8.5 TDD |
| Channel bandwidth | MHz | 40 | 40 | 40 | 20 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 106 | 106 | 106 | 51 | 106 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 23 | 23 | 23 | 23 | 23 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 20 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 64QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.55 | 0.48 | 0.48 |
| Number of MIMO layers |  | 1 | 2 | 2 | 1 | 2 |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i = 20 | Bits | 24576 | 49176 | 83976 | 11784 | 49176 |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24576 | 49176 | 83976 | 11784 | 49176 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i = 20 | Bits | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i = 20 | CBs | 3 | 6 | 10 | 2 | 6 |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | CBs | 3 | 6 | 10 | 2 | 6 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i = 20 | Bits | 48336 | 96672 | 145008 | 23256 | 91584 |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 50880 | 101760 | 152640 | 24480 | 101760 |
| Max. Throughput averaged over 2 frames | Mbps | 28.2624 | 56.5524 | 96.5724 | 13.5516 | 56.5524 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

<End of Change R4-2217439>

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217435>

Table A.3.2.2.2-30: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-30.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 27 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 51216 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 7 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 1,2,21,22 (Note 3) | Bits | 101856 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,20,23,…,39} | Bits | 106848 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 69.1416 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation. |  |  |  |  |  |  |

<End of Change R4-2217435>

#### << Unchanged sections omitted >>

## A.3.3	Reference measurement channels for PDCCH performance requirements

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217439>

### A.3.3.2	TDD

#### A.3.3.2.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.3.2.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 TDD | R.PDCCH.1-1.2 TDD | R.PDCCH.1-1.3 TDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

Table A.3.3.2.1-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 TDD | R.PDCCH.1-2.2 TDD | R.PDCCH.1-2.3 TDD | R.PDCCH.1-2.4 TDD | R.PDCCH.1-2.5 TDD | R.PDCCH.1-2.6 TDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 |

#### A.3.3.2.2	Reference measurement channels for SCS 30 kHz FR1

Table A.3.3.2.2-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-1.1 TDD | R.PDCCH.2-1.2 TDD | R.PDCCH.2-1.3 TDD | R.PDCCH.2-1.4 TDD |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |  |  |
| CORESET frequency domain allocation |  | 102 | 102 | 90 | 102 |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 |  |  |
| Aggregation level |  | 2 | 4 | 8 | 8 |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 2_6 |  |  |
| Payload (without CRC) | Bits | 41 | 53 | 53 | 12 |  |  |

Table A.3.3.2.2-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-2.1 TDD | R.PDCCH.2-2.2 TDD |  |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 |  |  |  |  |
| CORESET time domain allocation |  | 2 | 2 |  |  |  |  |
| Aggregation level |  | 16 | 2 |  |  |  |  |
| DCI Format |  | 1_0 | 1_0 |  |  |  |  |
| Payload (without CRC) | Bits | 41 | 41 |  |  |  |  |

<End of Change R4-2217439>

#### << Unchanged sections omitted >>

# B.3	High Speed Train Scenario

#### << Unchanged sections omitted >>

## <Start Of Change R4-2217435, R4-2220194>

## B.3.X1	HST-SFN Scheme A Channel Profile

There is an infinite number of RRHs distributed equidistantly along the railway track with the same Cell ID as illustrated in Figure B.3.X1-1. The PDCCH/PDSCH are transmitted in SFN manner and TRS are transmitted in TRP specific manner as illustrated in Figure B.3.X1-1.

Figure B.3.X1-1: Deployment of HST-SFN Scheme A

The location of RRH k is given as:

xk=k*Ds+j*Dmin	(B.3.X1.1)

where:	,  and Dmin is the distance between the RRHs and railway track, while Ds  is the distance of two RRHs, both in meters.

The train location is denoted as:

y=a+j*0	(B.3.X1.2)

where:	 and a means distance in meters, which means the train is right on the track.

The HST Scheme A scenario for the test of the baseband performance is a non-fading propagation channel with two taps, namely the two nearest RRHs. Thus, RRH k is visible for the train only in the range:

k*Ds-Ds≤a<k*Ds+Ds	(B.3.X1.3)

Power level Pk (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs, is given by:

Pk= -20logy-xk-10logi∈ii*Ds-Ds≤a<i*Ds+Ds1y-xi2 for k*Ds-Ds≤a<k*Ds+Ds			(B.3.X1.4)

Doppler shift (Hz) from kth RRH for TRSk, PDCCH, PDSCH as shown in figure B.3.X1-1 is given by:

FD,k= fc*real-v*y-xky-xk*C     for    k*Ds-Ds≤a<k*Ds+Ds       			                             (B.3.X1.5)

The relative delay Tk (s) for the signal from kth RRH can be derived as:

Tk= |y-xk|C      for      k*Ds-Ds≤a<k*Ds+Ds					(B.3.X1.6)

In the above v (m/s) is the moving speed of the train, fC (Hz) is the center frequency, and C (m/s) is the velocity of light.

Power level, Doppler shift and relative delay are given by equations B.3.X1.4 ~ B.3.X1.6 respectively, where the required input parameters listed in table B.3.X1-1 and the resulting Doppler shift shown in Figures B.3.X1-3 and B.3.X1-4 are applied for all frequency bands.

Table B.3.X1-1: HST-SFN scenario

| Parameter | Value |
| --- | --- |
|  | 700 m |
|  | 150 m |
|  | 500 km/h |
|  | 870 Hz for 15 kHz SCS test; 1667 Hz for 30 kHz SCS test |

NOTE 1:	Equation B.3.X1.5 is a general formula for Doppler shift calculation. When defining the requirement for FR1 HST, max Doppler shifts fd which corresponds to fc*v/C in equation B.3.X1.5 are selected as defined in Table B.3.X1-1 for HST-SFN Scheme A scenario.

Figure B.3.X1-2 Relative power level trajectories

Figure B.3.X1-3 Doppler shift trajectory (fd= 870 Hz) showing visibility of each RRH

Figure B.3.X1-4 Doppler shift trajectory (= 1667 Hz) showing visibility of each RRH

Figure B.3.X1-5 Absolute delay trajectories

Static channel matrix will be used as defined in Annex B.1.

<End of Change R4-2217435, R4-2220194>

## <Start Of Change R4-2217436, R4-2220195>

## B.3.x2	HST-SFN Scheme B Channel Profile

There is an infinite number of RRHs distributed equidistantly along the track with the same Cell ID as depicted in figure B.3.x2-1. The PDCCH/PDSCH are transmitted in SFN manner from two RRHs and TRS are transmitted in RRH specific manner as illustrated in Figure B.3.X2-1.

Figure B.3.x2-1: Deployment of HST-SFN Scheme B

The location of RRH k is given as:

xk=k*Ds+j*Dmin	(B.3.x2.1)

where:k∈[-∞,  +∞], j=sqrt(-1) and Dmin is the distance between the RRHs and railway track, while Ds is the distance of two RRHs, both in meters.

The train location is denoted as:

y=a+j*0	(B.3.x2.2)

where:	a∈[0,  +∞] and a means distance in meters, which means the train is right on the track.

The HST-SFN Scheme B scenario for the test of the baseband performance is a non-fading propagation channel with two taps, namely the two nearest RRHs. Thus, RRH k is visible for the train only in the range:

k*Ds-Ds≤a<k*Ds+Ds	(B.3.x2.3)

Power level Pk (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs, is given by:

Pk= -20logy-xk-10logi∈ii*Ds-Ds≤a<i*Ds+Ds1y-xi2for k*Ds-Ds≤a<k*Ds+Ds						(B.3.x2.4)

Doppler shift FD,k (Hz) from kth RRH for TRSk, PDCCH and PDSCH transmission and Doppler shift FD,k+1 (Hz) from (k+1)th RRH for PDCCH and PDSCH transmission as illustrated in Figure B.3.X2-1 is given by:

FD,k=fc×real-v×y-xky-xk×CFD,k+1=FD,k for k*Ds≤a<k*Ds+Ds	(B.3.x2.5)

The relative delay Tk (s) for the signal from kth RRH can be derived as:

Tk= |y-xk|C for k*Ds-Ds≤a<k*Ds+Ds	(B.3.x2.6)

In the above v (m/s) is the moving speed of the train, fC (Hz) is the center frequency, and C (m/s) is the velocity of light.

Power level, Doppler shift and relative delay are given by equations B.3.x2.4 ~ B.3.x2.6 respectively, where the required input parameters listed in table B.3.x2-1 and the resulting Doppler shift shown in Figures B.3.x2-3 and B.3.x2-4 are applied for all frequency bands.

Table B.3.x2-1: HST-SFN Scheme B scenario

| Parameter | Value |
| --- | --- |
|  | HST-SFN-SchemeB |
| Ds | 700 m |
| Dmin | 150 m |
| v | 500 km/h |
| fd | 870 Hz for 15 kHz SCS test; 1667 Hz for 30 kHz SCS test |

NOTE 1:	The trajectories of ralative power, Doppler shifts and absolute delays presented in Figures B.3.x2-2, B.3.x2-3, B.3.x2-4 and B.3.x2-5 are derived from the equations B.3.x2.4 ~ B.3.x2.6 respectively.

NOTE 2: Equation B.3.x2.5 is a general formula for Doppler shift calculation. When defining the requirement for FR1 HST, max Doppler shifts fd which corresponds to fc*v/C in equation B.3.x2.5 are selected as defined in Table B.3.x2-1 for HST-SFN scenario.

Figure B.3.x2-2 Relative power level trajectories

Figure B.3.x2-3 Doppler shift trajectories (fd = 870 Hz)

Figure B.3.x2-4 Doppler shift trajectories (fd = 1667 Hz)

Figure B.3.x2-5 Absolute delay trajectories

Static channel matrix will be used as defined in Annex B.1.

<End of Change R4-2217436, R4-2220195>