3GPP TSG-RAN WG4 Meeting # 105 -e	R4-2219087

Electronic Meeting, November 14th – 18th, 2022

Title:	TP to TR 38.717-02-01: Addition of CA_n25-n77

Source:	Nokia, Bell, Telus

Agenda item:	7.1.1.1

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-02-01 to add CA_25A-n77(3A) and CA_n25(2A)-n77(3A) with UL CA.

************************************* Start of TP*****************************************

## 5.x		CA_n25-n77

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1 Operating bands for CA

Table 5.X.1.1-1: CA band combination of band n25+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n25+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n25A-n77(3A) | CA_n77(2A) CA_n25A-n77A | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 0 |
|  |  | n77 | See CA_n77(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |
| CA_n25(2A)-n77(3A) | CA_n25(2A) CA_n77(2A) CA_n25A-n77A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 | 0 |
|  |  | n77 | See CA_n77(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |

#### 5.x.1.3	Co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n25-n77.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, the 2nd UL harmonic of n25 falls inside the n77 RX band.

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 | 9650 | 9975 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is harmonic issue for CA_n25-n77 having the 4th UL harmonic of n25 fall inside the 2nd harmonic of the DL of RX band n77.

Based on above table, there is harmonic issue for CA_n25-n77 having the fundamental TX of n77 fall inside the 2nd harmonic DL of n25 and the 2nd UL harmonic of n77 fall inside the 4th harmonic DL of RX band n25.

Both the UL harmonic and the harmonic mixing issue is known and captured in 38.101-1. There’s nothing new to adding n77(2A) to these requirements

#### 5.x.1.4	∆TIB and ∆RIB values

For CA_n25-n77, the TIB,c and RIB values are already specified in 38.101-1. There’s nothing new to having n77(2A) to these requirements. The values remain as captured below.

Table 5.x.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n25-n77 | 0.6 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n25-n77 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5	REFSENs requirements

There’s nothing new to adding n77(2A) to these requirements.

#### 5.x.1.6	OOB blocking exception requirements

There’s nothing new to adding n77(2A) to these requirements.

### 5.x.2		Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

There’s nothing new to adding n77(2A) to these requirements. The requirements are already covered in 38.101-1.

Table 5.x.2A.1.1-1: UE Power Class for intraband non-contiguous CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77(2A) | 23 | +2/-3 |

Table 5.x.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n77A |  |  |  |  | 23 | +2/-3 |  |  |

#### 5.x.2.2		UE co-existence

Table 5.x.2.2-1 gives IMD interference analysis for CA_ n25-n77 with 2 ULs in n77 having a spectrum restriction of 600MHz separation.

Table 5.x.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low+Ch | fy_low+MaxD | fy_high-MaxD | fy_high-Ch |
| --- | --- | --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| DL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| 2nd order IMD products | ¦fy_low+Ch – fx_low¦ | ¦fy_low+MaxD – fx_low¦ | ¦fy_low+Ch + fx_low¦ | ¦fy_high-Ch + fx_high¦ |
| IMD frequency limits (MHz) | 100 | 600 | 6700 | 8300 |
| 3rd order IMD products | ¦2*fx_low + fy_low+Ch¦ | ¦2*fx_high + fy_high-Ch¦ | ¦2*fx_low - fy_low+MaxD¦ | ¦2*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 10000 | 12500 | 2700 | 4800 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low+Ch¦ | ¦3*fx_high + fy_high-Ch¦ | ¦3*fx_low - fy_low+MaxD¦ | ¦3*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 13300 | 16700 | 6000 | 9000 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_low+Ch¦ | ¦2*fx_low - 2*fy_low+MaxD¦ | ¦2*fx_low + 2*fy_low+Ch¦ | ¦2*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 200 | 1200 | 13400 | 16600 |
| Two-tone 5th order IMD products | ¦4*fx_low + fy_low+Ch¦ | ¦4*fx_high + fy_high-Ch¦ | ¦4*fx_low – fy_low+MaxD¦ | ¦4*fx_high – fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 16600 | 20900 | 9300 | 13200 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_low+MaxD¦ | ¦3*fx_high – 2*fy_high-MaxD¦ | ¦2*fx_low + 3*fy_low+Ch¦ | ¦3*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 2100 | 5400 | 16800 | 20800 |

Based on the table 6.x.2.2-1, there is no intermodulation products of n77 up to 5th order that fall inside the RX band of n25.

Table 5.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration. There’s nothing new to adding n77(2A) to these requirements.

Table 5.x.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n77 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

#### 5.x.2.3		REFSENS requirements

There is no additional Refsens requirements to CA_n25A-n77(3A) and CA_n25(2A)-n77(3A)

************************************* End of TP*****************************************