3GPP TSG-RAN WG4 Meeting # 105 -e	R4-2219103

Electronic Meeting, November 14th – 18th, 2022

Title:	TP to TR 37.718-11-21: Addition of DC_8_n38-n40

Source:	Nokia, Telefonica

Agenda item:	7.6.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.718-11-21 to add DC_8_n38-n40 with 2UL CA. This TP has dependencies to the fallback DC_8A_n38A submitted for the same meeting.

************************************* Start of TP*****************************************

## 6.x		DC_8_n38-n40

### 6.x.1	 Operating bands for DC

Table 6.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n38-n40 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 6.x.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n38A-n40A | DC_8A_n38A DC_8A_n40A |

### 6.x.2 	Configuration for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n38A-n40A |  | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.x.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_8_n38 are shown below.

Table 6.x.3-1: The harmonic and IMD products caused by UL DC_8_n38

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 880 | 915 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 12850 | 13100 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 5280 | 5490 | 15420 | 15720 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 6160 | 6405 | 17990 | 18340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1655 | 1740 | 3450 | 3535 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 860 | 740 | 4225 | 4360 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4330 | 4450 | 6020 | 6155 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 20 | 175 | 6795 | 6980 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3480 | 3310 | 6900 | 7070 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5210 | 5365 | 8590 | 8775 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9600 | 9365 | 1090 | 900 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6100 | 5880 | 2395 | 2600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11160 | 11395 | 6090 | 6280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9470 | 9690 | 7780 | 7985 |

The harmonic and IMD products caused by UL DC_8_n40 are shown below.

Table 6.x.3-2: The harmonic and IMD products caused by UL DC_8_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 880 | 915 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 11500 | 12000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 5280 | 5490 | 13800 | 14400 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 6160 | 6405 | 16100 | 16800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1385 | 1520 | 3180 | 3315 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 640 | 470 | 3685 | 3920 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4060 | 4230 | 5480 | 5715 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 240 | 445 | 5985 | 6320 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3040 | 2770 | 6360 | 6630 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4940 | 5145 | 7780 | 8115 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8720 | 8285 | 1360 | 1120 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5440 | 5070 | 1855 | 2160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10080 | 10515 | 5820 | 6060 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8660 | 9030 | 7240 | 7545 |

The 5th order IMD product 2*2572MHz – 3*912MHz = 2408MHz will have a BW of the interference of 25MHz, which falls inside n40 upper channel.

There are no intermodulation products of DC_8_n40 falling inside the RX band of n38.

### 6.x.4	 ∆TIB and ∆RIB values

Referring to CA_n8A-n38A-n40A, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.x.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8_n38-n40 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.x.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8_n38-n40 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 6.x.5	 MSD requirements

Referring to CA_n8-n38-n40, the MSD requirement that addresses the 5th order product has been omitted. For DC_8_n38-n40 the IMD5 is captured in Table 6.x.5-1:

Table 6.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n38A-n40A | 8 | 912 | 5 | 25 | 957 | N/A | N/A |
|  | n38 | 2572 | 5 | 25 | 884 | N/A | N/A |
|  | n40 | 2397 | 5 | 25 | 2397 | 3.2 | IMD5 |

************************************* End of TP******************************************