3GPP TSG-RAN WG4 Meeting #105	R4-2219120

Toulouse, France, 14 November – 18 November 2022

Agenda Item:	8.17.1

Source:	Ericsson

Title:	Discussion on application layer throughput requirements

Document for:	Discussion

# 1	Introduction

One of the objectives of Rel-18 NR demodulation performance evolution WI is to specify the absolute physical layer throughput requirements with link adaptation [1].

- Specify absolute physical layer throughput requirements with link adaptation - Note: Rel-17 RAN4 study outcome documented in section 5.10 of TR 37.901-5 is a starting point for this objective

This contribution discusses our view on the application layer throughput requirements.

# 2	Discussion

## 2.1	Outcome of Rel-17 SI 5G NR UE Application Layer Data Throughput Performance

In Rel-17 SI [3], RAN4 studied the feasibility of defining link adaptation absolute physical layer requirements. In the SI, RAN4 discussed

- Alignment criteria for aligning the simulation results across companies, and

- Methodology to define the final requirements, if it is found to be feasible to define such requirements.

In the study, RAN4 compared the span of SNR to achieve X% of maximum throughput with the existing rank indication reporting test cases where all the CSI reporting restrictions (e.g., codebook/RI restrictions) are removed.

According to the simulation campaign, RAN4 concluded that it is feasible to define the absolute physical layer throughput requirements under link adaptation (TR37.901-5 5.10 [2]), where the recommended requirements are SNR to achieve T% of maximum throughput. We therefore propose to reuse the conclusion of Rel-17 SI to define the absolute physical layer throughput requirements.

Proposal 1: RAN4 define the absolute physical layer throughput requirements with link adaptation by reusing the outcome of Rel-17 SI 5G NR UE Application Layer Data Throughput Performance to:

- SNR (X dB) to achieve T (%) of the maximum throughput, e.g., T=40 (%) at X=20 (dB).

- The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank Y for 2Rx/4Rx UE, e.g., Y=2 for both 2Rx/4Rx UEs.

## 2.2	Scope of Rel-18 RAN4 demodulation requirements

Test scenarios

In Rel-17 SI, RAN4 have studied the feasibility of defining absolute throughput requirements based on the parameters used for Rel-15 RI reporting requirements. We think it is straightforward to reuse the parameters used in the SI.

Proposal 2: Define absolute physical layer throughput requirements with link adaptation with the following configuration listed in TR37.901-5 Table 5.10-3-1:

- Test 1: FR1 FDD, SCS/CBW=15kHz/10MHz, 2Tx, 2Rx/4Rx

- Test 2: FR1 TDD, SCS/CBW=30kHz/40MHz, 2Tx, 2Rx/4Rx, TDD UL/DL configuration: 7D1S2U

- Test 3: FR2 (Including both FR2-1 and FR2-2) TDD, SCS/CBW=120kHz/100MHz, 2Tx, 2Rx, TDD UL/DL configuration: DDSU

CSI configuration

In the link adaptation tests, TE schedules PDSCH MCS/precoder/rank according to the reported CQI/PMI/RI (follow CSI). Since the purpose of this test is to verify that UE estimates the proper CSI and UE decodes PDSCH with the estimated CSI with proper BLER, we should not put any restriction on PMI/RI reporting. Regarding the configured CQI table, since it is the first release to define the absolute throughput requirements, RAN4 should focus on the mandatory features specified in Rel-15. Moreover as same as Rel-17 SI, we should set the maximum number of HARQ transmission to 1 to verify the reported CSI accuracy.

Proposal 3: Define absolute physical layer throughput requirements with link adaptation with the following CSI reporting configurations:

- Configure CQI table 2 for FR1

- Configure CQI table 1 for FR2

- Configure Type I Single-Panel Codebook

- Not configure CodebookSubsetRestriction

- Set the maximum number of HARQ transmission to 1

Test metric

In the CSI reporting tests, RAN4 usually set two test points (higher SNR and lower SNR test points) to verify the UE behavior. To verify the UE performance on both cell-edge and cell-center, we propose to define the absolute throughput performance requirements with two test points. For higher SNR test points, we propose to reuse the existing RI test cases, i.e., 20dB for FR1 and 16dB for FR2. Regarding the lower SNR test points, the existing RI tests set SNR=0dB for both FR1 and FR2. We think it is relatively low and the reported CSI, especially PMI, might not be accurate. We therefore propose to set SNR=6dB to ensure the reliable CSI reporting. However if the simulation results are not well aligned, RAN4 should discuss whether to set lower SNR test points.

Regarding the test metric, i.e., T% of the maximum throughput, RAN4 should decide them according to the simulation result summary.

Proposal 4: RAN4 consider the following test points and metrics:

- SNR=6dB/20dB for FR1, SNR=6dB/16dB for FR2

  - RAN4 discuss whether to specify lower SNR test point case if the companies’ simulation results are not well aligned.

- Requirements (T% of the maximum throughput) depend on the simulation result summary

Channel model

In Rel-17 SI, because of the limited time, RAN4 studied only TDLA30-5 for FR1 and TDLA30-35 for FR2, by reusing the Rel-15 RI reporting requirements. In our understanding, one of the motivations of this absolute throughput requirement is to introduce the conformance tests that measure throughput close to what would be experienced by a user [3]. Especially in FR1, UEs can be located in larger delay spready and/or high Doppler conditions. We therefore propose to consider larger delay spread and Doppler spread channel modes as same as PDSCH demodulation requirements, e.g., TDLB100-400 or TDLC300-100.

Proposal 5: For FR1, use TDLA30-5. Also consider TDLB100-400 or TDLC300-100.

Proposal 6: For FR2, use TDLA30-35.

The simulation assumption we propose is summarized in Table 1.

Table 1	Simulation assumption for alignment (based on Table 5.10-3-1 in [2])

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0 to 20dB with 2dB step | 0 to 20dB with 2dB step | 0 to 16dB with 2dB step |
| Propagation channel |  |  | TDLA30-5 TDLB100-400 TDLC300-100 | TDLA30-5 TDLB100-400 TDLC300-100 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput (Note 3) at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). Note 3: The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank 2. |  |  |  |  |  |

## 2.3	Where the requirements specified in TS38.101-4

Since the absolute throughput performance requirements with link adaption is a combination of PDSCH demodulation and CSI reporting requirements, RAN4 should discuss where the requirements to be specified in TS38.101-4.

One option is to create a new clause like Clause 11 including both FR1 and FR2. Another option is to specify the requirements under the CSI reporting such as 6.x for FR1 and 8.x for FR2. We slight prefer to specify the requirements under the CSI reporting requirements because the test framework is very close to the existing RI reporting requirements.

Proposal 7: RAN4 specify the absolute physical layer throughput requirements with link adaptation under the CSI reporting requirements in TS38.101-4, that is, clause 6.x for FR1 and clause 8.x for FR2.

# 4	Summary

Proposal 1: RAN4 define the absolute physical layer throughput requirements with link adaptation by reusing the outcome of Rel-17 SI 5G NR UE Application Layer Data Throughput Performance to:

- SNR (X dB) to achieve T (%) of the maximum throughput, e.g., T=40 (%) at X=20 (dB).

- The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank Y for 2Rx/4Rx UE, e.g., Y=2 for both 2Rx/4Rx UEs.

Proposal 2: Define absolute physical layer throughput requirements with link adaptation with the following configuration listed in TR37.901-5 Table 5.10-3-1:

- Test 1: FR1 FDD, SCS/CBW=15kHz/10MHz, 2Tx, 2Rx/4Rx

- Test 2: FR1 TDD, SCS/CBW=30kHz/40MHz, 2Tx, 2Rx/4Rx, TDD UL/DL configuration: 7D1S2U

- Test 3: FR2 (Including both FR2-1 and FR2-2) TDD, SCS/CBW=120kHz/100MHz, 2Tx, 2Rx, TDD UL/DL configuration: DDSU

Proposal 3: Define absolute physical layer throughput requirements with link adaptation with the following CSI reporting configurations:

- Configure CQI table 2 for FR1

- Configure CQI table 1 for FR2

- Configure Type I Single-Panel Codebook

- Not configure CodebookSubsetRestriction

- Set the maximum number of HARQ transmission to 1

Proposal 4: RAN4 consider the following test points and metrics:

- SNR=6dB/20dB for FR1, SNR=6dB/16dB for FR2

  - RAN4 discuss whether to specify lower SNR test point case if the companies’ simulation results are not well aligned.

- Requirements (T% of the maximum throughput) depend on the simulation result summary

Proposal 5: For FR1, use TDLA30-5. Also consider TDLB100-400 or TDLC300-100.

Proposal 6: For FR2, use TDLA30-35.

Proposal 7: RAN4 specify the absolute physical layer throughput requirements with link adaptation under the CSI reporting requirements in TS38.101-4, that is, clause 6.x for FR1 and clause 8.x for FR2.

# References

- RP-222300, “Revised WID on NR demodulation performance evolution”, China Telecom.

- TR37.901-5, “Study on 5G NR User Equipment (UE) application layer data throughput performance”, 3GPP

- RP-212470, “Revised SID on 5G NR UE Application Layer Data Throughput Performance”, Qualcomm Inc.