3GPP TSG-RAN WG4 Meeting # 105	R4-2219137

# Toulouse, France, November 14 – November 18, 2022

Agenda item:	6.2.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on synchronization raster ambiguity in NTN

Document for:	Approval

# Introduction

In NTN network, the Doppler shift can be up to +/-24 ppm [1]. The larger frequency error will be seen for NTN UE initial cell search which leads to synchronization raster ambiguity issue. In this paper, we provide our considerations on the synchronization raster ambiguity in NTN network.

# Discussion

The Doppler shift of up to +/-24 ppm is added on top of Crystal Oscillator (XO) error which is +/- 5ppm (typical worst case), or 20ppm (worst-worst case where factory calibration is broken, or some low-cost device did not perform factory calibration). Therefore, the total uncertainty becomes +/- 29ppm (typical worst case) or +/-44ppm (worst-worst case). Then the frequency error for different cases can be calculated in below table:

Table 1: frequency error in NTN network

|  | ppm | Freq error at 2GHz (kHz) | Freq error at 3GHz (kHz) |
| --- | --- | --- | --- |
| Doppler shift | 24 | 48 | 72 |
| XO error (typical worst) | 5 | 10 | 15 |
| XO error (worst-worst) | 20 | 40 | 60 |
| Doppler shift + XO error (typical worst) | 29 | 58 | 87 |
| Doppler shift + XO error (worst worst) | 44 | 88 | 132 |

In NTN band, as shown in Figure 1, spacing between adjacent synchronization raster is 100kHz and the GSCN step size is 1.

Figure 1: GSCN parameters and SS raster in TS 38101-5

Due to large frequency error during initial cell search shown in Table 1, UE cannot unambiguously determine synchronization raster after PSS detection.

Figure 2: SS raster ambiguity in PSS detection

Instead, UE will have to try SSS search and PBCH/SIB1 decoding for up to 3 nearby synch raster hypotheses. With wrong synch raster assumption, UE may fail SIB1 decoding. UE can confirm synch raster from absoluteFrequencySSB in SIB1only after correctly decoding SIB1. The example procedure is as following:

Figure 3: Example procedure for SIB1 decoding due to the SS raster ambiguity

Observation 1: Due to large frequency error during initial cell search, UE cannot unambiguously determine synchronization raster after PSS detection.

Observation 2: UE will have to try SSS search and PBCH/SIB1 decoding for up to 3 nearby synch raster hypotheses which is not efficient for initial cell search in NTN.

Proposal: RAN4 to study and solve the synchronization raster ambiguity issue for NTN network.

# Conclusion

In this paper, we provided our considerations on the synchronization raster ambiguity in NTN network. We had the following observations and proposal:

Observation 1: Due to large frequency error during initial cell search, UE cannot unambiguously determine synchronization raster after PSS detection.

Observation 2: UE will have to try SSS search and PBCH/SIB1 decoding for up to 3 nearby synch raster hypotheses which is not efficient for initial cell search in NTN.

Proposal: RAN4 to study and solve the synchronization raster ambiguity issue for NTN network.

# References

- TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, V16.0.0