3GPP TSG-RAN WG4 Meeting # 105                                                            R4-2219204

# Toulouse, France, November 14 – November 18, 2022

Source:	ZTE Corporation

Title:	TP for TR 38.718-03-01_3DL_2UL CA_n39A-n41A-n79

Agenda Item:	7.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n39A-n41A-n79 in TR 38.718-03-01.

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2),v0.0.1, v0.2.0

- Text Proposal

----- Start of TP -----

  - 5.x			CA_n39-n41-n79

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n39+n41+n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n39A-n41A-n79A | CA_n39A-n41A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

For  CA_n39A-n41A-n79, the TIB,c and RIB,c values have already been included in the TS38.101-1.

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

Due to synchronization between band n39 and n41, so there are no IMD issues for n41caused by n39-n79, or n39 caused by n41-n79.

However,  asynchronization between band n39 and n79 or n41 and n79, in terms of the co-existence studies of CA_n39-n41, it can be observed:

IMD2 caused by 2UL  n39+n41 may fall into the its own band n79 Rx (4376MHz~4610MHz)

      - 5.x.2.2	REFSENS requirements

In terms of the co-existence studies, the impacted n79 Rx frequency range is from 4376MHz~4610MHz caused by 2UL n39 and n41. However, in China, the n79 frequency range is restricted to 4800-5000MHz. Therefore, considering this frequency range, n79 will not be impacted by IMD2 caused by 2UL n39 and n41. In the other words, there is no need to define the IMD2 MSD for CA n39A-n41A-n79. The MSD requirement are defined in table 5.x.2.2-1:

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n41-n79 | n39 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n41 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | TDD | IMD2x |
| NOTE x: There is no IMD2 product in band n79 downlink for n79 operating in 4800 – 5000 MHz frequency range. |  |  |  |  |  |  |  |  |

----- End of TP -----