| 3GPP TR 38.899 V0.0.1 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power UE for FR1 NR inter-band CA/DC or NR SUL band combination with y (1<y<=6) bands DL and x (x=1, 2) bands UL and power class m (m<3) and high power on TDD band(s); (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	TR Maintenance	7

5	High Power UE CA of 2 bands DL and 1 or 2 bands UL	8

5.x	CA_nX-nY	8

5.x.1	Configurations	8

5.x.2	Maximum output power	8

5.x.3	REFSENS requirements	8

5.x.3.1	Power class 2 case a	8

5.x.3.2	Power class 2 case b	8

5.x.3.3	Power class 2 case c	8

5.x.3.4	Power class 1.5 for single uplink nX	8

5.x.3.5	Power class 1.5 for single uplink nY	8

5.x.4	∆TIB and ∆RIB values	9

6	High Power UE CA of 3 bands DL and 1 or 2 bands UL	9

6.x	CA_nX-nY-nZ	9

6.x.1	Configurations	9

6.x.2	Maximum output power	9

6.x.3	REFSENS requirements	9

6.x.3.1	Power class 2 case a	9

6.x.4	∆TIB and ∆RIB values	9

7	Power class 2 CA with SUL	10

7.x	CA_nW-nX_SUL_nY-nZ	10

7.x.1	Configurations	10

7.x.2	Maximum output power	10

7.x.3	REFSENS requirements	10

7.x.4	∆TIB and ∆RIB values	10

Annex <A> (informative): Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for high power UE for NR inter-band Carrier Aggregation or Dual Connection and NR SUL band combinations with y (1<y<=6) bands downlink and x (x=1, 2) bands uplink under Rel-18 time-frame. The purpose is to gather the relevant background information and studies in order to complete the band-combination specific requirements for the newly requested band combinations for power class 2 and power class 1.5 UE. Table 1-1 lists the three types of band combinations covered by this TR.

Table 1-1 High power UE inter-band CA/DC and SUL band combinations

| # | Band combination | Power class for uplink |
| --- | --- | --- |
| 1 | Power class 2 NR Inter-band CA/DC for y bands DL with x bands UL (x=1, 2) | 1UL(TDD): PC2 on TDD band 2UL (FDD+TDD, TDD+FDD, TDD+TDD): PC3 on FDD band, PC2 or PC3 on TDD band |
| 2 | Power class 1.5 NR Inter-band CA for y bands DL with 1 band UL | 1UL (TDD): PC1.5 on TDD band |
| 3 | Power class 2 SUL band combinations with or without CA for y bands DL with 2 bands UL | SUL: PC3 on SUL band NUL(TDD): PC2 on TDD band NUL = Normal Uplink in contrast to SUL. |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-222623: “New WID: High power UE for FR1 NR inter-band CA/DC or NR SUL (supplementary uplink) band combination with y bands downlink (1<y<=6) and x bands uplink (x=1,2) and power class m (m<3) and high power on TDD band(s)”.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

BS				Base Station

BCS				Bandwidth Combination Set

CA				Carrier Aggregation

CC				Component Carriers

DC				Dual Connectivity

DL				Downlink

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

REFSENS				Reference Sensitivity power level

SCS				Subcarrier spacing

TDD				Time Division Duplex

UE				User Equipment

UL				Uplink

# 4	Background

At 3GPP RAN4#97-e meeting, a basket Work Item on “High power UE for FR1 NR inter-band CA/DC or NR SUL (supplementary uplink) band combination with y bands downlink (1<y<=6) and x bands uplink (x=1,2) and power class m (m<3) and high power on TDD band(s)” [2] was approved for Rel-18. The objectives of the core part are as follows:

Specify the band-combination specific RF requirements for all listed band combinations as defined in attached excel file of this WI.

The requirements that need to analyse and specify include

  - Maximum output power.

  - Analysing combinations that have self-desensitization, applicable ∆TIB, c and ∆RIB, c and reference sensitivity exceptions including MSD test cases.

  - Other additional impact to the requirements due to the high power on UL, if necessary

The present document is a technical report for this basket Work Item.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

# 5	High Power UE CA of 2 bands DL and 1 or 2 bands UL

## 5.x	CA_nX-nY

### 5.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.1-1x unless additional clarification>

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY | nX8,9, nY8,9, CA_nX-nY8 | nX |  |  |
|  |  | nY |  |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.x.2	Maximum output power

<Editor’s note: In table 5.x.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

#### 5.x.3.1	Power class 2 case a

#### 5.x.3.2	Power class 2 case b

#### 5.x.3.3	Power class 2 case c

#### 5.x.3.4	Power class 1.5 for single uplink nX

#### 5.x.3.5	Power class 1.5 for single uplink nY

### 5.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

# 6	High Power UE CA of 3 bands DL and 1 or 2 bands UL

## 6.x	CA_nX-nY-nZ

### 6.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.2-1x unless additional clarification>

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY-nZ | nX7,9, nY7,9, CA_nX-nY7 | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.x.2	Maximum output power

<Editor’s note: In table 6.x.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. The requirements in this TR are intended to be power class 2 cases based, however how to address in the spec will be further discussed. >

#### 6.x.3.1	Power class 2 case a

### 6.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

# 7	Power class 2 CA with SUL

## 7.x	CA_nW-nX_SUL_nY-nZ

### 7.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5C-4 unless additional clarification>

Table 7.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink configuration | NR Band | Channel Bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nW-nX_SUL_nY-nZ | SUL_nY-nZ | nW |  |  |
|  |  | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |

### 7.x.2	Maximum output power

### 7.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

### 7.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

# Annex <A> (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-11 | RAN4 #105 | R4-2219272 |  |  |  | TR skeleton | 0.0.1 |