3GPP TSG-RAN WG4 Meeting#105                              R4-2219365

Toulouse, France,14th Nov – 18th Nov,2022

Agenda item:	9.5.4

Source: 	ZTE Corporation

Title: 	Further discussion on SAN RF requirements for IoT over NTN

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221556] on NB-IoT/eMTC core & performance requirements for NTN was approved as one of Rel-18 RAN4 package. During the last RAN4 meeting, we have reached good consensus on SAN RF requirements for IoT over NTN, however there are still some remaining issues left for further discussions. In this contribution,we we want to share some further considerations on these issues.

Discussion

## 2.1. ACLR/ACS

For SAN DL ACLR requirement, it is mainly dependent on the coexistence case 3. In case 3, NTN SAN DL transmission would cause the interference to TN UE. The effective ACLR of NTN SAN is according to the section 6.3 of TR 36.802[3] . The simulation result for Case 3 are shown as the following table.

| Case 3 | ACLR [dB] | 0 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Rural TN_AAS_DL | Average throughput loss | 22.88 | 10.10 | 3.80 | 1.25 | 0.41 | 0.13 | 0.05 | 0.01 |
|  | Cell edge through loss | 31.78 | 13.49 | 4.91 | 1.49 | 0.50 | 0.16 | 0.05 | 0.02 |
| GEO_Rural TN_non_AAS_DL | Average throughput loss | 31.09 | 13.88 | 5.25 | 1.80 | 0.60 | 0.15 | 0.06 | 0.01 |
|  | Cell edge through loss | 53.77 | 26.93 | 10.44 | 3.41 | 0.89 | 0.29 | 0.11 | 0.05 |
| GEO_Urban_macro TN_AAS_DL | Average throughput loss | 0.24 | 0.09 | 0.04 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Cell edge through loss | 0.04 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| GEO_Urban_macro TN_non_AAS_DL | Average throughput loss | 0.28 | 0.08 | 0.02 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Cell edge through loss | 0.23 | 0.19 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| LEO600KM_Rural TN_AAS_DL | Average throughput loss | 65.07 | 43.80 | 24.39 | 10.97 | 4.16 | 1.39 | 0.44 | 0.15 |
|  | Cell edge through loss | 84.35 | 62.85 | 34.10 | 13.73 | 4.78 | 1.44 | 0.52 | 0.14 |
| LEO600KM_Rural TN_non_AAS_DL | Average throughput loss | 81.39 | 59.52 | 33.93 | 15.03 | 5.71 | 1.83 | 0.50 | 0.17 |
|  | Cell edge through loss | 100.00 | 100.00 | 57.48 | 29.79 | 11.85 | 4.23 | 1.39 | 0.42 |
| LEO600KM Urban_macro TN_AAS_DL | Average throughput loss | 2.47 | 0.84 | 0.28 | 0.13 | 0.04 | 0.02 | 0.00 | 0.00 |
|  | Cell edge through loss | 1.32 | 0.40 | 0.17 | 0.08 | 0.05 | 0.00 | 0.00 | 0.00 |
| LEO600KM Urban_macro TN_non_AAS_DL | Average throughput loss | 2.58 | 0.86 | 0.33 | 0.14 | 0.05 | 0.02 | 0.00 | 0.00 |
|  | Cell edge through loss | 3.89 | 0.91 | 0.05 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
| LEO1200KM_Rural TN_AAS_DL | Average throughput loss | 65.02 | 43.60 | 24.52 | 11.09 | 4.36 | 1.53 | 0.43 | 0.17 |
|  | Cell edge through loss | 84.37 | 62.61 | 34.51 | 14.18 | 4.92 | 1.69 | 0.45 | 0.14 |
| LEO1200KM_Rural TN_non_AAS_DL | Average throughput loss | 81.37 | 59.34 | 33.96 | 15.14 | 5.52 | 1.95 | 0.64 | 0.18 |
|  | Cell edge through loss | 100.00 | 100.00 | 57.16 | 29.37 | 11.36 | 3.94 | 1.41 | 0.48 |
| LEO1200KM Urban_macro TN_AAS_DL | Average throughput loss | 2.34 | 0.82 | 0.29 | 0.08 | 0.02 | 0.01 | 0.00 | 0.00 |
|  | Cell edge through loss | 0.91 | 0.21 | 0.05 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
| LEO1200KM Urban_macro TN_non_AAS_DL | Average throughput loss | 2.87 | 0.96 | 0.27 | 0.11 | 0.03 | 0.01 | 0.00 | 0.00 |
|  | Cell edge through loss | 3.96 | 0.81 | 0.10 | 0.02 | 0.00 | 0.00 | 0.00 | 0.00 |

Proposal 1: to define ACLR as 24dB for LEO SAN class and ACLR as 14dB for GEO SAN class without 100kHz frequency gap next to 1st adjacent carriers.

For ACS requirement for SAN supporting IoT over NTN, based on the simulation results for Case 2 and Case 6 in the companion contribution [6], it seems okay to reuse the ACS requirement as 38dBc for IoT NTN,

Proposal 2: to reuse ACS requirement 38dBc for IoT NTN SAN without 100kHz frequency gap next to victim carrier.

## 2.2. IoT level for dynamic range requirement for NB-IoT over NTN

Similar as Rel-17 NTN SAN, legacy LTE BS dynamic range requirement and FR2 BS dynamic range requirement discussion, we need to check the IoT level in different NTN deployment scenarios and the initial simulation results are shown in the following Figure 1/2/3/4/5/6. Based on the simulation results obtained so far, it could be found that IoT level@95% CDF for different deployment scenarios are also varying. The details of IoT level in the different deployment scenarios are summarized in the following table.

Figure 1a. IoT leve1 in GEO Rural scenario

Figure 1b. IoT level in GEO Urban macro scenario

Figure 1c. IoT leve1 in GEO Rural scenario with single tone 15kHz

Figure 1d. IoT level in GEO Urban macro scenario with single tone 15kHz

Figure 1e. IoT leve1 in GEO Rural scenario with single tone 3.75kHz

Figure 1f. IoT level in GEO Urban macro scenario with single tone 3.75kHz

Figure 2a. IoT leve1 in LEO1200km Rural scenario

Figure 2b. IoT level in LEO1200km Urban macro scenario

Figure 2c. IoT leve1 in LEO1200km Rural scenario with single tone 15kHz

Figure 2d. IoT level in LEO1200km Urban macro scenario with single tone 15kHz

Figure 2e. IoT leve1 in LEO1200km Rural scenario with single tone 3.75kHz

Figure 2f. IoT level in LEO1200km Urban macro scenario with single tone 3.75kHz

Figure 3a. IoT leve1 in LEO600km Rural scenario

Figure 3b. IoT level in LEO600km Urban macro scenario

Figure 3c. IoT leve1 in LEO600km Rural scenario with single tone 15kHz

Figure 3d. IoT level in LEO600km Urban macro scenario with single tone 15kHz

Figure 3e. IoT leve1 in LEO600km Rural scenario with single tone 3.75kHz

Figure 3f. IoT level in LEO600km Urban macro scenario with single tone 3.75kHz

Table 1. summarized IoT level for IoT over NTN

|  | Multiple tone | Single tone 15kHz | Single tone 3.75kHz | Recommendation |
| --- | --- | --- | --- | --- |
| GEO Rural scenario | 4.7 | 11.2 | 18.2 | 22 |
| GEO Urban scenario | 7.0 | ~12 | 22 |  |
| LEO1200KM Rural scenario |  |  |  | 24 |
| LEO1200KM Urban scenario |  |  | 24 |  |
| LEO600KM Rural scenario |  |  |  | 24 |
| LEO600KM Urban scenario |  |  | 24 |  |

Based on the initial simulation results summarized in the above table 1, it could be found that IoT level as 24dB for both GEO and LEO should be sufficient to test its performance.

It should be noted that we still observe the high IoT level compared for GEO scenario especially for single tone 3.75kHz case which is different from the Rel-17 NR over NTN.

Proposal 3: to agree the IoT level as 24dBc for GEO and LEO dynamic rang requirements.

Based on the above IoT proposal for NTN and the following legacy approach to define dynamic range requirement for NB-IoT BS as following:

For standalone operation, the interfering signal level and wanted signal level are:

Different from terrestrial standalone NB-IoT BS, noise figure should be updated as 7.4dB for GEO and 4.3dB for LEO and IoT level should be updated as 24dB in the above formula. Based on the above analysis, we proposed to define the dynamic range requirement for IoT over NTN in GEO and LEO scenarios as following:

Proposal 4: to for GEO and LEO dynamic rang requirements.

Table 2: Dynamic range of SAN supporting standalone NB-IoT operation  (GEO and LEO class payload)

| SAN channel bandwidth [kHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | [-96.4] | [-92.7] | AWGN |
| 200 | FRC A15-2 in Annex A.15 | [-102.3] | [-92.7] | AWGN |

Conclusions

In this contribution, we want to share some further views on SAN RF requirement for IoT over NTN and proposals are made as following:

Proposal 1: to define ACLR as 24dB for LEO SAN class and ACLR as 14dB for GEO SAN class without 100kHz frequency gap next to 1st adjacent carriers.

Proposal 2: to reuse ACS requirement 38dBc for IoT NTN SAN; .

Proposal 3: to agree the IoT level as 24dBc for GEO and LEO dynamic rang requirements.

References

- RP-221556, WID on NB-IoT/eMTC core & performance requirements for NTN

- R1-2200042, LS on DL synchronization enhancements for IoT NTN, RAN1 (Jan-22)

- R4-2214382,WF on SAN RF requirement for IoT over NTN, ZTE, approved.

- R4-2217474 WF on SAN RF requirements for IoT over NTN, approved.

- R4-2217473 WF on co-existence assumption for IoT over NTN, MTK, approved.

- R4-2219364,Further discussion on simulation assumptions and evaluation results for IoT over NTN, ZTE