3GPP TSG-RAN WG4 Meeting#105                                R4-2219381

Toulouse, France,14th Nov – 18th Nov,2022

Agenda item:	8.23.4

Source: 	ZTE Corporation

Title: 	Further discussion on UE RF requirements for NTN in Ka-band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. During the last RAN4 meeting, there were some initial discussions for NTN UE in Ka-band with some high level agreement reached in [9]. In this contribution, we want to share some further understandings for NTN UE in Ka-band.

The following covers the objectives for NR-NTN deployment in above 10 GHz bands. This work is expected to start after June 2022.

- Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4]

  - Consider the satellite harmonized Ka band as a reference, according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations, define an example band suitable for development of generic 3GPP minimum performance requirements (the example RAN4 band may be a portion of or the entire harmonized Ka band). [RAN4]

  - Study implications of FDD operation in FR2 and derive requirements for the identified example band appropriately. Satellite bands introduced in 3GPP for NTN for FDD shall not impact the existing 3GPP TDD specifications for terrestrial bands adjacent to the NTN band (see note 3 of the approved way forward RP-211596 in RAN#92-e). [RAN4]

  - Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow. [RAN4]

  - For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands [RAN4].

  - Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region; (see proposal 2 of the approved way forward RP-211596 in RAN#92-e) [RAN4]

Discussion

## 2.1. The use case for VSAT and ESIM

First of all, before going into the detailed discussion for NTN UE RF requirement, it’s better to have some high level overview of use case for VSAT and ESIM in Ka-band. As shown in the following Figure 1, the NTN UE in Ka-band could be categorized into the following three different types:

Table 1. The introduction of fixed VSAT and different ESIM types

| Use case | Applicable note |
| --- | --- |
| Aeronautical ESIM or A-ESIM | It’s expected to have phase antenna array to be equipped on top of aircraft to enable the on-board communication service with satellite. |
| Maritime ESIM or M-ESIM | It’s expected to have parabolic antenna with cable connected with transceiver to enable the WiFi service. FFS for phase antenna array of maritime ESIM. |
| Land ESIM or L-ESIM | It’s expected to have parabolic antenna with cable connected with transceiver or phase antenna array. The potential use case could be as following - emergency communication to provide the service in case of any disconnection due to any unexpected reasons, e.g. earthquake or other disasters; - Vehicular service for cars travelling in the areas without IMT coverage; - Portable ESIM to act as mobile small BS to provide the service for other reasons. |
| - Fixed VSAT | It’s expected to have parabolic antenna with cable connected with transceiver. The potential use case could be the backhual link for some important business service (e.g. banking service or other public service) to prevent the disconnection in case of any unexpected reasons, e.g. earthquake or other disasters. |

Figure 1. The commercial deployment for Fixed VSAT and mobile ESIM

Based on the above analysis, RAN4 need to further discuss whether all use case should be included in Rel-18 and there are any commonalities between different use cases.

Proposal 1: RAN4 should discuss whether all following use case should be included in Rel-18 or any deprioritization considering the workload and there are any commonalities between different use cases.

- Aeronautical ESIM or A-ESIM

- Maritime ESIM or M-ESIM

- Land ESIM or L-ESIM

- Fixed VSAT

In addition, based on the above observations for different use case, at least parabolic antenna for fixed VSAT and ESIM should be considered in Rel-18 which could be applicable for Maritime ESIM, Land ESIM and Fixed VSAT. Regarding the phase antenna array, this is more relevant for Aeronautical ESIM or Land ESIM (.e.g. portable mobile small BS). From our understanding, this could be discussed in the 2nd phase if there are no much market demands on it.

Proposal 2：At least parabolic/Dish antenna for fixed VSAT and ESIM should be considered in Rel-18 as first priority. Regarding the phase antenna array for ESIM, this could be discussed in the 2nd phase if there are no much market demands on it.

From our understanding, there are lots of merits and demerits between parabolic antenna and phase antenna array. The details could be found as following table.

Table 2. The summarized merits and demerits for parabolic antenna and phase antenna array

| Antenna implementations | Description | Merits | Demerits |
| --- | --- | --- | --- |
| Parabolic antenna/Dish antenna | The beam tracking for parabolic antenna could be maintained by 2/3-Axis Stabilized + Auto Skew operation platform with assistance information Built in Gyro or GPS information. Basically, the beam steering capability is mechanical steering which is expected to be slower than digital beam steering. Antenna aperture and output power of transceiver will decide the final EIRP values. | Mature and cheap and easy to be installed in many use cases. | A bit too bulky for some use case, for instance, aircraft or other portable mobile small BS. |
| Phase antenna array | The beam tracking could be maintained by phase antenna array with digital beamforming, the beam steering capability is digital steering instead of mechanical steering Antenna array size and output power of transceiver will decide the final EIRP values. | The beam steering capability is digital level which should be much faster than parabolic antenna by 2/3-Axis Stabilized + Auto Skew operation platform. | A bit expensive and might be not so typical for lots of use case. |

Observation 1: merits and demerits between parabolic antenna and phase antenna array are summarized in table 2.

In addition, in the last RAN4 meeting, there were some discussions for difference between fixed VSAT and ESIM and whether we need to specify separate power class for fixed VSAT and ESIM or not. From our understanding, the RF part for fixed VSAT and ESIM should be similar without expecting much difference between them. The potential difference expected is that ESIM should be equipped with operation platform with 2/3-Axis Stabilized + Auto Skew to track the GSO satellite for initial satellite tracking and further refinement due to the movement. However for fixed VSAT connecting with GSO, then such kind of operation platform with 2/3-Axis Stabilized + Auto Skew to track the GSO satellite might be not needed. If beam tracking capability is also part of RF requirement for NTN UE in Ka-band, then the separated power class for ESIM and fixed VSAT are still needed.

If the Fixed VSAT are also expected to connect with NGSO, then beam tracking capability would be also needed however it might be different from ESIM connecting with GSO.

Table 3. The application scenario for ESIM and fixed VSAT allowed by WRC.

| Terminal types | Applicable scenario |  |
| --- | --- | --- |
|  | GSO | NGSO |
| ESIM | Yes | No |
| Fixed VSAT | Yes | ? |
| NOTE: ESIM operations in Ka-band are allowed by ITU-R with GSO satellite (WRC-19), while ESIM with NGSO satellite is for consideration under Agenda Item 1.16 at WRC-23. |  |  |

Observation 2: RF part for fixed VSAT and ESIM should be similar, however the satellite tracking capability might be different between ESIM connecting with GSO and fixed VSAT connecting with GSO or NGSO.

Proposal 3: if satellite tracking capability is part of RF requirement and the required satellite tracking capability are also different between ESIM and fixed VSAT, then we need to specify the different power class for ESIM and fixed VSAT.

## 2.2. The diagram for dish antenna and phase antenna array for VSAT or ESIM

As mentioned in the previous section, both parabolic antenna and phase antenna array could be considered for NTN UE in Ka-band as shown in the following Figure 2 and Figure 3. During the last RAN4 meeting, there were some discussions whether we need to define the OTA requirement for NTN UE only or conducted requirement could be specified for NTN UE as well.

Based on the initial online feedbacks from satellite vendors during the GTW discussions, ESIM and fixed VSAT are different from the terrestrial TN UEs, the cable connector could be still allowed. Based on the further investigation on the commercially available product in Ku-band for Maritime ESIM with parabolic antenna at least, cable connector under the operation platform as shown in Figure 2 could been seen which means the conducted requirement is testable and could be allowed and specified.

Figure 2. The illustration of dish antenna for ESIM [6]

Proposal 4: for ESIM or fixed VSAT with parabolic antenna implementation, then at least conducted requirement should be defined/specified and further discuss whether we need to specify the radiated requirements for it.

For phase antenna array for Aeronautical ESIM as shown in Figure 3 or portable ESIM to act as mobile small BS, from our understanding, this should be similar as terrestrial FR2 NR BS or FR2 NR UE in which there are no accessible connectors for conformance testing, otherwise there will be lots of unexpected insertion loss especially for mmWave frequency.

Figure 3. The illustration of phase antenna array for ESIM [7]

Proposal 5: for ESIM or [fixed VSAT] with phase antenna array, only radiated requirement should be defined/specified.

## 2.3. RF requirement for VSAT and ESIM

During the last RAN4 meeting, we have some initial discussions on the implementation details for VSAT and ESIM which might have the impacts the RF requirement. In addition, companies think that it might be premature to discuss the detailed RF requirements for NTN UE before knows its details especially for its power class assumption. In the following section, we want to share some initial views on its power class and beam correspondence for NTN VSAT and ESIM UE firstly.

As mentioned in the section 2.2, conducted requirement could be defined for VSAT or ESIM with parabolic antenna. The following transmit power 33dBm used for NTN coexistence study in table 2.3-1 could be used as baseline.

Table 2.3-1: NTN UE characteristics for system level simulation

| Characteristics | VSAT (Note 2) |  |  |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) |  |  |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter of TR 38.821 |  |  |
| Polarisation | circular |  |  |
| Rx Antenna gain | 39.7 dBi |  |  |
| Antenna temperature | 150 K |  |  |
| Noise figure | 1.2 dB |  |  |
| Tx transmit power | 2 W (33 dBm) |  |  |
| Tx antenna gain | 43.2 dBi |  |  |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |

Proposal 6: the transmit power 33dBm could be used as baseline for power class definition of VSAT or ESIM with parabolic antenna.

In addition, we also noticed that there are the different antenna aperture from the same frequency range which are used to meet different coverage requirement. For instance, for 60cm parabolic antenna aperture which could be used for M-ESIM close to sea coast, however for 80cm parabolic antenna aperture which could be used for E-SIM far away from sea coast for international shipping.

In addition, based on the following required EIRP for User spot 11 L beam on Inmarsat 5-F3 at 179.6°E in Figure 4, to meet the uplink coverage requirement in different areas, then EIRP are required differently by satellite vendors. The required EIRP might be also depending on specific satellite and its specific beams and its geographical locations of VSAT and ESIM. From our understanding, it’s not necessary to define any EIRP limits and leave this to the market demand.

Figure 4. The required EIRP for the User Spot 11 L beam is available on Inmarsat 5-F3 at 179.6°E [12]

Proposal 7: for VSAT and ESIM with parabolic antenna, propose not to define EIRP limit for it and leave this to the implementation based on the legacy approach.

For VSAT or ESIM with phase antenna array, since only radiated requirement might be defined considering its high insertion loss if RF connectors are available and accessible for conformance testing, then minimum peak EIRP, UE maximum output power limits and UE spherical coverage requirement are still needed. However for these EIRP requirements as mentioned above, this highly depends on specific satellite and its specific beams and its geographical locations of VSAT and ESIM. More inputs from satellite vendors are needed especially for the range of the required EIRP covering different kind of service.

Proposal 8: for VSAT and ESIM with phase antenna array, minimum peak EIRP, UE maximum output power limits and UE spherical coverage requirements should be defined. More inputs from satellite vendors are needed especially for the range of the required EIRP covering different kind of service.

In addition, in last RAN4 meeting, there were some initial discussion whether Beam correspondence requirement for NTN UE in Ka-band is valid or not. At least from our understanding, beam correspondence requirement for VSAT and ESIM with parabolic antenna is not needed. The beam steering capability is adjusted by the operation platform with 2/3-Axis Stabilized + Auto Skew to track the GSO satellite with the assistant local GPS information or Gyro information and ephemeris information from satellite regardless sent from SIB19 or embedded information. In short, for uplink beam tracking for VSAT and ESIM, this could depend on the operation platform with 2/3-Axis Stabilized + Auto Skew to track the satellite with the assistant local GPS information or Gyro information and ephemeris information, therefore beam correspondence capability based on downlink SSB+CSI-RS or SSB only or CSI-RS only based beam correspondence capability is not needed.

Proposal 9:  for VSAT and ESIM with parabolic antenna, beam correspondence is not needed since its uplink satellite tracking capability is not replying on downlink SSB or CSI-RS signals.

For beam correspondence requirement for VSAT and ESIM with phase antenna array, similar logic could be applied here, its uplink satellite tracking capability should depend on digital beamforming or analog beam switching with the assistant local GPS information or Gyro information and ephemeris information. In addition, beam correspondence capability in FDD band in Ka-band is also questionable since its downlink and uplink channel is separated by around 9.8GHz for MSS band, so it is quite difficult to predict its uplink beam with downlink reference signal. In addition, the RF match network between downlink and uplink within VSAT or ESIM is also questionable due to its large frequency separation.

Proposal 10:  for VSAT and ESIM with phase antenna array, beam correspondence is not needed since its uplink satellite tracking capability might be not replying on downlink SSB or CSI-RS signals anymore.

Table 1. Summary of related UE RF requirement

|  | Band-specific or not | Applicability |
| --- | --- | --- |
| Transmitter Characteristics |  |  |
| General | No | To follow the existing text from TN UE in TS 38.101-2 |
| Tx power | Yes | To define new power for NTN VSAT with parabolic antenna should be defined and the following requirement should be considered: - Power class : 33dBm If necessary, new power for NTN VSAT with phase antenna array should be defined and the following requirement should be considered - UE minimum peak EIRP - UE maximum output power limits - UE spherical coverage (TBD %-tile CDF should be revisited for NTN VSAT UE) |
| MPR | No | This depends on the ACLR, SEM and EVM requirement and discussion could be postponed until other requirement is more clear. |
| A-MPR | Yes | This depends on other coexistence requirement or regulatory requirement. Operators ‘s input are encouraged. |
| Configured Tx power | No | To follow the existing text from TN UE in TS 38.101-1 for NTN VSAT with parabolic antenna . To follow the existing text from TN UE in TS 38.101-2 for NTN VSAT with phase antenna array |
| Output Power Dynamics | No | The minimum output power for NTN VSAT UE, this could be further discussed. Transmitter OFF power and ON-OFF time mask and power control related parameter in TS 38.101-1/2 could be good starting point. |
| Transmit signal quality |  |  |
| - Frequency error | No | to follow the requirement defined in TS38.101-5 where UE UL pre-compensation is still needed. |
| - Transmit modulation quality | No | To follow the existing requirement defined for TS 38.101-1/2, however the maximum modulation order could be further discussed similar as Rel-17 NR over NTN Carrier leakage and in-band emission are also power class specific requirement and this could be further discussed. |
| Output RF spectrum emissions |  |  |
| - Occupied bandwidth | No | To follow the existing requirement defined for TS 38.101-1/2. |
| - Out of band emission |  |  |
| - SEM | No | This depends on the outcome of coexistence study. |
| - Additional SEM | Yes | additional requirement are expected for ITU resolution 169 in WRC-19 and [156] for WRC-15 |
| - ACLR | No | This depends on the outcome of coexistence study. |
| - Spurious emission |  |  |
| - General | No | To follow the existing requirement defined for TS 38.101-2. |
| - For UE coexistence | Yes | Coexistence requirement for the surrounding TN bands should be considered. |
| Transmit intermodulation | No | Not applicable similar as FR2 UE RF |
| Beam correspondence | No | Please see the above analysis |
| Receiver characteristics |  |  |
| General | No |  |
| Diversity characteristics | No |  |
| Reference sensitivity | Yes | For NTN VSAT with parabolic antenna, the following requirements should be defined for NTN VSAT UE. - conducted sensitivity power level For NTN VSAT with phase antenna array, the following requirements should be defined for NTN VSAT UE. - Reference sensitivity power level - EIS spherical coverage requirement |
| Maximum input level | No | Further system level evaluation is needed and this requirement might be relaxed similar as Rel-17 NR NTN. |
| ACS | No | This depends on the outcome of coexistence study. |
| Blocking characteristics |  |  |
| - In-band | No | This depends on the outcome of coexistence study. |
| - Out-of-band | NA | NA |
| - Narrow band | NA | NA |
| Spurious response | NA | NA. |
| Intermodulation | NA | NA |
| Spurious emissions | No | To follow the existing requirement defined for TS 38.101-1/2. |

Conclusions

In this contribution, we want to share some initial views on UE RF requirements for NTN in Ka-band.and proposals are made as following:

Proposal 1: RAN4 should discuss whether all following use case should be included in Rel-18 or any deprioritization considering the workload and there are any commonalities between different use cases.

- Aeronautical ESIM or A-ESIM

- Maritime ESIM or M-ESIM

- Land ESIM or L-ESIM

- Fixed VSAT

Proposal 2：At least parabolic/Dish antenna for fixed VSAT and ESIM should be considered in Rel-18 as first priority. Regarding the phase antenna array for ESIM, this could be discussed in the 2nd phase if there are no much market demand on it.

Observation 1: merits and demerits between parabolic antenna and phase antenna array are summarized in table 2.

| Parabolic antenna/Dish antenna | The beam tracking for parabolic antenna could be maintained by 3-Axis Stabilized + Auto Skew operation platform with assistance information Built in Gyro or GPS information. Basically, the beam steering capability is mechanical steering which is expected to be slower than digital beam steering. Antenna aperture and output power of transceiver will decide the final EIRP values. | Mature and cheap and easy to be installed in many use cases. | A bit too bulky for some use case, for instance, aircraft or other portable mobile small BS. |
| --- | --- | --- | --- |
| Phase antenna array | The beam tracking could be maintained by phase antenna array with digital beamforming, the beam steering capability is digital steering instead of mechanical steering Antenna array size and output power of transceiver will decide the final EIRP values. | The beam steering capability is digital level which should be much faster than parabolic antenna by 3-Axis Stabilized + Auto Skew operation platform. | A bit expensive and might be not so typical for lots of use case. |

Observation 2: RF part for fixed VSAT and ESIM should be similar, however the satellite tracking capability might be different between ESIM connecting with GSO and fixed VSAT connecting with GSO or NGSO.

Proposal 3: if satellite tracking capability is part of RF requirement and the required satellite tracking capability are also different between ESIM and fixed VSAT, then we need to specify the different power class for ESIM and fixed VSAT.

Proposal 4: for ESIM or fixed VSAT with parabolic antenna implementation, then at least conducted requirement should be defined/specified and further discuss whether we need to specify the radiated requirements for it.

Proposal 5: for ESIM or [fixed VSAT] with phase antenna array, only radiated requirement should be defined/specified.

Proposal 6: the transmit power 33dBm could be used as baseline for power class definition of VSAT or ESIM with parabolic antenna.

Proposal 7: for VSAT or ESIM with parabolic antenna, propose not to define EIRP limit for it and leave this to the implementation based on the legacy approach.

Proposal 8: for VSAT and ESIM with phase antenna array, minimum peak EIRP, UE maximum output power limits and UE spherical coverage requirements should be defined. More inputs from satellite vendors are needed especially for the range of the required EIRP covering different kind of service.

Proposal 9: for VSAT and ESIM with parabolic antenna, beam correspondence is not needed since its uplink satellite tracking capability is not replying on downlink SSB or CSI-RS signals.

Proposal 10: for VSAT and ESIM with phase antenna array, beam correspondence is not needed since its uplink satellite tracking capability might be not replying on downlink SSB or CSI-RS signals anymore.

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- https://www.kingsat-tech.com/product/enhanced-vsat-ku-band-3-axis-stabilized-and-4-axis-tracking114.html

- https://www.mitsubishielectric.com/news/2020/0206.html

- R4-2217792, Email discussion summary for [104-bis-e][140] NR_NTN_enh_UERF, ZTE, Noted

- R4-2217743, WF on NTN UE in Ka-band, ZTE, Approved

- http://satstar.net/satellites/space2.html

- https://www.satbeams.com/satellites

- http://satstar.net/beams/inmar5f3_sb11.html

- R4-2216515,NTN enhancements – Regulatory aspects, Ericsson, Noted

- Solution 169 pdf limit ESIM to France  E-PFD mask

- ITU-R recommendation S.524