3GPP TSG-RAN WG4 Meeting # 105	R4-2219392

Toulouse, France, November 14 – November 18, 2022

Title: 	Discussion on	remaining open issues for RedCap

Source: 	Huawei, HiSilicon

Agenda item:	6.6.2

Document for:	Discussion

Introduction

This contribution is to further discuss the remaining open issues on core requirements for RedCap UE.

Discussion

- Leftovers in handover

One left open issue on handover is whether additional delay is needed when the measured SSB is different with the handover-to SSB [1].

Requirements for HO to a BWP which has different SSB with the one used for measurement - Option 1 (Ericsson, HW, Nokia, Apple, Intel, CMCC, OPPO, Xiaomi, HW): No additional delay is needed when UE handover to a BWP which has different SSB type with the one used for measurement. - Option 2 (QC, MTK): The scenario when handover is performed to a BWP which has different SSB than the one used during measurement should be considered as handover to an unknown cell. Capture the above condition as a note in the Handover related section in TS38.133 - Option 2a (QC): When handover is performed to a known cell where the SSB associated with the first active BWP is different from the SSB used during intra-frequency or inter-frequency measurements of the target cell, then Tsearch = Trs ms if the target cell is intra-frequency cell and Tsearch = 2* Trs ms if the target cell in inter-frequency cell.” - Compromise (new) Option 3 (Ericsson, vivo, Apple): An additional delay is allowed if some conditions are met, such as the frequency difference between NCD-SSB and CD-SSB is larger than a threshold. Option 4 (new) (Ericsson): Additional delay is allowed if the frequency difference between NCD-SSB and CD-SSB is larger than a threshold.

The maximum cell bandwidth is 100MHz for FR1 and 300MHz for FR2. The frequency difference between CD-SSB and NCD-SSB doesn’t exceed cell bandwidth. According to RAN2 NCD-SSB signalling, only NCD-SSB periodicity, offset can be specific configured for NCD-SSB. For other properties, e.g., NCD-SSB power, NCD-SSB has the same value as the corresponding CD-SSB. Then the measured signal quality on CD-SSB and NCD/SSB would be similar. As a result, for intra-frequency measurement, the AGC is not required to adjust.

NonCellDefiningSSB-r17 ::= SEQUENCE { absoluteFrequencySSB-r17 ARFCN-ValueNR, ssb-Periodicity-r17 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL, -- Need S ssb-TimeOffset-r17 ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 } OPTIONAL, -- Need S ... }

nonCellDefiningSSB If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the cell-defining SSB of the serving cell (e.g. obtaining sync, measurements, RLM). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

Moreover CD-SSB and NCD-SSB shall have the same PCI. In last meeting RAN1 agree to adopt the following agreement for TS 38.213. It means the QCL relationship is the same for NCD-SSB and CD-SSB. Then the best beam measured on CD-SSB is also the best beam of NCD-SSB if UE would measure on NCD-SSB.

Proposal 1: No strong justification is observed to introduce additional delay when UE handover to a BWP which has different SSB type with the one used for measurement.

Conclusions

This contribution provides analysis on the remaining issues for RedCap UE. The following proposal is provided:

Proposal 1: No strong justification is observed to introduce additional delay when UE handover to a BWP which has different SSB type with the one used for measurement.

Reference

- R4-2210592, WF on RedCap RRM requirements, Ericsson