3GPP TSG-RAN WG4 Meeting #105	R4-2219451

# Toulouse, France,  14th Nov – 18th Nov, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | <CR#> | rev | <Rev#> | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to 37.941: 71 GHz Extension TE MU update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With FR2-2 added, TE MU values are also updated and needs to be captured in TR37.941 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Tables for TE MU values are updated in Annex C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No information provided for MU update details |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.141-2 ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<unchanged text skipped>>

Table C.2-2a: Test equipment uncertainty values for FR2-1

| UID | Instrument | Standard uncertainty σ (dB) |  | Probability distribution |
| --- | --- | --- | --- | --- |
|  |  | 24.25 < f ≤ 29.5 GHz | 37 < f ≤ 43.5 GHz |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - high power | 0.50 | 0.70 | Gaussian |
| C1-2 | RF signal generator | 0.90 | 0.90 | Gaussian |
| C1-3 | Network Analyzer | 0.30 | 0.30 | Gaussian |
| C1-7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - low power (UEM) | 0.90 | 0.90 | Gaussian |
| C1-8 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - relative (ACLR) | 0.75 | 0.90 | Gaussian |
| C1-9 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - relative (ACLR) | 1.25 | 1.45 | Gaussian |

Table C.2-2b: Test equipment uncertainty values for FR2-2 inband measurement

| UID | Uncertainty source |  |
| --- | --- | --- |
|  | Test equipment | 57 < f ≤ 71 GHz | Note |
| C1-3 | Uncertainty of the Network Analyzer | 0.85 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, power sensor) | 2.36 |  |
| C1-7 | Unceratinty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) for IF used with mixer | 0.6 | IF frequency range up to 20 GHz for inband |
| A2-20 | Uncertainty of the mixer (FR2 only) | 2.25 |  |
|  | Combined MU of C-7 and A2-20 by RSS for C1-1 | 2.36 | RSS of above two and IF mismatch = 0.41 |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) – low power (UEM absolute ACLR) | 2.36 | Use the same value as C1-1 |
| C1-8 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) – relative (ACLR) | 2.36 | Use the same value as C1-1 |
| C1-9 | Uncertainty of the RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2 | 2.61 | Use FR2-1 Tx off power value for frequency in 24~29G 1.25 as IF MU number With IF mismatch 0.41 With RSS of 1.25, 2.25, 0.41 |
| C1-2 | Uncertainty of the RF Signal Generator | 2.37 | For Modulated signal with mixer 2.25 and IF mismatch 0.29 RSS of 0.7, 0.29, 2.25 |
| MUOOBint | Additional uncertainty for the CW interferer signal | 2.32 | For CW signal, RSS with mixer 2.25, IF mismatch = 0.29 and SG IF (CW) = 0.5 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 0.97 |  |
| A2-5b | Mismatch of receiver chain for low power receiver | 0.97 |  |
| B2-5 | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) | 1.09 |  |
| A2-19 | Uncertainty of the LNA (FR2 only) | 1.50 |  |
| MUPA | MUPA Uncertainty due to use of PA | 1.50 | Same as LNA A2-19 |
|  | OTA modulation quality | 1.1% |  |

Table C.2-2c: Test equipment uncertainty values for FR2-2 out of band measurement

| UID | Uncertainty source |  |  |
| --- | --- | --- | --- |
|  | Test equipment | 60 < f ≤ 110 GHz for Unwanted Emission | 110 < f ≤ 142 GHz for Unwanted Emission | Note |
| C1-3 | Uncertainty of the Network Analyzer | 0.85 | 0.85 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, power sensor) | 2.39 | 2.49 |  |
| C1-7 | Unceratinty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) for IF used with mixer | 0.7 | 0.7 | Value 0.6 is for IF frequency range up to 20 GHz, 0.7 up to 29G |
| A2-20 | Uncertainty of the mixer (FR2 only) | 2.25 | 2.35 |  |
|  | Combined MU of C-7 and A2-20 by RSS for C1-1 | 2.39 | 2.49 | RSS of above two with IF mismatch = 0.41 |
| MUOOBint | Additional uncertainty for the OOB interferer signal | 2.32 | 2.42 | CW signal, RSS with mixer 2.25 or 2.35 and IF mismatch = 0.29, SG IF CW 0.5 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 1.19 | 1.14 |  |
| A2-5b | Mismatch of receiver chain for low power receiver | 1.19 | 1.14 |  |
| B2-5 | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) | Not used | Not used |  |
| A2-19 | Uncertainty of the LNA (FR2 only) | 1.50 | 1.50 |  |
| MUPA | MUPA Uncertainty due to use of PA | 1.50 | 1.50 |  |