3GPP TSG-RAN WG4 Meeting # 105	 R4-2219466

Toulouse, France, November 14 ‒ 18, 2022

Title: 	RF issues related to carrier phase measurement for positioning

Agenda Item:	8.19.3

Source: 	Ericsson

Document for:	Discussion

# Introduction

RAN4#104bis-e discussed RF issues related to carrier phase measurement for positioning. There were no open issues identified based on the output of the discussions. Therefore, in this paper we briefly highlight RAN1 agreements from their last meeting and implication of those agreements to the scoping of RAN4 work.

# Discussion

RAN4#104bis-e agreed on the following [1].

Agreement:

- RAN4 shall wait for conclusion from RAN1 evaluation of impact of different error sources on carrier phase measurement before starting study on RF requirement for NR carrier phase measurement.

Agreement:

- No further discussion is needed in RAN4 at this time

RAN1#110bis-e made the following agreements:

Agreement

The existing DL PRS and UL SRS for positioning can be re-used as the reference signals to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning.

- FFS: Whether to consider enhancements of the existing DL PRS and UL SRS for better positioning performance.

Agreement

For UE-assisted NR carrier phase positioning, at least consider the following options

- the difference between the carrier phase measured from the DL PRS signal(s) of the target TRP and the carrier phase measured from the DL PRS signal(s) of the reference TRP.

- the carrier phase measured from the DL PRS signal(s) of a TRP.

Agreement

Make the following modification to the previous agreement on the initial phase model with an additional note:

- In the evaluation of NR carrier phase positioning, both the initial phase of the transmitter and the initial phase of the receiver can be modelled as independent random variables uniformly distributed within [0, 2pi].

- Note: The initial phase of a transmitter applies to all subcarriers of the same carrier frequency associated with the transmitter, and the initial phase of a receiver applies to all subcarriers of the same carrier frequency associated with the receiver.

Agreement

Further study the benefits of using the carrier phase measurements of multiple DL positioning frequency layers for NR carrier phase positioning, which may include the impact of the time gap between the carrier phase measurements of multiple DL PFLs.

- Note 1: The initial phase error and the frequency error for each PFLs can be modelled independently

- Note 2: For the evaluation, the PRS signals of all PFLs of a TRP can be assumed to be transmitted from the same ARP or from different ARPs of the TRP.

- Note 3: The location error for ARPs can be modelled independently.

- Note 4: The timing errors of the PFLs may not be the same for PFLs in different bands or frequency ranges.

- Note 5: In Rel-17, simultaneous reception of DL PRS from multiple frequency layers is not supported.

Agreement

For UL UE-assisted NR carrier phase positioning, at least consider the carrier phase measured from the UL SRS for positioning purpose.

- Note: The use of MIMO SRS for positioning purpose is transparent to UE.

Agreement

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

- The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item. Based on the study, it is concluded that multipath/NLOS deteriorates the performance of carrier phase positioning and it is necessary to consider multipath mitigation for NR carrier phase positioning.

- The evaluation results for the impact of the multipath/NLOS on NR carrier phase positioning will be presented in Section 6.3.2.

Agreement

Add the following note to the previous agreement on error modelling of the initial phase:

- Note: The initial phases of a transmitter for different carriers can be assumed to be independent of each other. Similarly, the initial phases of a receiver for different carriers can be assumed to be independent of each other.

Agreement

Add a row of "PRU assumptions" in Table B.4.X.1-1: NR carrier phase positioning enhancements – evaluation scenarios and parameters” in Draft TR 38.859.

- Note: PRU deployment assumptions may include the assumptions of the number of PRUs, the PRU locations and location errors etc.

Agreement

Capture the following TP into TR 38.859 as an evaluation observation:

The impact of the initial phases of the transmitter and the receiver on NR carrier phase positioning is evaluated in the study item. The evaluation results from the sources (e.g., Huawei[1], vivo[2], CATT[6], ZTE[9]) show that if the initial phases of the transmitter and the receiver are not eliminated, it is impossible to support centimetre-accuracy positioning.

The effectiveness of using double differential technique with PRU to eliminate the impact of the initial phases of the transmitter and the receiver on NR carrier phase positioning are evaluated in the study item. The evaluation results from the sources (Huawei[1], CATT[6], ZTE[9], Ericsson [23]) show that the initial phases of the transmitter and the receiver can be removed effectively by the double differential technique with the use of the PRU:

- Source [Huawei, 1] shows the positioning accuracy of <1cm (80%) for Inf-SH and < 1cm (50%) for Inf-DH can be reached when the PRU is located within a distance of 5m from the target UE.

- Source [CATT, 6] shows the positioning accuracy of <1cm (80%) for Inf-SH and <1cm (50%) for Inf-DH can be reached under the under condition that the PRU is located a fixed location in LOS of the TRP.

- Source [Ericsson 23] shows that the accuracy of <1cm (50%) when the PRU is located within 1m of the target UE. However, the effectiveness reduces when the PRU is located away from the target UE because the channel conditions of the PRU is different from the target UE.

- Note: in the above results, all other error sources (except initial phase error) were not modelled.

(Not captured in TR) Note: The number of sources and the references, and the observations, can be further updated in next meeting depending on companies’ updates of simulation results.

Agreement

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

The impact of the residual CFOs of the transmitter and the receiver on NR carrier phase positioning are evaluated during the study item.

- The evaluation results from the sources (Huawei[1], ZTE[9]) shows the impact of residual CFOs on carrier phase positioning is negligible.

- The evaluation results from the source (CATT[4]) shows the impact of the residual CFOs on the positioning performance of carrier phase positioning is removed with the use of the double differential technique with the PRU that is located a fixed location in LOS of the TRP.

(Not captured in TR) Note: The number of sources and the references, and the observations, can be further updated in next meeting depending on companies’ updates of simulation results.

Agreement

Capture the following TP into TR 38.859 (Section 6.3.1):

- The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been studied during the study item.

  - For DL NR carrier phase positioning, the PRU works as a UE to receive the DL PRS reference signals and provide the DL carrier phase measurements to the LMF, where the double differential measurements can be obtained by the difference of the DL carrier phase measurements from the target UE and those from the PRU for eliminating the measurement errors.

  - For UL NR carrier phase positioning, the PRU works as a UE to transmit the UL SRS signals for positioning purpose. The TRPs provides the UL carrier phase measurements obtained from the UL SRS signals of the target UE and of the PRU to the LMF, where the double differential measurements can be obtained by the difference of these UL carrier phase measurements for eliminating the measurement errors.

Agreement

Further study the effectiveness of the following multipath mitigation methods for the carrier phase positioning and the potential on the standard work:

- Identify and separate the first path and other paths.

- Reporting of the carrier phase of the first path, and optionally, the additional paths.

- The use of LOS/NLOS indication for the carrier phase measurements.

  - Note: Rel-17 LOS/NLOS indicator can be considered as a starting point.

- The report of other channel information, such as RSRP/RSRPP.

Agreement

Further study the following approaches for NR carrier phase positioning, and identify the potential impact on the standard.

- the reporting of the carrier phase measurements together with the existing positioning measurements.

- the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements.

From RAN1 list of agreements from the latest meeting it can be observed that RAN1 is yet to conclude its evaluation of impact of different error sources on carrier phase measurement for positioning and therefore the scope of work is still not clear to RAN4. Since this is the last meeting of the SI phase, we propose RAN4 to discuss, if needed, issues related to carrier phase measurement during the WI phase.

Observation 1: Conclusion from the evaluation of impact of different error sources on carrier phase measurement for positioning is yet to be finalized in RAN1.

Proposal 1: Based on observation 1, RAN4 should uphold its previous agreement on RF issues related to carrier phase measurement for positioning.

# Summary

In the last meeting RAN4 agreed to wait for conclusion from RAN1 evaluation and not to have further discussion in RAN4. In our view the situation has not much changed since the last meeting and observe the following:

Observation 1: Conclusion from the evaluation of impact of different error sources on carrier phase measurement for positioning is yet to be finalized in RAN1.

Based on observation 1, we propose the following:

Proposal 1: Based on observation 1, RAN4 should uphold its previous agreement on RF issues related to carrier phase measurement for positioning.

# References

- R4-2217739, “WF on expanded and improved NR positioning – UE RF aspects”, Intel.

- RAN1#110bis-e, Chair’s notes, v17.