3GPP TSG-RAN WG4 Meeting # 105	R4-2219503

# Toulouse, France, November 14 – November 18 ,2022

Title: 	Simulation results for FR2-2 PDSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       6.3.6.2.1

Document for:	Discussion

Background

As per [1], RAN 4 defined many cases for FR2-2 PDSCH simulation work. In this paper, we provide our simulation results.

Simulation results

The common simulation assumptions are captured in Table 2-1 and simulation results are captured in Table 2-2 and Table 2-3

Table 2-1: Common simulation assumptions for FR2-2 PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | DDDSU for 120kHz SCS S=10D:2G:2U 14D2S4U for 480kHz SCS S1=12D:2G:0U, S2=0D:6G:8U |
| Bandwidth |  | MHz | For single CC: 100,400 For CA: 400 |
| SCS |  | kHz | 120,480 |
| PDSCH configuration | PDSCH mapping type |  | Type A |
|  | Start symbol |  | 2 |
|  | Symbol length |  | 12 |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | For case with 70% requirement: 8 for 120kHz SCS and 16 for 480kHz SCS. For case with 30% requirements:16 |
| HARQ transmissions |  |  | 4 |
| HARQ transmission sequence |  |  | {0,2,3,1} |
| Phase noise model |  |  | Set 2 specified in TR 38.808 |
| Phase noise compensation |  |  | CPE only |

- Schedule pattern for single CC:

- 120kHz

  - PDSCH is scheduled with one TDD pattern periodicity (DDDSU).

- 480kHz

  - One scheduling pattern includes two TDD pattern period which is 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U.

  - There are 7 DCIs in every scheduling pattern and each DCI schedules 4 PDSCHs if not overlapped with SSB, otherwise, each DCI schedules 3 PDSCHs. PDSCH is scheduled in every DL slots except slot #33 and #34 and slot contain SSB

- Schedule pattern for CA:

- 120kHz

  - For FR1+FR2-2 with 120kHz SCS, PDSCH is scheduled in slot 0,1,2,3,5,6,7,8 in every 40 slots of FR2-2 considering 8 HARQ processes is assumed.

- 480kHz

  - For FR1+FR2-2 with 480kHz SCS, PDSCH is scheduled in slot 0~14 and slot 20 in every 160 slots of FR2-2 considering 16 HARQ processes is assumed. Each DCI schedule 4 TBs.

Single CC requirements

Table 2-2: Simulation results for FR2-2 PDSCH requirements

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Antenna Conf. | Throughput | Num PRB | Ideal SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 120kHz/100MHz | MCS4 | TDLA30-650 | 2x2 Low | 70% | 66 | -2.2 |
|  | MCS13 | TDLA30-200 | 2x2 Low | 70% | 66 | 6.0 |
|  | MCS13 | TDLA30-650 | 2x2 Low | 30% | 66 | -0.9 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | 66 | 8.6 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | 32 | 8.8 |
| 480kHz/400MHz | MCS4 | TDLA10-200 | 2x2 Low | 70% | 66 | -2.7 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | 20 | 5.2 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | 32 | 4.9 |

CA requirements

Table 2-2: Simulation results for FR2-2 PDSCH requirements

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Antenna Conf. | Throughput | Num PRB | Ideal SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 120kHz/100MHz | MCS4 | TDLA30-650 | 2x2 Low | 70% | 66 | -2.3 |
|  | MCS13 | TDLA30-200 | 2x2 Low | 70% | 66 | 6.3 |
|  | MCS13 | TDLA30-650 | 2x2 Low | 30% | 66 | -1.1 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | 66 | 8.3 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | 32 | 8.5 |
| 480kHz/400MHz | MCS4 | TDLA10-200 | 2x2 Low | 70% | 66 | -2.3 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | 20 | 5.3 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | 32 | 5.1 |

Conclusion

In this paper, we provide our simulation results for FR2-2 PDSCH.

Reference

[1]   R4-2217391 WF on FR2-2 UE demodulation requirements. Qualcomm Incorporated