3GPP TSG-RAN WG4 Meeting # 105-e	R4-2219507

# Toulouse, France, November 14 – November 18, 2022

Title: 	Discussions on remaining issues on FR2-2 PUSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:        6.3.6.3.1

Document for:	Discussion

Background

As per [1], RAN4 left many open issues for FR2-2 PUSCH requirements. In this paper we provide our views.

Discussions

Channel model and MCS selection

The options for this issue are listed as follows:

Channel model for 16 QAM for PUSCH requirements - Option 1: Update channel model for MCS 16 with 120 kHz SCS as - For 1x2 Low, use TDLA30-650 - For 2x2 Low, use TDLA30-650 - Option 2: Update channel model for MCS 16 with 120 kHz SCS as - For 1x2 Low, use TDLA30-650 - For 2x2 Low, use TDLD30-650 - Option 3: Update channel model for MCS 16 with 120 kHz SCS as - For 1x2 Low, use TDLA30-650 - For 2x2 Low, use TDLD30-200 - Option 4: Update channel model for MCS 16 with 480 kHz SCS as - For 1x2 Low, use TDLA30-650 - For 2x2 Low, use TDLD30-200 - Option 5: Update channel model for MCS 16 with 120 kHz SCS and 480kHz SCS: - For 1x2 Low, use TDLA30-650 for 100MHz CBW, TDLA10-650 for 400MHz CBW. - For 2x2 Low, use TDLD30-200 for 100MHz CBW, TDLD10-200 for 400MHz CBW. MCS and number Tx/Rx branches for PUSCH requirements <Way forward> Proposals regarding 64 QAM requirements. - Option 1: MCS 20 with 1T2R Low and MCS 18 with 2T2R Low - Note: If the final link budget agreement indicate 20Db SNR limit could be applied for FR2-2 - Option 2: MCS20 with 1T2R and 2T2R Low

The simulation results for CP-OFDM are captured in Table 2-1 and Table 2-2 with simulation assumption captured in [2].

Table 2-1: Simulation results for CP-OFDM for minimum bandwidth for each SCS

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP (WO PN) | 70% of max TP (With PN) | Performance Loss (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | -2.5 | -2.1 | 0.4 |
|  |  |  |  | 2x2 Low | 2.0 | 2.4 | 0.4 |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 8.8 | 9.6 | 0.7 |
|  |  |  |  | 2x2 Low | 17.4 | 19.1 | 1.7 |
| 120 | 100 | 16 | TDLD30-200 | 2x2 Low | 10.5 | 11.3 | 0.8 |
| 120 | 100 | 18 | TDLD30-200 | 2x2 Low | 11.6 | 12.6 | 1.0 |
| 120 | 100 | 20 | TDLD30-200 | 1x2 Low | 9.7 | 10.9 | 1.2 |
|  |  |  |  | 2x2 Low | 13.6 | 15.2 | 1.6 |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | -3.8 | -3.5 | 0.3 |
|  |  |  |  | 2x2 Low | -0.3 | 0.2 | 0.5 |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 8.0 | 8.7 | 0.7 |
|  |  |  |  | 2x2 Low | 15.4 | 17.1 | 1.7 |
| 480 | 400 | 16 | TDLD10-200 | 2x2 Low | 10.2 | 11.0 | 0.8 |
| 480 | 400 | 18 | TDLD10-200 | 2x2 Low | 11.4 | 12.4 | 1.0 |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low | 9.7 | 10.9 | 1.2 |
|  |  |  |  | 2x2 Low | 13.4 | 15.1 | 1.7 |

Table 2-2: Simulation results for CP-OFDM for 120kHz/400MHz

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP (WO PN) | 70% of max TP (With PN CPE) | 70% of max TP (With PN CPE+ICI) | Performance Loss (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 400 | 4 | TDLA30-650 | 1x2 Low | -2.3 | -2.1 | / | 0.2 |
|  |  |  |  | 2x2 Low | 2.1 | 2.3 | / | 0.2 |
| 120 | 400 | 16 | TDLA30-650 | 1x2 Low | 8.9 | 9.5 | / | 0.6 |
|  |  |  |  | 2x2 Low | 17.9 | 19.4 | / | 1.5 |
| 120 | 400 | 16 | TDLD30-200 | 2x2 Low | 10.6 | 11.4 | / | 0.8 |
| 120 | 400 | 18 | TDLD30-200 | 2x2 Low | 11.7 | 12.7 | 13.4 | 1.0 |
| 120 | 400 | 20 | TDLD30-200 | 1x2 Low | 9.9 | 11.1 | 11.1 | 1.2 |
|  |  |  |  | 2x2 Low | 13.7 | 15.4 | 15.4 | 1.7 |

In the first meeting of this WI, RAN4 agreed to select FRC (MCS) to satisfy that the performance degradation due to phase noise is less than 1dB, we copy this agreement as follows:

Issue 1-4-1: Study on phase noise impact for requirements derivation Companies deliver two sets of ideal simulation results for requirement discussion. Result set#1 is without phase noise and set#2 is with phase noise. - No Tx phase noise is modelled - Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less than 1 dB) - 70 GHz carrier frequency is assumed

This criteria is also accepted for FR2-2 PDSCH requirements definition. Based on our understanding, phase noise is unpredictable in the real test hence the performance loss due to phase noise should be included in impairment results and the ideal simulation alignment should be based on results without phase noise modelled. This procedure is also used for Rel-15 FR2 PUSCH/PDSCH requirements definition.

Proposal 1: RAN 4 to select the MCS and channel model based on following criteria:

- The performance loss due to phase noise is not much more than 1dB

- The performance loss due to phase noise should be included in impairment results and the ideal simulation alignment should be based on results without phase noise modelled.

Based on the above agreements, we marked the requirements that performance loss is more than 1dB with yellow.  We can observe following:

- For cases with MCS16, 2T2R, TDLA, the performance loss is much more than 1dB and target SNR is quite closed to 20dB. If we use TDLD instead, performance is much improved and the performance loss is less than or equal to 1dB.

- For cases with MCS20, 2T2R, the performance loss is much more than 1dB. If we use MCS18 instead, the performance loss can be reduced to less than 1dB

Observation 1: For cases with MCS16, 2T2R, TDLA, the performance loss is much more than 1dB and target SNR is quite closed to 20dB. If we use TDLD instead, performance is much improved and the performance loss is less than or equal to 1dB.

Observation 2: For cases with MCS20, 2T2R, the performance loss is much more than 1dB. If we use MCS18 instead, the performance loss can be reduced to less than 1dB

Therefore, we propose the cases listed in Table 2-3 for final FR2-2 CP-OFDM PUSCH requirements definition:

Proposal 2: Use cases listed in Table 2-3 for final FR2-2 CP-OFDM PUSCH requirements definition.

Table 2-3: Proposed cases for FR2-2 CP-OFDM PUSCH requirements

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD30-200 | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 20 | TDLD30-200 | 1x2 Low | 70% of maximum TP |
| 120 | 100 | 18 | TDLD30-200 | 2x2 Low | 70% of maximum TP |
| 120 | 400 | 4 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 400 | 16 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD10-200 | 2x2 Low | 70% of maximum TP |
| 120 | 400 | 20 | TDLD10-200 | 1x2 Low | 70% of maximum TP |
| 120 | 400 | 18 | TDLD10-200 | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD10-200 | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low | 70% of maximum TP |
| 480 | 400 | 18 | TDLD10-200 | 2x2 Low | 70% of maximum TP |

Channel bandwidth for PUSCH requirements with 120kHz SCS

We listed the issues as follows:

Channel bandwidth for PUSCH requirements with 120 kHz SCS <Agreement> Define PUSCH BS demodulation requirements for 120kHz SCS with 100MHz and 400MHz - Further discuss test applicable rules considering the mandatory CHBW sets for BS

Based on our understanding, the existing applicability rules for BS supporting different bandwidth can be reused for FR2-2 PUSCH requirements. We listed it as follows:

For each subcarrier spacing declared to be supported, the test requirements for a specific channel bandwidth shall apply only if the BS supports it (see D.7 in table 4.6-1). Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

Proposal 3: Reuse existing applicability rules for BS supporting different bandwidth to FR2-2 PUSCH requirements.

Conclusion

In this paper we provide our views on open issues for FR2-2 PUSCH requirements. The proposals are:

Proposal 1: RAN 4 to select the MCS and channel model based on following criteria:

- The performance loss due to phase noise is not much more than 1dB

- The performance loss due to phase noise should be included in impairment results and the ideal simulation alignment should be based on results without phase noise modelled.

Observation 1: For cases with MCS16, 2T2R, TDLA, the performance loss is much more than 1dB and target SNR is quite closed to 20dB. If we use TDLD instead, performance is much improved and the performance loss is less than or equal to 1dB.

Observation 2: For cases with MCS20, 2T2R, the performance loss is much more than 1dB. If we use MCS18 instead, the performance loss can be reduced to less than 1dB

Proposal 2: Use cases listed in Table 2-3 for final FR2-2 CP-OFDM PUSCH requirements definition.

Proposal 3: Reuse existing applicability rules for BS supporting different bandwidth to FR2-2 PUSCH requirements.

Reference

[1]   R4-2214388_WF_PUSCH_FR2-2. Nokia, Nokia Shanghai Bell.