3GPP TSG-RAN WG4 Meeting # 105	R4-2219510

# Toulouse, France, Novemeber 14- November 18, 2022

Title: 	Simulation results on FR2-2 PRACH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       6.3.6.3.3

Document for:	Discussion

Background

As per [1], RAN4 close the discussions on FR2-2 PRACH demodulation requirements. In this paper we provide our simulation results.

Simulation results

| Format | SCS | Prach sequence length | Ncs | Propagation condition | Frequency Offset (Hz) | Time estimation tolerance | Ideal SNR | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A2 | 120kHz | 139 | 69 | AWGN | 0 | 70ns | -14.3 | -12.3 |
| B4 | 120kHz | 139 | 69 | AWGN | 0 | 70ns | -17.8 | -15.8 |
| C2 | 120kHz | 139 | 69 | AWGN | 0 | 70ns | -14.3 | -12.3 |
| A2 | 120kHz | 139 | 69 | TDLA30-650 | 7100 | 220ns | -7.6 | -5.6 |
| B4 | 120kHz | 139 | 69 | TDLA30-650 | 7100 | 220ns | -10.9 | -8.9 |
| C2 | 120kHz | 139 | 69 | TDLA30-650 | 7100 | 220ns | -7.9 | -5.9 |
| A2 | 120kHz | 571 | 285 | AWGN | 0 | 70ns | -20.3 | -18.3 |
| B4 | 120kHz | 571 | 285 | AWGN | 0 | 70ns | -23.5 | -21.5 |
| C2 | 120kHz | 571 | 285 | AWGN | 0 | 70ns | -20.2 | -18.5 |
| A2 | 120kHz | 571 | 285 | TDLA30-650 | 7100 | 220ns | -14.2 | -12.2 |
| B4 | 120kHz | 571 | 285 | TDLA30-650 | 7100 | 220ns | -17 | -15 |
| C2 | 120kHz | 571 | 285 | TDLA30-650 | 7100 | 220ns | -13.6 | -11.6 |
| A2 | 120kHz | 1151 | 575 | AWGN | 0 | 70ns | -23.3 | -21.3 |
| B4 | 120kHz | 1151 | 575 | AWGN | 0 | 70ns | -26.7 | -24.7 |
| C2 | 120kHz | 1151 | 575 | AWGN | 0 | 70ns | -23.2 | -21.2 |
| A2 | 120kHz | 1151 | 575 | TDLA30-650 | 7100 | 220ns | -17.4 | -15.4 |
| B4 | 120kHz | 1151 | 575 | TDLA30-650 | 7100 | 220ns | -20.3 | -18.3 |
| C2 | 120kHz | 1151 | 575 | TDLA30-650 | 7100 | 220ns | -17.3 | -15.3 |
| A2 | 480kHz | 139 | 69 | AWGN | 0 | 18ns | -14.2 | -12.2 |
| B4 | 480kHz | 139 | 69 | AWGN | 0 | 18ns | -17.7 | -15.7 |
| C2 | 480kHz | 139 | 69 | AWGN | 0 | 18ns | -14.3 | -12.3 |
| A2 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68ns | -6.9 | -4.9 |
| B4 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68ns | -13 | -11 |
| C2 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68ns | -6.8 | -4.8 |
| A2 | 480kHz | 571 | 285 | AWGN | 0 | 18ns | -20.3 | -18.3 |
| B4 | 480kHz | 571 | 285 | AWGN | 0 | 18ns | -23.7 | -21.7 |
| C2 | 480kHz | 571 | 285 | AWGN | 0 | 18ns | -20.2 | -18.2 |
| A2 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68ns | -13.3 | -11.3 |
| B4 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68ns | -16.7 | -14.7 |
| C2 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68ns | -13.4 | -11.4 |

Conclusion

In this paper we provide our simulation results for FR2-2 PRACH requirements.

Reference

[1]   R4-2217374 WF on PRACH demodulation requirements for FR2-2. Samsung