3GPP TSG-RAN WG4 Meeting #105	R4-2219533

Toulouse, France, 14 – 18 November, 2022

Title: 	Further simulation results for PRS accuracy

Source: 	Huawei, HiSilicon

Agenda item:	5.3

Document for:	Discussion

Introduction

RRM accuracy requirements for Rel-17 positioning are discussed in RAN4#104-bis-e, and the outcomes are captured in [1]. Based on [1], the following issues need to be further discussed.

- PRS-RSRPP accuracy

In this paper we will provide further simulation results for PRS-RSRPP.

Discussion

Simulation assumption

Issue 4-1-3: PRS accuracy with 4 samples Agreements: - Define PRS-RSRPP absolute accuracy requirements with 4 samples and number of repetitions ≥ 1 for PRS Es/Iot ≥ -3 dB and -13 dB. Issue 4-1-4: PRS-RSRPP accuracy with 1 sample Agreements: - Define PRS-RSRPP absolute accuracy requirements with 1 sample for same number of repetitions and side conditions (PRS Es/Iot) as used for PRS-RSRP absolute accuracy requirements with 1 sample.

The simulation results for PRS-RSRPP is approved at RAN4#103-e in [2]. In [3] we have provided our initial results based on [2]. Based on above agreements [1] in RAN4#104-e, the PRS-RSRPP accuracy will be defined for following cases:

- Case 1: 4-sample, repetitions ≥ 1, PRS Es/Iot ≥ -13 dB

- Case 2: 4-sample, repetitions ≥ 1, PRS Es/Iot ≥ -3 dB

- Case 3: 1-sample, repetitions ≥ 1, PRS Es/Iot ≥ -6 dB

- Case 4: 1-sample, repetitions ≥ 1, PRS Es/Iot ≥ TBD dB

  - The PRS Es/Iot for Case 4 is TBD because it has not been decided for PRS-RSRP.

The updates of results in this paper compared to [3] are

- For Case 1 and Case 3, provide new results based on repetitions ≥ 1. In [3] the results for smallest RB size of each SCS are based on repetitions ≥ 4.

- For Case 2, provide new results. In [3] there was no result for Case 2.

The results are summarized in the next section, and the updates compared to [3] are highlighted. For Case 4, we will provide our results after the PRS Es/Iot is decided.

Simulation results

Table 1 to Table 3 list the PRS-RSRPP error for Case 1 to Case 3 in section 2.1.

Table 1: PRS-RSRPP error for 4-sample, Es/Iot of -13dB

| Parameter | Value |  |  | PRS-RSRPP error |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 5% | 50% | 95% |
|  | 15 | 24 | 1 | -6.5 | -3.5 | 4.0 |
|  |  | 52 | 1 | -3.1 | -2.3 | 2.5 |
|  |  | 104 | 1 | -2.2 | -1.5 | 1.8 |
|  | 30 | 24 | 1 | -6.3 | -2.7 | 3.9 |
|  |  | 48 | 1 | -4.0 | -1.5 | 2.6 |
|  |  | 132 | 1 | -3.2 | -1.5 | 2.5 |
|  | 60, FR1 | 24 | 1 | -5.8 | -3.0 | 3.8 |
|  |  | 64 | 1 | -3.7 | -1.0 | 2.4 |
|  |  | 132 | 1 | -3.0 | -1.5 | 2.3 |
|  | 60, FR2 | 24 | 1 | -5.8 | -3.0 | 3.8 |
|  |  | 64 | 1 | -3.7 | -1.0 | 2.4 |
|  |  | 132 | 1 | -3.0 | -1.5 | 2.3 |
|  | 120 | 32 | 1 | -5.3 | -3.0 | 3.3 |
|  |  | 64 | 1 | -2.6 | -1.0 | 1.6 |
|  |  | 128 | 1 | -2.3 | -1.5 | 1.5 |

Table 2: PRS-RSRPP error for 4-sample, Es/Iot of -3dB

| Parameter | Value |  |  | PRS-RSRPP error |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 5% | 50% | 95% |
|  | 15 | 24 | 1 | -6.0 | -2.5 | 2.1 |
|  |  | 52 | 1 | -3.2 | -1.5 | 2.6 |
|  |  | 104 | 1 | -2.3 | -1.0 | 1.6 |
|  | 30 | 24 | 1 | -6.1 | -3.5 | 2.1 |
|  |  | 48 | 1 | -2.4 | -1.4 | 1.6 |
|  |  | 132 | 1 | -2.1 | -1.0 | 1.4 |
|  | 60, FR1 | 24 | 1 | -5.5 | -3.0 | 2.0 |
|  |  | 64 | 1 | -2.3 | -1.0 | 1.4 |
|  |  | 132 | 1 | -2.0 | -1.5 | 1.3 |
|  | 60, FR2 | 24 | 1 | -5.5 | -3.0 | 2.0 |
|  |  | 64 | 1 | -2.3 | -1.0 | 1.4 |
|  |  | 132 | 1 | -2.0 | -1.5 | 1.3 |
|  | 120 | 32 | 1 | -5.0 | -3.0 | 1.9 |
|  |  | 64 | 1 | -2.4 | -1.0 | 1.5 |
|  |  | 128 | 1 | -2.1 | -1.5 | 1.2 |

Table 3: PRS-RSRPP error for 1-sample, Es/Iot of -6dB

| Parameter | Value |  |  | PRS-RSRPP error |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 5% | 50% | 95% |
|  | 15 | 24 | 1 | -5.0 | -2.5 | 2.1 |
|  |  | 52 | 1 | -3.2 | -1.5 | 2.6 |
|  |  | 104 | 1 | -2.3 | -1.0 | 1.6 |
|  | 30 | 24 | 1 | -5.1 | -3.5 | 2.0 |
|  |  | 48 | 1 | -2.4 | -1.4 | 1.6 |
|  |  | 132 | 1 | -2.1 | -1.0 | 1.4 |
|  | 60, FR1 | 24 | 1 | -4.9 | -3.0 | 1.8 |
|  |  | 64 | 1 | -2.3 | -1.0 | 1.4 |
|  |  | 132 | 1 | -2.0 | -1.5 | 1.3 |
|  | 60, FR2 | 24 | 1 | -4.5 | -3.0 | 2.0 |
|  |  | 64 | 1 | -2.3 | -1.0 | 1.4 |
|  |  | 132 | 1 | -2.0 | -1.5 | 1.3 |
|  | 120 | 32 | 1 | -4.2 | -3.0 | 1.9 |
|  |  | 64 | 1 | -2.4 | -1.0 | 1.5 |
|  |  | 128 | 1 | -2.1 | -1.5 | 1.2 |

Proposal: Take the simulation results in Table 1 to Table 3 for deriving the PRS-RSRPP accuracy.

Conclusions

In this paper we provided further simulation results for PRS-RSRPP.

Proposal: Take the simulation results in Table 1 to Table 3 for deriving the PRS-RSRPP accuracy.

Reference

- R4-2214491, WF on NR Positioning Enhancements (Part 1), Ericsson

- R4-2211061, Simulation assumption for PRS-RSRPP, Huawei, HiSilicon

- R4-2213551, Simulation results for PRS-RSRPP, Huawei, HiSilicon