3GPP TSG-RAN WG4 Meeting #105	R4-2219556

Toulouse, France, 14 – 18 November, 2022

Title: 	Discussion on NW-B requirements with MUSIM gaps

Source: 	Huawei, HiSilicon

Agenda item:	8.22.2.4

Document for:	Discussion

Introduction

NW-B requirements with MUSIM gaps are discussed in RAN4#104-bis-e, and the outcomes are captured in [1]. Based on [1], the following issues need to be further discussed.

- Whether to define requirements for NW-B

- General principle for NW-B requirements, if to be defined

In this paper we will provide our views on NW-B requirements with MUSIM gaps.

Discussion

Whether to define requirements for NW-B

Issue 1-6-1: Whether to define network B requirements - Proposals: - Option 1: Deprioritize NW B requirement in R18 - Option 2: No measurement requirements in network B will be defined by RAN4 - Option 3: RAN4 does not define new UE idle/inactive measurement requirements for measurements on Network B for a UE configured with MUSIM gaps (exiting UE measurement requirement for Idle/Inactive mode applies) - Option 4: RAN4 to define measurement requirement for NW-B Idle mode which is helpful for both NW-A and NW-B Agreements: No

We suggest RAN4 not to define NW-B requirements in Rel-18.

On one hand, it is up to UE implementation to request and use the MUSIM gaps, so it is not straightforward to define requirements. One MUSIM gap may be used for multiple NW-B operations, e.g. one MUSIM gap is requested for all measurement including serving cell and mobility measurements, and when UE needs to transmit or receive in the serving cell, UE may prioritize serving cell measurement over neighbour cell measurement, and may not be able to meet the reselection requirements.

On the other hand, we do not see clear necessity to define NW-B requirements. Based on the discussions so far, the concern with not defining NW-B requirements is that UE may request but not utilize MUSIM gaps configured by NW-A. We do not think a reasonable UE would request MUSIM gaps for nothing as it will only degrade its own performance in NW-A (NW-A can always schedule other UEs during the MUSIM gaps), but the UE will try to complete NW-B operations with minimum impact to NW-A.

Proposal 1: RAN4 not to define NW-B requirements in Rel-18.

General principle for NW-B requirements, if to be defined

Issue 1-6-2: Network B requirements if it will be defined - Proposals: - P1: RAN4 has to define some requirement for NW B in R18, RAN4 shall focus on cell reselection - P2: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP) - P3: UE measurement requirements for idle/inactive mode in network B shall be the same as existing Idle/Inactive measurement requirements. - UE measurement requirement for Idle/Inactive mode on network B, when assigned with requested MUSIM gaps, are the same as for network A. Hence, same as legacy requirements Agreements: Postpone discussion until there is outcome of issue 1-6-1

If requirements for measurements in NW B are anyway to be defined, the existing measurement requirements for IDLE/INACTIVE should be used as baseline. We do not think UE is expected to meet a more stringent requirements just because the measurement is performed based on MUSIM gaps, e.g. UE should not be assumed to perform more than one measurement per DRX cycle.

As the periodicity of MUSIM gaps can be up to 5.12s, it is possible that the periodicity of MUSIM gaps is larger than the DRX cycle in NW B. In this case, the requirements should be based on the maximum between the DRX cycle and the periodicity of MUSIM gaps.

Proposal 2: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP).

Conclusions

In this paper we provided our views on NW-B requirements with MUSIM gaps.

Proposal 1: RAN4 not to define NW-B requirements in Rel-18.

Proposal 2: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP).

Reference

- R4-2217261, WF on RRM requirements for Rel-17 MUSIM gaps, vivo