3GPP TSG-RAN WG4 Meeting #105-e	R4-2219655

# Toulouse, 14th – 18th Nov, 2022

Title: 	Simulation results on BS coverage enhancement demod PUCCH

Source: 	Huawei, HiSilicon

Agenda item:	6.4.2.2

Document for:	Information

Background

During RAN4#104bis-e meeting, PUCCH demodulation performance of Rel-17 NR coverage enhancement was discussed and the fruitful agreements are noted in [1]. In this contribution, we provide our final updated simulation results about the BS coverage enhancement PUCCH demodulation requirements.

Simulation

PUCCH demodulation with Joint Channel Estimation (JCE)

Here we provide the final updated simulation results for PUCCH JCE for alignment, as shown in Table 2.1-2 and Table 2.1-3, based on simulation assumption as shown in Table 2.2-1.

Table 2.1-1 Simulation assumption for PUCCH JCE

| Parameter | Value |
| --- | --- |
| PUCCH formats | Format 1 and Format 3 |
| Duplex mode | FDD and TDD |
| SCS | 15/30 kHz for FR1 (TDD and FDD) |
|  | 60/120 kHz for FR2 (TDD) |
| Antenna | 1T2R |
| Channel Model | TDLA30-10 for FR1 |
|  | TDLA30-75 for FR2 |
| TDD UL-DL pattern (same as PUSCH) | 7D1S2U, S=6D:4G:4U for 15/30kHz SCS |
|  | DDSUU, S=10D:2G:2U for 60/120kHz SCS |
| PRB number | 1 PRB |
| CBW | 5MHz for 15kHz SCS |
|  | 10MHz for 30kHz SCS |
|  | 50MHz for 60/120kHz SCS |
|  | Interested companies can bring simulation results for the largest CHBW for each SCS. |
| Intra-slot frequency hopping | Disabled |
| Inter-slot frequency hopping | Disabled |
| aTDW length = cTDW length (same as PUSCH) | 2 for TDD |
|  | 8 for FDD |
| PUCCH repetition number (same as PUSCH) | 2 for TDD |
|  | 8 for FDD |
| Start symbol | 0 |
| Number of symbols | 14 |
| phase and power offset (same as PUSCH) | Ideal |
| Test metric | format 1: Test NACK to ACK detection probability and ACK missed detection probability format 3: Test UCI block error probability |
| Bit payload | format 1: 2 bits format 3: 16 bits |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift (format1) | 0 |
| Index of orthogonal cover code (timeDomainOCC) (format1) | 0 |
| Modulation order (format 3) | QPSK |
| DMRS configuration (format 3) | DMRS1+1, DMRS1+0 |

Table 2.1-2 Simulation results for PUCCH JCE (PF1)

| Case Number | SCS/kHz | SNR@1% 1st BLER |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | FDD |  | TDD |  |
|  |  | ACK missed | NACK to ACK | ACK missed | NACK to ACK |
| 1 | 15 | -11.23 | -11.16 | -6.92 | -5.00 |
| 2 | 30 | -10.22 | -11.68 | -6.18 | -5.50 |
| 3 | 60 | / | / | -7.65 | -7.09 |
| 4 | 120 | / | / | -7.94 | -6.19 |

Table 2.1-3 Simulation results for PUCCH JCE (PF3)

| Case Number | SCS/kHz | SNR@1% 1st BLER |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | FDD |  | TDD |  |
|  |  | DMRS 1+0 | DMRS 1+1 | DMRS 1+0 | DMRS 1+1 |
| 1 | 15 | -8.34 | -8.52 | -2.21 | -2.25 |
| 2 | 30 | -8.08 | -8.01 | -2.03 | -1.95 |
| 3 | 60 | / | / | -0.06 | -0.57 |
| 4 | 120 | / | / | -0.53 | -0.64 |

Conclusion

In this contribution, we provide our final updated simulation results on BS PUCCH demodulation requirements for coverage enhancement for alignment.

Reference

R4-2216915, Email discussion summary for [104-bis-e][321] NR_cov_enh_Demod, RAN4#104bis-e, Moderator (China Telecom)