3GPP TSG-RAN WG4 Meeting #105-e	R4-2219671

# Toulouse, 14th – 18th Nov, 2022

Title: 	Simulation results on satellite NTN demod PUCCH

Source: 	Huawei, HiSilicon

Agenda item:	6.2.6.2.2

Document for:	Information

Background

During RAN4#104bis-e meeting, WF [1] on NTN SAN demodulation requirements was approved. In this contribution, we provide our updated simulation results about the NTN PUCCH demodulation requirements.

Simulation

PUCCH format 0

Here we provide the updated simulation results for PUCCH format 0 for alignment, as shown in Table 3.1-2, based on simulation assumption as shown in Table 3.1-1.

Table 3.1-1 Simulation assumption for PUCCH format 0

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Number of OFDM symbols | 2 |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | ACK missed detection probability <1% |

Table 3.1-2 Ideal simulation results for PUCCH format 0

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x1 | 6.92 |
| 2 | 15 | 5 | 1x2 | 0.72 |
| 3 | 30 | 10 | 1x1 | 8.58 |
| 4 | 30 | 10 | 1x2 | 1.42 |

PUCCH format 1

Here we provide the updated simulation results for PUCCH format 1 for alignment, as shown in Table 3.2-2, based on simulation assumption as shown in Table 3.2-1.

Table 3.2-1 Simulation assumption for PUCCH format 1

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 3.2-2 Ideal simulation results for PUCCH format 1

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ACK missed detection | NACK to ACK |
| 1 | 15 | 5 | 1x1 | -1.19 | -0.29 |
| 2 | 15 | 5 | 1x2 | -7.21 | -5.77 |
| 3 | 30 | 10 | 1x1 | 0.44 | 3.65 |
| 4 | 30 | 10 | 1x2 | -6.25 | -3.39 |

PUCCH format 2

ACK missed detection

Here we provide the updated simulation results for PUCCH format 2 for ACK missed detection for alignment, as shown in Table 3.3.1-2, based on simulation assumption as shown in Table 3.3.1-1.

Table 3.3.1-1 Simulation assumption for PUCCH format 2 for ACK missed detection

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 3.3.1-2 Ideal simulation results for PUCCH format 2 for ACK missed detection

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x1 | 9.64 |
| 2 | 15 | 5 | 1x2 | 1.38 |
| 3 | 30 | 10 | 1x1 | 8.88 |
| 4 | 30 | 10 | 1x2 | 1.21 |

UCI BLER

Here we provide the updated simulation results for PUCCH format 2 for UCI BLER for alignment, as shown in Table 3.3.2-2, based on simulation assumption as shown in Table 3.3.2-1.

Table 3.3.2-1 Simulation assumption for PUCCH format 2 for UCI BLER

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

Table 3.3.2-2 Ideal simulation results for PUCCH format 2 for UCI BLER

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x1 | 3.94 |
| 2 | 15 | 5 | 1x2 | -2.00 |
| 3 | 30 | 10 | 1x1 | 3.93 |
| 4 | 30 | 10 | 1x2 | -1.98 |

PUCCH format 3

Here we provide the updated simulation results for PUCCH format 3 for alignment, as shown in Table 3.4-2, based on simulation assumption as shown in Table 3.4-1.

Table 3.4-1 Simulation assumption for PUCCH format 3

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DM-RS, Additional DM-RS |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

Table 3.4-2 Ideal simulation results for PUCCH format 3

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | DMRS1+0 | DMRS1+1 |
| 1 | 15 | 5 | 1x1 | 3.95 | 4.06 |
| 2 | 15 | 5 | 1x2 | -2.28 | -2.54 |
| 4 | 30 | 10 | 1x1 | 7.14 | 6.96 |
| 5 | 30 | 10 | 1x2 | -0.09 | -0.27 |

PUCCH format 4

Here we provide the updated simulation results for PUCCH format 4 for alignment, as shown in Table 3.5-2, based on simulation assumption as shown in Table 3.5-1.

Table 3.5-1 Simulation assumption for PUCCH format 4

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DMRS, Additional DM-RS |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

Table 3.5-2 Ideal simulation results for PUCCH format 4

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | DMRS1+0 | DMRS1+1 |
| 1 | 15 | 5 | 1x1 | 14.46 | 14.29 |
| 2 | 15 | 5 | 1x2 | 1.37 | 1.35 |
| 3 | 30 | 10 | 1x1 | 17.22 | 16.19 |
| 4 | 30 | 10 | 1x2 | 1.65 | 1.65 |

PUCCH format 1 (multi-slot)

Here we provide the updated simulation results for PUCCH format 1 (multi-slot) for alignment, as shown in Table 3.6-2, based on simulation assumption as shown in Table 3.6-1.

Table 3.6-1 Simulation assumption for PUCCH format 1 (multi-slot)

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 3.6-2 Ideal simulation results for PUCCH format 1 (multi-slot)

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ACK missed detection | NACK to ACK |
| 1 | 15 | 5 | 1x1 | -4.15 | -2.27 |
| 2 | 15 | 5 | 1x2 | -10.26 | -9.08 |
| 3 | 30 | 10 | 1x1 | -3.57 | -0.20 |
| 4 | 30 | 10 | 1x2 | -9.88 | -7.18 |

Conclusion

In this contribution, we provide our updated simulation results on NTN PUCCH demodulation requirements.

Reference

R4-2210662, WF on NTN SAN demodulation requirements, RAN4#103-e, Huawei, HiSilicon