3GPP TSG-RAN WG4 Meeting # 105                	R4-2219730

Toulouse, France, November 14 – 18, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUSCH simulation results for the extension to 71 GHz

Agenda item: 	6.3.6.3.1

Document for: 	Discussion

Introduction

This paper presents PUSCH simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Simulation assumptions

Simulation assumptions are shown below and are derived considering the agreements on the WF [1].

Table 1 Common simulation parameters

| Parameter | value |
| --- | --- |
| Transform precoding | Disabled and enabled |
| CP | Normal CP |
| Carrier frequency | 70 GHz |
| DMRS symbols | [1 9] |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| SCS | 120, 480, and 960 kHz |
| TDD pattern | 120 kHz: 3D1S1U, S=10D:2G:2U 480 kHz: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U 960 kHz: 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U |
| DMRS type | Type 1 with single-symbol DM-RS (l0=2) |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups with data | 2 |
| Symbol length | 10 |
| Start symbol index | 0 |
| PRTS | On |
| HARQ | ON |
| Time domain resource allocation type | type B |
| Frequency domain resource without transform precoding | For 120 kHz SCS and - CHW= 100 MHz: 66 - CHW= 400 MHz: 264 For 480 kHz SCS and - CHW= 400 MHz: 66 |
| Frequency domain resource without transform precoding | For 120 kHz SCS and - CHW= 100 MHz: 64 For 480 kHz SCS and - CHW= 400 MHz:64 |
| MCS index | MCS 4, 16, 18, 20 |
| Propagation condition | TDLA30-200 TDLA30-650 TDLD30-200 TDLA10-200 TDLA10-650 TDLD10-200 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz; CBW = 400 MHz SCS = 480 kHz; CBW = 400 MHz |
| Phase noise model | 38808 Set2 |
| Number of HARQ transmissions | 4 |
| RV sequence | {0.2.3.1} |
| Testing metric | SNR at 70% of the maximum throughput |
| Phase noise compensation | CPE only |

Simulation results

Note: Simulation results are being updated in accordance to the agreed WF. Previous simulation results are shown here for information.

Table 2 Simulation results with transform precoding disabled

| SCS | CBW | MCS | Channel model | Antenna configuration | SNR @ 70% TPUT With PN |  | SNR @ 70% TPUT Without PN |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal | Ideal | Ideal | Impair |
| 120 | 100 | 4 | TDLA30-650 | 1x2 | -3.3 | 0.0 | -3.3 | 0.0 |
| 120 | 100 | 4 | TDLA30-650 | 2x2 | 0.5 | 3.8 | 0.4 | 3.7 |
| 120 | 100 | 16 | TDLA30-650 | 1x2 | 8.5 | 11.8 | 8.1 | 11.4 |
| 120 | 100 | 16 | TDLA30-650 | 2x2 | 16.0 | 19.3 | 15.3 | 18.6 |
| 120 | 100 | 16 | TDLD30-200 | 1x2 | 7.1 | 10.4 | 6.8 | 10.1 |
| 120 | 100 | 16 | TDLD30-200 | 2x2 | 24.1 | 27.4 | 22.3 | 25.6 |
| 120 | 100 | 18 | TDLD30-200 | 1x2 | 8.2 | 11.5 | 7.9 | 11.2 |
| 120 | 100 | 18 | TDLD30-200 | 2x2 | 24.3 | 27.6 | 22.2 | 25.5 |
| 120 | 100 | 20 | TDLD30-200 | 1x2 | 10.2 | 13.5 | 9.6 | 12.9 |
| 120 | 100 | 20 | TDLD30-200 | 2x2 |  |  |  |  |
| 120 | 400 | 4 | TDLA10-650 | 1x2 | -3.4 | -0.1 | -3.5 | -0.2 |
| 120 | 400 | 4 | TDLA10-650 | 2x2 | 0.9 | 4.2 | 0.8 | 4.1 |
| 120 | 400 | 16 | TDLA10-650 | 1x2 | 8.3 | 11.6 | 8.0 | 11.3 |
| 120 | 400 | 16 | TDLA10-650 | 2x2 | 17.4 | 20.7 | 16.7 | 20.0 |
| 120 | 400 | 16 | TDLD10-200 | 1x2 | 7.1 | 10.4 | 6.8 | 10.1 |
| 120 | 400 | 16 | TDLD10-200 | 2x2 | 24.4 | 27.7 | 22.5 | 25.8 |
| 120 | 400 | 18 | TDLD10-200 | 1x2 | 8.2 | 11.5 | 7.7 | 11.0 |
| 120 | 400 | 18 | TDLD10-200 | 2x2 | 24.5 | 27.8 | 22.4 | 25.7 |
| 120 | 400 | 20 | TDLD10-200 | 1x2 | 10.2 | 13.5 | 9.6 | 12.9 |
| 120 | 400 | 20 | TDLD10-200 | 2x2 |  |  |  |  |
| 480 | 400 | 4 | TDLA10-650 | 1x2 | -3.7 | -0.4 | -3.8 | -0.5 |
| 480 | 400 | 4 | TDLA10-650 | 2x2 | 0.4 | 3.7 | 0.3 | 3.6 |
| 480 | 400 | 16 | TDLA10-650 | 1x2 | 7.9 | 11.2 | 7.5 | 10.8 |
| 480 | 400 | 16 | TDLA10-650 | 2x2 | 15.8 | 19.1 | 15.1 | 18.4 |
| 480 | 400 | 16 | TDLD10-200 | 1x2 | 6.8 | 10.1 | 6.5 | 9.8 |
| 480 | 400 | 16 | TDLD10-200 | 2x2 | 23.8 | 27.1 | 22.0 | 25.3 |
| 480 | 400 | 18 | TDLD10-200 | 1x2 | 8.2 | 11.5 | 7.8 | 11.1 |
| 480 | 400 | 18 | TDLD10-200 | 2x2 | 25.1 | 28.4 | 22.5 | 25.8 |
| 480 | 400 | 20 | TDLD10-200 | 1x2 | 10.2 | 13.5 | 9.6 | 12.9 |
| 480 | 400 | 20 | TDLD10-200 | 2x2 |  |  |  |  |

Table 3 Simulation results with transform precoding enabled

| SCS | CBW | MCS | Channel model | Antenna configuration | SNR @ 70% TPUT With PN |  | SNR @ 70% TPUT Without PN |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal | Ideal | Ideal | Impair |
| 120 | 100 | 4 | TDLA30-650 | 1x2 | -3.0 | 0.3 | -3.1 | 0.2 |
| 480 | 400 | 4 | TDLA10-650 | 1x2 | -3.5 | -0.2 | -3.5 | -0.2 |

References

- R4-2217372, WF on PUSCH demodulation requirements for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2217371, WF on general aspects for FR2-2 demodulation requirements, Huawei,HiSilicon

- R4-2210664, WF on general and BS aspects for FR2-2 demodulation requirements, Intel Corporation

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation

- R4-2212675, PUSCH simulation results for the extension to 71 GHz, Nokia, Nokia Shanghai Bell