3GPP TSG-RAN WG4 Meeting #105 	R4-2219734

Toulouse, France, November 14 – November 18, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues for PRACH demodulation requirements in FR2-2

Agenda item:	6.3.6.3.3

Document for:	Discussion

Introduction

This paper presents discussion on remaining details for PRACH demodulation requirements for FR2-2.

Discussion

As part of the PRACH vendor declarations, the following fields relate to PRACH [2].

D.103; PRACH format and SCS; Declaration of the supported PRACH format(s) as specified in TS 38.211 [20], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [20], i.e.: - For BS type 1-O: 15 kHz, 30 kHz or both. - For BS type 2-O: 60 kHz, 120 kHz or both.; c; x; x. D.113; PRACH format with LRA = 1151 for 15 kHz SCS and LRA = 571 for 30 kHz SCS; Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both.; c; x; n/a.

For the normal PRACH, D.103 can be updated to include requriements for 120 kHz in FR2-1 and FR2-2 and requirements for 480 kHz. As for the enhanced formats with LRA=1151, and 571, the field D113 can be updated to differentiate between FR2-2 requirements and FR1 reuqirements.

Update D.103 with the following description:

| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [20], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [20], i.e.: - For BS type 1-O: 15 kHz, 30 kHz or both. - For BS type 2-O. in FR2-1: 60 kHz, 120 kHz or both. - For BS type 2-O. in FR2-2: 120 kHz, 480 kHz or both. |
| --- | --- | --- |

Update D.113 as:

| D.113 | PRACH format with LRA = 1151 for 15 kHz SCS, LRA = 571 for 30 kHz SCS, LRA = 1151 and LRA = 571 for 120 kHz SCS and LRA = 571 for 480 kHz SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e.: -For BS type 1-O: 15 kHz, 30 kHz or both. -For BS type 2-O: 120 kHz, 480 kHz or both. |
| --- | --- | --- |

Conclusion

In the paper, the following Proposals were made:

- Update D.103 with the following description:

| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [20], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [20], i.e.: - For BS type 1-O: 15 kHz, 30 kHz or both. - For BS type 2-O. in FR2-1: 60 kHz, 120 kHz or both. - For BS type 2-O. in FR2-2: 120 kHz, 480 kHz or both. |
| --- | --- | --- |

Update D.113 as:

| D.113 | PRACH format with LRA = 1151 for 15 kHz SCS, LRA = 571 for 30 kHz SCS, LRA = 1151 and LRA = 571 for 120 kHz SCS and LRA = 571 for 480 kHz SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e.: -For BS type 1-O: 15 kHz, 30 kHz or both. -For BS type 2-O: 120 kHz, 480 kHz or both. |
| --- | --- | --- |

- References

- R4-2217374, WF on PRACH demodulation requirements for FR2-2, Samsung

- TS 38.141-2, NR; Base Station (BS) conformance testing, Part 2: Radiated conformance testing