3GPP TSG-RAN WG4 Meeting # 105	R4-2219842

# Electronic Meeting, November 14 – 18, 2022

Source: 	Qualcomm Incorporated

Title: 	Discussion on FR2-2 CQI Requirements

Agenda item:	6.3.6.2.3

Document for:	Discussion

- Introduction

In this paper, we address the options included in the Way Forward from RAN4#104-bis-e [2] related to FR2-2 UE CQI Performance requirements

- Scope of the CSI Reporting requirements

For the purpose of defining CQI requirements, the scope of the CQI requirements is limited to SCS 120kHz, CBW 100 MHz, the maximum testable SNR is 9.8dB.

Observation 1: For the purposes of CQI Requirements in FR2-2, consider Max SNR = 9.8dB;

Considering the SNR values under discussion, we support using Rank 2 according to the configuration used in the legacy FR2-1 CQI Requirements for static channel.

Proposal 1: For the definition of CQI Requirements, R4 should use Rank 2;

The maximum SNR considered does not allow us to introduce 64 QAM requirements, so we support excluding this from the requirements.

Proposal 2: For the definition of CQI Requirements, R4 should not consider 64QAM;

- SNR Test Points

## SNR Test points for 16 QAM;

Proposal 3: For 16QAM CQI Requirements, we support using 8/9 dB Test Points;

## SNR Test points for QPSK;

Proposal 4: For 16QAM CQI Requirements, we support using 0/1 dB Test Points;

- Conclusions

Observation 1: For the purposes of CQI Requirements in FR2-2, consider Max SNR = 9.8dB;

Proposal 1: For the definition of CQI Requirements, R4 should use Rank 2;

Proposal 2: For the definition of CQI Requirements, R4 should not consider 64QAM;

Proposal 3: For 16QAM CQI Requirements, we support using 8/9 dB Test Points;

Proposal 4: For 16QAM CQI Requirements, we support using 0/1 dB Test Points;

- References

[1] RP-213540, “Revised WID:  Extending current NR operation to 71 GHz”, 3GPP TSG RAN Meeting #94-e, Electronic Meeting, December 06-17, 2021

[2] R4-2217391, “WF on FR2-2 UE demodulation requirements”, 3GPP TSG-RAN WG4 Meeting #104-bis-e, Electronic Meeting, October 10 – 19, 2022

[3] 3GPP TR 38.884, “Study on enhanced test methods for FR2 NR UEs (Release 18, v18.2.0)”;