3GPP TSG-RAN WG4 Meeting #105	R4- 2219867

Toulouse, France, 14 – 18 November 2022

Source: 	Huawei, HiSilicon

Title:	TP on UE ACLR and SEM for IoT NTN

Agenda Item:	9.5.5

Document for:	Approval

# 1	Introduction

The WF[1] agreements in RAN4#104bis-e were captured in this contribution. The text proposal is for clauses 6 of TS36.102.

# 2	Discussion

The text proposal uses TN ACLR and SEM requirements as the baseline for IoT NTN, which are subject to the confirmation of the coexistence simulations.

# References

- R4-2217748 Way Forward on UE A-MPR and Emissions requirements, ZTE, RAN4#104bis-e

# Text proposal for TS 36.102

### 6.5.2A	 Out of band emission for category M1

#### 6.5.2A.1	Spectrum emission mask

Editor’s note: <Assume TN as baseline, and reconfirm after coexistence verification. >

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned E-UTRA channel bandwidth. For frequencies offset greater than ΔfOOB as specified in Table 6.5.2A.1.1-1 the spurious requirements in subclause 6.5.3 are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2A.1-1 for the specified channel bandwidth.

Table 6.5.2A.1-1: Category M1 spectrum emission mask

| ΔfOOB (MHz) | 1.4 MHz | Measurement bandwidth |
| --- | --- | --- |
| 0-1 | -10 | 30 kHz |
| 1-2.5 | -10 | 1 MHz |
| 2.5-2.8 | -25 | 1 MHz |

NOTE:	 As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5.2A.2	Additional Spectrum Emission Mask for category M1

Editor’s note: < RAN4 to determine the A-MPR values for NS_24, NS_100 or new FCC requirement (issue 3-2-4) by simulations/measurements.>

#### 6.5.2A.3	Adjacent Channel Leakage Ratio for category M1

Editor’s note: < Wait for coexistence verification outcome. >

E-UTRA category M1 Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA category M1 channel power and adjacent E-UTRA category M1 channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2A.3-1. If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the value specified in Table 6.5.2A.3-1.

Table 6.5.2A.3-1: Category M1 ACLR requirements

|  | Channel bandwidth / E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | 1.4 MHz |
| E-UTRAACLR | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4/-1.4 |

### 6.5.2B	 Out of band emission for category NB1 and NB2

#### 6.5.2B.1	Spectrum emission mask

Editor’s note: <Assume TN as baseline, and reconfirm after coexistence verification.>

The spectrum emission mask of the category NB1 and NB2 UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned category NB1 or NB2 channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.5.2B.1-1 the spurious requirements in subclause 6.5.3 are applicable.

The power of any category NB1 or NB2 UE emission shall not exceed the levels specified in Table 6.5.2B.1-1. The spectrum emission limit between each ΔfOOB is linearly interpolated.

Table 6.5.2B.1-1: Category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

#### 6.5.2B.2	Additional Spectrum Emission Mask for category NB1 and NB2

Editor’s note: < RAN4 to determine the A-MPR values for NS_24, NS_100 or new FCC requirement (issue 3-2-4) by simulations/measurements.>

#### 6.5.2B.3	Adjacent Channel Leakage Ratio for category NB1 and NB2

Editor’s note: < Wait for coexistence verification outcome. And the GSM ACLR may be redundant since the OBW requirement implies an ACLR of 23 dB with even smaller offset (i.e. +/100kHz) of the adjacent channel. >

Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The assigned category NB1or NB2 channel power and adjacent channel power are measured with filters and measurement bandwidths specified in Table 6.5.2B.3-1. If the measured adjacent channel power is greater than –50dBm then the category NB1 or NB2 UE ACLR shall be higher than the value specified in Table 6.5.2B.3-1. GSMACLR requirement is intended for protection of GSM system. UTRAACLR requirement is intended for protection of UTRA and E-UTRA systems.

Table 6.5.2B.3-1: category NB1 and NB2 UE ACLR requirements

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | 20 dB | 37 dB |
| Adjacent channel center frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |