3GPP RAN4 Meeting #105	R4-2219896

# Toulouse FR, November 14 - 18, 2022

Agenda item:	6.3.1

Source: 	Qualcomm Incorporated

Title: 	60 GHz UE PTRS capabilities

Document for:	Approval

# Introduction

In this paper we discuss PTRS capabilities as they relate to FR2-2.

# PTRS configuration for 16QAM and 64QAM

PTRS configuration has as much impact on phase noise mitigation as the phase noise profile itself. This aspect is not related to the EVM calculator, but it in effect is a discussion on the side conditions for the EVM test. RAN2 signalling already exists (ptrs-DensityRecommendationSetUL) to allow the UE to convey to the network what it’s PTRS preferences are in terms of preferred MCS/BW thresholds. We propose to make a slight modification to the PTRS configuration note in 38.101-2 for n263, indicating the EVM test should follow the IE indication when determining how to configure PTRS for 16QAM and 64QAM.

The existing note in the EVM tables is ‘NOTE 1:	PTRS is configured for 16 QAM and 64 QAM’ and we propose to void the note and add ‘NOTE 2:  PTRS is configured according to the UE preference in ptrs-DensityRecommendationSetUL’

Proposal: Void NOTE 1 in FR2-2 EVM tables 6.4.2.1-2a, 3a, 3b and add NOTE 2:  PTRS is configured according to the UE preference in ptrs-DensityRecommendationSetUL’

# Conclusions

Proposal: Void NOTE 1 in FR2-2 EVM tables and add NOTE 2:  PTRS is configured according to the UE preference in ptrs-DensityRecommendationSetUL’

# References

[1] 3GPP TS 38.306 V17.2.0 ;User Equipment (UE) radio access capabilities