3GPP TSG-RAN WG4 Meeting #105	R4-2219909

Toulouse, France, November 14 – November 18, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	PRS measurement requirements for Deferred MT-LR Periodic Location

Agenda item:	4.4

Document for:	Discussion

Introduction

At RAN4 #104-e, the discussion on specifying PRS measurement period requirements for deferred MT-LR for Rel-16 NR positioning [1] [2] was continued. In particular, measurement requirements for “Periodic Location” were discussed after measurement requirements for events other than “Periodic Location” were settled at RAN4 #103-e with endorsed Draft CR [3].

For the event “Periodic Location”, no agreement was achieved at RAN4 #104-e and the Draft CR [4] was postponed.

This paper presents our view on the open issue and presents a proposal how to specify PRS measurement requirements for “Periodic Location” in TS 38.133.

Discussion

At RAN4 #103-e, measurement requirements for events other than “Periodic Location” were settled with endorsed Draft CR [3], which determines the start of the PRS measurement period as follows:

At RAN4 #104-e, the following open issue was treated, and the candidate options are listed in the WF [2]:

#### Issue 2-2-1: Start of measurement period for deferred MT-LR Candidate options: - Option 1 (HW, CATT, E///, Nokia) - For the event of periodic location in deferred MT-LR, update the start point of measurement period as: - the time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs. - Option 2 (QC) - For the event of periodic location in deferred MT-LR, update the start point of measurement period as: - For deferred MT-LR with “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall finish the measurements by time T where T is the time when “Periodic Location” event occurs. The requirements apply when the time T is known by the UE no later than Tref, where Tref=T-TRSTD,Total-maxTavailable_PRS,i. - Option 3 (vivo) - No need to specify starting point or ending point for periodic location report. - As long as UE finishes the measurements during the periodic interval, UE can report the measurements periodically

During the discussion, the Draft CR was revised in [4] based on option 3 and then was postponed. The proposed change was as follows:

In our view, option 3 should not be selected for “Periodic Location” due to following reasons:

- In fact, TS 23.073, clause 4.1a.5 does not distinguish between different (i.e. periodic/non-periodic) trigger cases in regard to the time of reporting. There is no mention of measurement duration and subsequent report. Thus, adopting option 1 for non-periodic events (i.e. PRS measurement after trigger event occurs) as shown in section 1 and option 2 or 3 for periodic events (i.e. PRS measurement prior to trigger event occurs) has some level of inconsistency.

- For example, in case the period interval is rather long compared to the PRS measurement duration, then additional, undesired variance in the PRS measurement is introduced with option 3 and also option 2, because of measurement timing uncertainty.

- Thereagainst, option 1 has no measurement timing uncertainty, as it requires to perform the PRS measurement as close as possible to event T and accurately defines the measurement start point and furthermore ensures consistency with location reporting for non-periodic events.

Thus, option 1 is preferred. This results in following proposed change to TS 38.133 for “Periodic Location” aligning requirements to non-periodic trigger events:

RAN4 to agree the following change in clause 9.9.2.5, and corresponding changes in 9.9.3.5 and 9.9.4.5, on Deferred MT-LR with “Periodic Location”:

For deferred MT-LR as defined in clause 4.1a.5.1 [TS 23.273], the time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

Conclusion

This paper has presented our view on the open issue related to specifying PRS measurement requirements for deferred MT-LR with “Periodic Location” in the context of Rel-16 NR positioning.

As part of this discussion, we make following proposal:

- RAN4 to agree the following change in clause 9.9.2.5, and corresponding changes in 9.9.3.5 and 9.9.4.5, on Deferred MT-LR with “Periodic Location”:

For deferred MT-LR as defined in clause 4.1a.5.1 [TS 23.273], the time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

The companion CR is submitted in [5].

- References

- R4-2214254, Email discussion summary for [104-e][201] Maintenance_R15_R16_RRM, Moderator (Huawei)

- R4-2214324, WF on R15 and R16 RRM maintenance	, Huawei, Hisilicon

- R4-2210969, Draft CR 38.133 on PRS measurement period R16, Huawei, Hisilicon

- R4-2214721, Draft CR 38.133 on PRS measurement period R16, Huawei, Hisilicon

- R4-2219910, CR 38.133 on PRS measurement period for Deferred MT-LR (Rel-16), Nokia, Nokia Shanghai Bell