3GPP TSG-RAN WG4 Meeting #105	R4-2219924

Meeting, November 14 – November 18, 2022

Agenda Item:	8.22.2.4

Source:	MediaTek Inc.

Title:	Discussion on NW B RRM requirements for MUSIM

Document for:	Discussion

# Introduction

In NR Rel-17 specification, RAN4 has introduced gap patterns particularly for MUSIM purpose. However, their corresponding RRM requirements were not specified. The impact of MUSIM gaps on NW A will require new RRM requirements to be defined, where NW A in RRC_CONNECTED state. However, whether to introduce RRM requirements for NW B, where NW B is in RRC_IDLE/_INACTIVE state, is still under discussion, and in this paper, we provide our view.

# Discussion

The issue on whether to define the requirements for NW B is captured below from the WF [1]:

Issue 1-6-1: Whether to define network B requirements - Proposals: - Option 1: Deprioritize NW B requirement in R18 - Option 2: No measurement requirements in network B will be defined by RAN4 - Option 3: RAN4 does not define new UE idle/inactive measurement requirements for measurements on Network B for a UE configured with MUSIM gaps (exiting UE measurement requirement for Idle/Inactive mode applies) - Option 4: RAN4 to define measurement requirement for NW-B Idle mode which is helpful for both NW-A and NW-B Agreements: No

For Issue 1-6-1, we support Option 2. In our view, it is not straightforward to identify the new requirements in NW B (in IDLE/INACTIVE) when we could have different configurations for MUSIM gaps. Therefore, we prefer not to define measurement requirements in NW B.

Proposal 1: No measurement requirements in network B will be defined by RAN4

# Summary

In this contribution, the introduction of RRM requirements for NW B under MUSIM operation was discussed and the following proposal is calculated:

Proposal 1: No measurement requirements in network B will be defined by RAN4

# References

- R4-2217261, WF on RRM requirements for Rel-17 MUSIM gaps, vivo.