3GPP TSG-RAN WG4 Meeting #105 	R4-2219948

# Toulouse, France, Nov. 14 – Nov. 18, 2022

Agenda Item:	8.8.3.3

Source: 	Ericsson

Title:	Discussion on L3 measurements and procedures

Document for:	Discussion

# Introduction

The WID [1] includes the following objective of the core part:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4-layer DL MIMO

  - Enhanced RF requirements:

    - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs

    - The legacy spherical coverage requirement for reception from a single direction will be kept

    - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier

  - Enhanced RRM requirements:

    - The following requirements should be studied and specified if necessary:

      - L1-RSRP measurement delay

      - L3 measurement delay (both cell detection delay and measurement period can be considered)

        - The starting point is the enhancements related to L1-RSRP measurement enhancements

      - RLM and BFD/CBD requirements

      - Scheduling/measurement restrictions

      - TCI state switching delay with dual TCI

      - Receive timing difference between different directions (different QCL Type D RSs)

The WID [1] includes the following objective of the performance part:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier

  - RRM Performance requirements and test cases corresponding to the new core requirements

  - RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused

  - UE demodulation requirements:

    - Specify performance requirements for enhanced FR2-1 UEs supporting up to 4 DL MIMO layers with dual TCI with different QCL typeD on a single component carrier.

      - Demod requirements on up to 4 DL MIMO layers

      - CSI requirements to support up to 4-layer with 2 TCIs and Rel-17 mTRP Type I codebook

    - Extension of requirements to cover intra-band CA can be considered after the single carrier requirements are defined

The agreed WF on RRM from RAN4#104-e is captured in [2], but there was no progress on L3 measurements discussion.

The agreed WF on general RRM issues from RAN4#105 is captured in [3], where there was no agreement on L3 measurements, but the following issues have been discussed:

Issue 2-1-1: Feasibility/necessity of enhancing requirements for L3 measurements

- Proposals

  - Option 1 (MTK, Intel, Huawei, Qualcomm, OPPO, Ericsson, Samsung, NTT DOCOMO): No L3 measurement requirements enhancement in R18 multi-Rx chain WI.

    - Option 1a (MTK, Intel): Not to enhance L3 measurement requirement in R18 multi-Rx chain WI.

    - Option 1b (Huawei): For R18 multi-Rx DL reception, it is suggested not to enhance L3 measurement requirements.

      - For R18 multi-Rx DL reception, UE is not assumed to support simultaneous L3 measurements with two different beam directions.

      - In R18, it is suggested not to enhance the beam sweeping factor for L3 measurement requirements due to searcher limitation.

      - For R18 multi-Rx DL reception, UE is not assumed to support simultaneous L1 measurements and L3 measurements and the sharing factor between L1 and L3 needs to be kept in L3 measurement requirements.

      - For R18 multi-Rx DL reception, the enhancement on scheduling restrictions due to L3 measurements is not considered.

    - Option 1c (Qualcomm): RAN4 to not discuss the following items for RRM requirement enhancements under the work item of FR2 multi-Rx chain DL reception:

      - RRM requirement enhancements that require

        - more than two cell searchers for cell and SSB detection and SSB measurements

        - L3 measurements by using concurrently activated multiple Rx panels, e.g. FR2 SCell activation delay reduction

      - Idle mode measurements

    - Option 1d (OPPO): Except L3 measurement delay reduction, other impacts on L3 measurements can be studied based on the enhancements on L1 measurements.

    - Option 1e (Ericsson) 1: Deprioritize L3 measurements in the current WI.

      - Cell detection and L3 measurement period, SSB-based:

        - No simultaneous reception for SSB-only based measurements and procedures are assumed.

  - Option 2 (CMCC, LGE, Xiaomi, vivo, ZTE, Nokia): L3 measurement requirements are enhanced in R18 multi-Rx chain WI.

    - Option 2a (CMCC): For L3 measurement in connected mode, all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG) need to be considered for the delay improvement with multi-beam simultaneous reception.

    - Option 2b (LGE):

      - L3 measurement enhancements should be considered since L3 measurement through multi-antenna panels is a natural UE behaviour when multi-antenna panels are activated.

      - Introduce the following case for L3 measurement enhancement first

        - For L3 measurement in connected mode, including all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG).

    - Option 2c (Xiaomi): To agree on the following L3 measurement requirement enhancement:

      - Handover to FR2-1 unknown cell

      - FR2-1 unknown Scell activation

      - L3 measurement in connected mode, including intra-frequency measurement with and without MG.

    - Option 2d (vivo):

      - FR2 SCell activation delay for unknown SCell can be enhanced by reducing L1-RSRP measurement time and/or cell search time for multi-Rx chain UE.

      - Handover delay, PSCell addition/change delay and SCG activation delay for FR2 unknown target cell/PSCell can be enhanced by reducing cell search time for multi-Rx chain UE.

      - FFS if enhancement of L3 measurements in connected mode for multi-Rx chain simultaneous DL reception is feasible.

      - A new capability, which is different from simultaneousReceptionDiffTypeD-r16 or other L1 measurement related UE capability, should be introduced for L3 procedure/measurement delay reduction for UE supporting multi-Rx chain simultaneous reception, if L3 measurement enhancement is agreed to be specified.

    - Option 2e (ZTE): Referring to whether L3 measurement can be enhanced by multi-panel reception, we believe same logic as the enhancement for L1 measurement can be referenced and would not lead to much additional workload.

    - Option 2f (Nokia):

      - RAN4 to consider the following requirements for multi Rx enhancements in RRC connected mode

        - a.	L3 measurement in connected mode

        - b.	Handover to FR2-1

        - c.	RRC re-establishment

        - d.	Intra-frequencey measurements

        - e.	Inter-frequency measurements

      - RAN4 to focus on L3 enhancements for requirements in RRC connected.

        - RAN4 to discuss the need of L3 enhancements for requirements in RRC idle and inactive in a later phase of the work item.

Issue 1-1-6: Simultaneous L3 measurements and L1 measurements

- Proposals

  - Option 1 (MTK, Huawei, Intel, NTT DOCOMO, Qualcomm, Ericsson): In R18 multi-Rx, UE is not required to perform both L3 measurements and L1 measurements at a time.

  - Option 2 (vivo, OPPO, Nokia, ZTE, Xiaomi, Apple): RAN4 to identify use cases for simultaneous L3 measurements and L1 measurements and study the feasibility

  - Option 3 (CMCC, Nokia, ZTE): To consider simultaneous L3 measurements and L1 measurements with multi-RX chain reception

    - Further check whether Klayer1_measurement of 1.5 can be removed (or Klayer1_measurement = 1)

Issue 1-5-2: UE capability of simultaneous reception of measured RS and data

- Proposals

  - Option 1 (Huawei): For R18 multi-Rx DL reception, the enhanced RRM requirements can be developed based on the following principles:

    - UE can be assumed to support simultaneous data receptions with two different beam directions.

    - UE does not support simultaneous data receptions and L1 measurements with different beam directions.

    - UE does not support simultaneous data receptions and L3 measurements.

  - Option 2 (Samsung, LGE, ZTE): For UE capability of simultaneous reception of measured RS and data, if UE support simultaneousReceptionDiffTypeD-r16, it should support:

    - simultaneous reception of L1 measured RS and PDCCH/PDSCH/TRS/CSI-RS for CQI

    - simultaneous reception of L3 measured RS and PDCCH/PDSCH/TRS/CSI-RS for CQI

  - Option 3 (MTK, Qualcomm, vivo, Huawei): RAN4 to further study how to guarantee that network can know when to apply schedule restriction or when not to.

  - Option 4 (OPPO): UE capability of simultaneous reception of L1/L3 RS and data can be decided after the feasibility issues of L1 and L3 enhancements are concluded.

  - Option 5 (Nokia, Xiaomi): For R18 multi-Rx DL reception, the enhanced RRM requirements can be developed based on the following principles:

    - UE can be assumed to support simultaneous data receptions with two different beam directions.

    - UE supports simultaneous data receptions and L1 measurements with different beam directions.

    - UE supports simultaneous data receptions and L3 measurements with different beam directions.

    - UE supports simultaneous L1 and L3 measurements with different beam directions.

  - Option 6 (Ericsson): for requirements definition, RAN4 to consider

    - UE supports simultaneous data receptions and L1 measurements with different beam directions.

    - UE supports simultaneous L1 measurements with different beam directions.

# Discussion

In this WI [1], a UE simultaneously receives two RSs from different QCL type D infos. Since SSBs are never target RSs in QCL info but rather an associated RS, it can be assumed that CSI-RS based measurements and procedures are in the main scope of the WI [1]. Hence, RAN4 shall not assume simultaneous reception for SSB-based cell detection and L3 SSB-based measurements.

- Proposal 1: No simultaneous reception for SSB-only based measurements and procedures, i.e., SSB+SSB, is assumed in this WI [1].

The above suggests that L3 measurements can be deprioritized in this WI [1]. This applies to all three potential scenarios with L3 measurements:

- Simultaneous reception of two RSs, when both are intended for L3 measurements

- Simultaneous reception of two RSs, when one is intended for L3 measurement and another one is for L1 measurements

- Simultaneous reception of RS and data, when the RS reception is a part of a L3 measurement

- Proposal 2: Deprioritize simultaneous reception of two RSs, when both are intended for L3 measurements.

- Proposal 3: Deprioritize simultaneous reception of two RSs, when one is intended for L3 measurement and another one is for L1 measurements.

- Proposal 4: Deprioritize simultaneous reception of RS and data, when the RS reception is a part of a L3 measurement.

# Summary

The following have been observed and proposed in this contribution.

- Proposal 1: No simultaneous reception for SSB-only based measurements and procedures, i.e., SSB+SSB, is assumed in this WI [1].

- Proposal 2: Deprioritize simultaneous reception of two RSs, when both are intended for L3 measurements.

- Proposal 3: Deprioritize simultaneous reception of two RSs, when one is intended for L3 measurement and another one is for L1 measurements.

- Proposal 4: Deprioritize simultaneous reception of RS and data, when the RS reception is a part of a L3 measurement.

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.

[3] R4-2217246, WF on RRM impacts and general aspects for multi-Rx, vivo, Oct. 2022.