3GPP TSG-RAN WG4 Meeting #105                           		R4-2219979

Toulouse, France, November 14 – November 18, 2022

Source: 	Huawei, HiSilicon

Title: 	TP to TS 36.108 (section 5)

Agenda Item:	9.5.4

Document for:	Approval

# Introduction

Based on the pre-arranged work-split, a TP to TS 36.108 v0.0.1 is provided for section 5.

# Discussion

Content of this TP was drafted based on the TS 36.108 v0.0.1, reusing the content of the existing TS 38.108 specification for the SAN operating in NR, where possible.

Some of the related decisions are provided below, for reference:

| Tdoc | Title | Comments |
| --- | --- | --- |
| R4-2214380, RAN4#104-e | WF on system parameter for IoT over NTN | Issue 1-2-1: Channel bandwidth and spectral utilization for eMTC over NTN Option 2: For Cat-M1, define only 1.4MHz UE channel bandwidth, with corresponding transmission bandwidth configuration and minimum guardband as defined in 36.101. See no value of a different approach for SAN. |
|  |  | Issue 1-2-2: Channel bandwidth and spectral utilization for IoT over NTN Option 2: For NB-IoT, re-use existing requirements from 36.101 for UE channel bandwidth, transmission bandwidth configurations for 15kHz and 3.75kHz SCS, and minimum guardband. |
|  |  | Issue 1-3: Channel spacing Reuse TN specification 36.101/36.104 nominal channel spacing for Cat-M1 and NB-IoT. Further discuss the frequency gap for NB-IoT over NTN deployment. Removal of in-band/guard band item. |
|  |  | Issue 1-4-1: Channel raster and EARFCN Further discuss the 100khz and 200khz channel raster in 2nd round. Option 1: 200khz channel raster Option 2: 100khz raster with signaling Option 2 agreed for band 255 and band 256. |
| R4-2214467, RAN4#104-e | Way forward on NB-IoT/eMTC NTN agenda item 12.5.1 & 12.5.4 | Issue 1-2-1: Deployment scenario (standalone vs other operating modes) Agree to focus the WI requirements work on the standalone deployment scenario for NB-IoT and cat-M1 operation. |
| R4-2217747, RAN4#104bis-e | Way Forward on UE RF and System Parameters for IoT NTN | SAN Foffset: Agree on reuse of Foffset for NTN operation as baseline way forward. Further improvements can be discussed, but if not agreeable next meeting they would not be covered in this WI. |
|  |  | Channel Numbering: Band 256 and 255 are based on 100 kHz channel raster NTN Operating Band; ΔFRaster (kHz); Downlink; Uplink. FDL_low (MHz); NOffs-DL; Range of NDL; FUL_low (MHz); NOffs-UL; Range of NUL. 256; 100; 2170; 229076; 229076 –<1>– 229375; 1980; 261844; 261844 –<1>– 262143. 255; 100; 1525; 228736; 228736 – <1>– 229075; 1626.5; 261504; 261504 –<1>– 261843. NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. <Add for Cat-M1 the following>: This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4 MHz.. NOTE: there could be option to have 200 kHz channel raster in the future for other bands. FFS whether to capture the above NOTE in TS or WF. |

Furthermore, a consolidated specification content proposed by the TS 36.108 specification editor was also considered during drafting.

# Conclusions

Based on the above discussion, the following proposal was formulated:

Proposal 1: Approve the attached TP to TS 36.108.

# References

[1]	3GPP TS 36.108, v 0.0.1

# Annex A: TP to TS 36.108

------------------------------ Modified section ------------------------------

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWChannel	Channel bandwidth

BWConfig	Transmission bandwidth configuration

ΔFRaster

FC 	Frequency of the carrier centre frequency

FDL

FDL_high	The highest frequency of the downlink operating band

FDL_low	The lowest frequency of the downlink operating band

FUL

FUL_high	The highest frequency of the uplink operating band

FUL_low	The lowest frequency of the uplink operating band

MDL	Offset of category NB1/NB2 Channel Number to downlink EARFCN

MUL	Offset of category NB1/NB2 Channel Number to uplink EARFCN

NDL	downlink EARFCN

NOffs-DL	Offset used for calculating downlink EARFCN

NOffs-UL	Offset used for calculating uplink EARFCN

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

Ntone 15kHz 	Transmission bandwidth configuration for category NB1 and NB2 with 15 kHz sub-carrier spacing, expressed in units of tones.

Ntone 3.75kHz	Transmission bandwidth configuration for category NB1 and NB2 with 3.75 kHz sub-carrier spacing, expressed in units of tones.

NUL 	uplink EARFCN

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Cat-M1

eMTC

EARFCN

NB1

NB2

NB-IoT

NPSS

NSSS

SAN

RB

------------------------------ Next modified section ------------------------------

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and SAN channel bandwidths defined in the present release of specifications.

## 5.2	Operating bands

SAN is designed to operate in the operating bands defined in table 5.2-1.

Table 5.2-1 SAN operating bands

| SAN operating band | Uplink (UL) operating band SAN receive UE transmit |  |  | Downlink (DL) operating band SAN transmit UE receive |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from 256. |  |  |  |  |  |  |  |

## 5.3	Satellite Access Node channel bandwidth

### 5.3.1	SAN channel bandwidth for eMTC

The SAN channel bandwidth supports a single RF carrier in the uplink or downlink at the SAN.

SAN channel bandwidth for eMTC operation is 1.4 MHz (6 RB).

Figure 5.3.1-1 shows the relation between the SAN channel bandwidth for eMTC and the transmission bandwidth configuration (NRB). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

.

Figure 5.3.1-1: Definition of SAN channel bandwidth and transmission bandwidth configuration for eMTC operation

The placement of the UE channel bandwidth is flexible but can only be completely within the SAN channel bandwidth. The SAN shall be able to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the number of carrier resource blocks on the RF carrier, in any part of the carrier resource blocks.

### 5.3.2	SAN channel bandwidth for NB-IoT

The SAN channel bandwidth supports a single RF carrier in the uplink or downlink at the SAN.

SAN channel bandwidth for standalone NB-IoT operation is 200 kHz for both the UE category NB1 and NB2. NB-IoT requirements in present document are specified for SAN channel bandwidth as in table 5.3.2-1.

Table 5.3.2-1: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel (kHz) | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Figure 5.3.2-1 shows the relation between the SAN channel bandwidth for NB-IoT standalone operation and the transmission bandwidth configuration (Ntone 15kHz, or Ntone 3.75kHz). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Figure 5.3.2-1 Definition of SAN channel bandwidth and transmission bandwidth configuration for NB-IoT standalone operation

The placement of the UE channel bandwidth is flexible but can only be completely within the SAN channel bandwidth. The SAN shall be able to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the number of carrier resource blocks on the RF carrier, in any part of the carrier resource blocks.

For NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset as defined in Table 5.3.2-2.

Table 5.3.2-2: Foffset for NB-IoT standalone operation

| Lowest or H\ighest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | 200 kHz |

## 5.4	Channel arrangement

### 5.4.1	Channel spacing

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent carriers is defined as following:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2)) / 2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective carriers.

The channel spacing can be adjusted to optimize performance in a particular deployment scenario.

Nominal channel spacing for standalone NB-IoT operation is 200 kHz.

### 5.4.2	Channel raster

The global frequency raster is defined for all frequencies. The granularity of the global frequency raster is 100 kHz, which means that the carrier centre frequency must be an integer multiple of 100 kHz. For each operating band, a subset of frequencies from the global frequency raster are applicable and forms a channel raster with a granularity ΔFRaster.

### 5.4.3	Carrier frequency and EARFCN for eMTC

The carrier frequency for UE Cat-M1 in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 - 262143. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in table 5.4.3-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in table 5.4.3-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

Table 5.4.3-1 captures information on the band specific EARFCN ranges for DL and UL.

Table 5.4.3-1: E-UTRA channel numbers for NTN IoT operation with eMTC

| Operating band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 256 | 100 | 2170 | 229076 | 229076 –<1>– 229375 | 1980 | 261844 | 261844 –<1>– 262143 |
| 255 |  | 1525 | 228736 | 228736 – <1>– 229075 | 1626.5 | 261504 | 261504 –<1>– 261843 |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. In case of eMTC operation for Cat-M1 this implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4 MHz. |  |  |  |  |  |  |  |

### 5.4.4	Carrier frequency and EARFCN for NB-IoT

The carrier frequency for UE category NB1/NB2 in the downlink is designated by the EARFCN in the range 0 – 262143.

The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.7.3-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

The carrier frequency of category NB1/NB2 in the uplink is designated by EARFCN in the range 0 –262143.

The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.7.3-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

[NOTE:	For the carrier including NPSS/NSSS for stand-alone operation, MDL = -0.5.]

## 5.5	Requirements for contiguous spectrum

Unless otherwise stated, the requirements in the present specification apply for SAN configured with contiguous spectrum operation, only.

------------------------------ End of modified section ------------------------------