3GPP TSG-RAN WG4 Meeting RAN4#105     					R4-2219993 Toulouse, France

November 14 – 18, 2022

Agenda Item:	8.6.2

Source:	Lenovo, Motorola Mobility

Title:	EVM for 4x4 UL MIMO

Document for:  Approval

- Introduction

A new work item on further RF requirements enhancements for NR frequency range 1 was agreed in [1] and includes the following objective:

Specify the UE RF requirements to support 4Tx

  - First priority: 4x4 UL MIMO

In this contribution, we consider how the EVM requirements are applied for 4x4 UL MIMO.  This issue was previously addressed in [2].

- EVM for 4x4 UL MIMO

In TS 38.101-1 [2], the EVM for closed loop spatial multiplexing is defined in 6.4D.2.1 as follows:

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in clause 6.4.2.1 apply per layer.

However, the details of how the per-layer EVM should be measured have not been specified.

For 4x4 UL MIMO, the following agreements were made in the way forward [3].

- Define the EVM for 4 Tx UL MIMO transmission on a per layer basis.

- For full-rank transmission, measure the EVM using a zero-forcing MIMO receiver.

As a result of the first agreement, the EVM requirement for UL MIMO is defined per layer for both 2 Tx and 4 Tx.

From the second agreement, a zero-forcing MIMO receiver will be used to define EVM for 4x4 UL MIMO.  Because the zero-forcing receiver is uniquely defined for full-rank transmission, it follows that the zero-forcing MIMO receiver is given by (as in [4])

GZF=H-1

where H is the 4x4 effective channel matrix given by H=HW, H is the 4x4 channel matrix, and W is the 4x4 precoding matrix.

There is a fundamental difference between the EVM measurement for FR1 and FR2 in that for FR2 measurements are radiated while conductive measurements are used for FR1. The effects of antenna coupling and reverse intermodulation are included in radiated EVM measurement while these aspects are not included in conductive measurements taken at the antenna connectors.  As a result, the conductive EVM measurement may underestimate the magnitude of the transmitter noise and may also reflect a combining gain over the transmitter noise that is not achievable if the transmitter noise is highly correlated.

The EVM definition for 2 Tx transmit diversity explicitly removes the combining gain that results if the transmitter noise is uncorrelated by assuming worst case correlation of the transmitter noise [5].  The same approach has been proposed for 4 Tx transmit diversity in [6].  We now consider if this same approach can be considered when defining EVM for 4x4 UL MIMO as well one-, two-, and three-layer transmission.

For 4x4 UL MIMO, the signal at the antenna connectors is given by

y=H W x+n ,

where x is the 4x1 data vector, W is the 4x4 precoding matrix, H is the 4x4 channel matrix, and n is the 4x1 vector of transmitter noise at the antenna connectors.  If the zero-forcing receiver is used, then the data estimate is given by

x=HW-1H W x+n .

where HW is an estimate of HW based on reference symbols. If HW=HW, then

x=HW-1H W x+n

=x+v ,

where v=W-1 n. The covariance of the per-layer transmitter noise is then given by

Ev vH=W-1En nH W-1H

=WH Σ W .

where we have defined Σ=En nH  and used the fact that for a unitary matrix  W-1=WH.

If the transmitter noise at the antenna connectors is uncorrelated so that the covariance matrix can be represented by the diagonal matrix Σ=diagσ12,σ22,σ32,σ42, then the resulting noise variance for the i-th layer is given by

WH Σ Wi,i=j=14Wj,i2σj2 .

For a radiated measurement or in the case that directional couplers are introduced between the PA outputs for MPR/A-MPR measurements in order to emulate antenna coupling, the transmitter noise will be correlated.  In this case, the noise covariance for the i-th layer is given by

WH Σ Wi,i=j=14k=14Wj,i*Σj,kWk,i .

In the Appendix, it is shown that

j=14k=14Wj,i*Σj,kWk,i≤j=14Wi,jσj2

where Σi,i,=σi2.

If coupling between the antennas leads to significant correlation of the transmitter noise, and if the transmitter noise is uncorrelated in the absence of antenna coupling, then the combining gain for the i-th layer can be as large as the ratio

Gi=j=14Wi,jσj2j=14Wi,j2σj2 .

This maximum combining gain represents the maximum increase of the layer signal-to-noise ratio that can result if the conducted transmitter noise is independent while the radiated transmitter noise has worst-case correlation due to antenna coupling.

In order to estimate the covariance of the transmitter noise n, it would be necessary to form the estimate

n=H-1y-W x ,

where H is an estimate of the channel and x is either reference symbols or demodulated data.  Alternatively, if the variance of the transmitter noise at each of the antenna connectors is assumed to be equal so that σi2=σ2 for all i, then the maximum combining gain for the i-th layer is given by

Gi=j=14Wi,j2j=14Wi,j2 .

To avoid underestimating the radiated EVM with a conductive measurement, the maximum combining gain can be removed from the EVM measurement. If EVMi denotes the conductive EVM of the i-th layer, then the signal-to-noise ratio of the i-th layer is given by

SNRi=100EVMi2 .

The SNR of the i-th layer with the maximum combining gain removed is then given by

SNRi'=1Gi100EVMi2=100EVMiGi2=100EVMi'2

where EVMi'=EVMiGi.  Thus, the maximum combining gain Gi can be removed from the conductive EVM measurement EVMi by multiplying by the square root of Gi so that

EVMi'=EVMiGi.

Alternatively, since the correlation of the radiated transmitter noise may not be worst case, it may be better to scale the conductive EVM measurement by a fraction f of the maximum combining gain so that

EVMi'=EVMi ∙f ∙Gi,

where f is in the interval (0, 1].

For 4x4 UL MIMO, the agreed precoder is given by

W=121000010000100001 ,

so that the maximum combining gain is

j=14Wi,j2j=14Wi,j2=1 .

Thus, for this 4-layer precoder there is no combining gain over the transmitter noise for any of the MIMO layers, and at least from this perspective, the conductive EVM measurement should be similar to the radiated EVM measurement.

It can be noted that if the following 4x4 UL precoder were to be used (TPMI 3)

W=1411111-11-111-1-11-1-11 ,

then the maximum combining gain for the i-th layer is given by

Gi=j=14Wi,j2j=14Wi,j2=4 .

for all i. If this maximum combining gain is removed from the conductive EVM measurement EVMi, then the adjusted EVM is given by

EVMi'=EVMiGi=2 EVMi .

- EVM for M-layer transmission from N-antenna ports

In the case that the number of transmission layers is less than the number of antenna ports, the received signal is given by

y=H W x+n ,

where x is the Mx1 data vector, W is the NxM precoding matrix, H is the NxN channel matrix, and n is the Nx1 vector of transmitter noise at the antenna connectors. At least two unbiased linear receivers can be used to separate the layers, and these are the unbiased linear MMSE receiver and the pseudo-inverse receiver. The pseudo-inverse receiver Apseudo is given by

Apseudo=HWHHW-1HWH

When the pseudo-inverse receiver is applied to the M-layer transmission, the result is

Apseudo y=HWHHW-1HWH H W x+n

=x+HWHHW-1HWH H n ,

=x+v ,

where the covariance of the per layer transmitter noise v is given by

Ev vH=B Σ BH  ,

and the MxN matrix B is defined as

B=HWHHW-1HWH H= ApseudoH .

If the transmitter noise is uncorrelated so that Σ=diagσ12,…,σN2, then the noise variance of the i-th layer is given by

B Σ BHi,i=j=14Bi,j2σj2

If the transmitter noise is correlated the covariance matrix is not diagonal, then the noise variance for the i-th layer is given by

B Σ BHi,i=j=14k=14Bi,jΣj,kBi,k* .

From the result in the Appendix, it follows that

j=14k=14Bi,jΣj,kBi,k*≤j=14Bi,jσj2.

where Σi,i,=σi2.  Thus, if coupling between the antennas leads to significant correlation of the transmitter noise, and if the transmitter noise is uncorrelated in the absence of antenna coupling, then the maximum combining gain for the i-th layer is given by

Gi=j=14Bi,jσj2j=14Bi,j2σj2 .

As before, if it is assumed that σi2=σ2 for all i, then the maximum combining gain is given by

Gi=j=14Bi,j2j=14Bi,j2 .

To avoid underestimating the radiated EVM with the conductive measurement, the maximum combining gain can be removed from the conductive EVM measurement by multiplying by the square root of Gi so that

EVMi'=EVMiGi

Alternatively, since the correlation of the radiated transmitter noise may not be worst case, it may be better to scale the conductive EVM measurement by a fraction f of the maximum combining gain so that

EVMi'=EVMi ∙f ∙Gi,

where f is in the interval (0, 1].

If we consider the example two-layer precoder given by

W=122111111-1-1

and consider the case that H=I, then

B=HWHHW-1HWH H= WH.

For this example, we have

Gi=j=14Bi,j2j=14Bi,j2=4

for all i, and

EVMi'=EVMiGi=2 EVMi .

- EVM adjustment based on the MPR/A-MPR increase allowed for multi-antenna transmission

# In some cases, increased MPR or increased A-MPR is allowed for multi-antenna transmission.  These increased values are typically based on PA measurements taken using conductive measurements but with directional couplers used between the PA’s to emulate the coupling that occurs between antennas with limited isolation.  An example can be seen in Table 6.2.2-2 and 6.2D.2-1 below from [2].

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2D.2-1 Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

# Instead of using the maximum combining gain to estimate the radiated EVM from the conducted EVM, an alternative is to use the MPR increase allowed for multi-antenna transmission to estimate the EVM increase due to radiative coupling.  To adjust the conducted EVM measurement to reflect the increased MPR, there must be a mapping between the need for increased MPR and the increased EVM due to antenna coupling. The EVM that is allowed for each modulation type is shown in the Table 6.4.2.1-1 from [2] below.

Table 6.4.2.1-1: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

# The relationship between EVM and MPR for single antenna transmission can be seen in the Figure 1 below.

Figure 1: MPR vs. EVM for CP-OFDM Inner Allocation

# From this Figure, it can be observed that as the MPR is increased by 1 dB, the EVM is decreased by approximately 2.  This relationship can be expressed as

# EVM2=EVM1 2 ∆MPR=EVM1 2 ∆MPR2

# where EVM1 and EVM2 are the EVM values before and after the power reduction by ∆MPR.

# If it is assumed that this same relationship holds with respect to EVM and the increased MPR due to antenna coupling and reverse intermodulation, the relationship between the conducted EVM measurement and the radiated EVM measurement can be expressed as

# EVMi'=EVM  2 ∆MPR2

# where ∆MPR is the increased MPR allowed for dual Tx for the given modulation type.  In some cases, it may be better to adjust the radiated EVM by only a fraction f of this increase so that

# EVMi'=EVM∙f∙2 ∆MPR2 ,

# where f is in the interval (0, 1].

- Summary

# Based on the discussion and analysis above, we have the following proposals:

# Proposal 1:  For less than full-rank transmission, measure the EVM for less than four-layer 4x4 UL MIMO

# using the pseudo-inverse receiver.

# Proposal 2: To account for antenna correlation not observed in conductive measurements, increase the conductive EVM measurement by some fraction of the square root of the maximum combining gain so that

# EVMi'=EVMi ∙f ∙Gi

# where f is in the interval (0, 1] and apply this increased EVM against the EVM requirement in Table 6.4.2.1-1 [4].

# Proposal 3: Alternatively, in the case that increased MPR is defined for multi-antenna transmission, account for antenna correlation not observed in conductive measurements by increasing the conductive EVM measurement by

# EVMi'=EVM∙f∙2 ∆MPR2 ,

# where f is in the interval (0, 1] and apply this increased EVM against the EVM requirement in Table 6.4.2.1-1 [4].

# References

- RP-220987, “New WID on Further RF requirements enhancement for NR frequency range 1 (FR1),” Huawei, HiSilicon, 3GPP TSG-RAN Meeting RAN#95-e.

- R4-2217725, “WF on FR1 4Tx UE RF requirements,” Vivo, 3GPP TSG-RAN WG4 Meeting #104-bis-e.

- R4-2216879, “EVM Definition for 4x4 UL MIMO,” Lenovo, 3GPP TSG-RAN WG4 Meeting #104-bis-e.

- TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.

- R4-2011520, “On the Transmit EVM Requirement for UL MIMO Transmission,” Lenovo, Motorola Mobility, RAN4#96-e.

- 3GPP TR38.884 Study on enhanced test methods for FR2 NR UEs (Release 17), v1.1.0.

- R4-2107369, “On the EVM Definition for Transmit Diversity and Single Layer Transmission,’ Lenovo, Motorola Mobility, RAN4#98bis-e.

- R4-2214067, “EVM for Transmit Diversity with 4 Tx,” Lenovo, RAN4#104-e.

# Appendix: Maximum Per-Layer Noise Variance with Correlated Transmitter Noise

For 4x4 UL MIMO, the variance of the transmitter noise at the output of the zero-forcing receiver is given by

Ev vH=WHEn nHW

=WH Σ W ,

where

Σ=En nH=σ12⋯ε1,N⋮⋱⋮εN,1⋯σN2

and εi,j=Eni nj*.

The noise variance for the i-th layer is given by

WH Σ Wi,i=j=1Nk=1NWj,i*Σj,kWk,i .

Expanding and using the fact that εi,j=εi,j*, this gives

WH Σ Wi,i=j=1Nk=1NWj,i*Wk,i εj,k

=j=1Nk=1j-1Wj,i*Wk,i εj,k+Wk,i*Wj,iεk,j +j=1NWj,i2εj,j

=j=1Nk=1j-12ReWj,i*Wk,i εj,k+j=1NWj,i2σj2 ,

≤j=1Nk=1j-12 Wj,i*Wk,iεj,k+j=1NWj,i2σj2 .

Let ρi,j= εi,jσiσj and note that ρi,j≤1, so that

WH Σ Wi,i≤j=1Nk=1j-12 Wj,iWk,iρj,kσjσk+j=1NWj,i2σj2

≤j=1Nk=1j-12 Wj,iWk,iσjσk+j=1NWj,i2σj2

=j=1N Wj,iσi2.