3GPP TGS-RAN WG4 Meeting #105	     R4-2220026
Toulouse, France, November 14 – 18, 2022

Agenda item:	6.3.1

Source:	Intel Corporation

Title:	System parameters maintenance for FR2-2

Document for:	Discussion

# 1	Introduction

In this paper we address CA bandwidth classes as part of the system parameters maintenance of FR2-2.

# 2	Discussion

The core part of the NR extension to 71GHz WI, including both single carrier operation and intra-band contiguous DL CA, was concluded in RAN #97e [1]. During RAN4 #104e, the following system parameter aspects were agreed to be further discussed in the maintenance stage of the work item [2]:

  - NR CA bandwidth class

Agreement:

- No new bandwidth classes are specified for n*100 MHz

- The discussion on the new channel bandwidth classes for FR2-2 is allowed in the maintenance of this WI

  - Configurations for intra-band contiguous CA

Agreement:

- This is related to NR CA bandwidth classes and therefore can also be discussed in maintenance stage

CA bandwidth class

New CA bandwidth classes for FR2-2 have been discussed in past RAN4 meetings [3-5]. Considering the approved channel bandwidths for FR2-2 [6] and the agreement to address bandwidth classes for FR2-2 in the maintenance of this WI [2], we propose introducing new classes within FBG1 (4*400 MHz and 5*400 MHz) to extend support to 2000 MHz. Details on the classes are found in the proposal below. The content of NOTE 4 has been updated based on feedback received in RAN4 #104Bis-e [5].

Proposal 1: Introduce the new CA bandwidth classes captured in the table below for FR2-2

| NR CA BW class | Aggregated channel bandwidth | # cont. CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz for FR2-1 (this note is not relevant for UE capability parsing by the network). NOTE 4: This bandwidth class is applicable only for operating bands within FR2-2. |  |  |  |

# 3	Conclusions

In this contribution we discussed CA bandwidth classes for FR2-2 and made the following proposal:

Proposal 1: Introduce the new CA bandwidth classes captured in the table below for FR2-2

| NR CA BW class | Aggregated channel bandwidth | # cont. CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz for FR2-1 (this note is not relevant for UE capability parsing by the network). NOTE 4: This bandwidth class is applicable only for operating bands within FR2-2. |  |  |  |

# 4	References

- RP-222655, “SR for Extending current NR operation to 71GHz,” Qualcomm Inc., RAN #97e, September 2022

- R4-2214422, “WF on system parameters for FR2-2,” Intel Corporation, RAN4 #104e, August 2022

- R4-2212845, “Contiguous DL CA system parameters for FR2-2,” Nokia, Nokia Shanghai Bell, RAN4 #104e, August 2022

- R4-2214228, “Email discussion summary for [104-e][110] NR_ext_to_71GHz_Part_1,” Moderator (Intel), RAN4 #104e, August 2022

- R4-2216940, “Email discussion summary for [104-bis-e][102] NR_ext_to_71GHz,” Moderator (Qualcomm), RAN4 #104Bis-e, October 2022

- 3GPP TS 38.101-2, v 17.7.0, September 2022