3GPP TSG-RAN WG4 Meeting # 105												R4-2220071

Toulouse, France, November 14th – November 18th, 2022

Agenda item:			8.19.5

Source:	Moderator (Ericsson)

Title:	Topic summary for [105][225] FS_NR_pos_enh2_RRM

Document for:	Information

# Introduction

The document contains discussion related to the positioning measurement requirements. The document contains the following 2 main topics:

•	Topic #1: PRS/SRS bandwidth aggregation for intra-band carriers (AI: 8.19.4.1)

•	Topic #2: NR carrier phase measurements (AI: 8.19.4.2)

# Topic #1: PRS/SRS Bandwidth Aggregation for Intra-band Carriers

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2218441 | CATT | Proposal 1: PRS resources in different PFLs to be aggregated for MC positioning measurements can be transmitted by the co-located TRPs. Proposal 2: PRS aggregation should not impact the CA/DC operation for communication. Proposal 3: The capability for the number of carriers in MC positioning measurement is a separate capability with the supported number of carriers in CA/DC communication. |
| R4-2218517 | Qualcomm Incorporated | Proposal 1: From RRM perspective, RAN4 considers that it is feasible to define requirements for measurements obtained with PRS/SRS bandwidth aggregation under the following assumptions: - For PRS bandwidth aggregation, a common numerology is required across all intra-band contiguous PFLs to be aggregated. - For SRS bandwidth aggregation, a common numerology is required across all intra-band contiguous SRS resources to be aggregated. - PRS resources from different PFLs to be aggregated are transmitted by the same TRP and associated with a common Antenna Reference Point (ARP) - PRS resources from different PFLs to be aggregated can have different bandwidths (i.e. different number of RBs) - SRS resources in different carriers to be aggregated can have different bandwidths (i.e. different number of RBs) - PRS resources from different PFLs to be aggregated are transmitted in the same slot and in the same symbols - SRS resources in different carriers to be aggregated are transmitted in the same slot and in the same symbols Proposal 2: If support for PRS/SRS bandwidth aggregation is specified in Rel-18, RAN4 will define corresponding requirements for DL-TDOA and multi-RTT. Proposal 3: If support for PRS/SRS bandwidth aggregation is specified in Rel-18, RAN4 understands that requirements for PRS/SRS bandwidth aggregation will be specified in the WI phase subject to new UE processing capabilities defined by RAN1 and including limits on concurrence between multi-carrier/layer processing for positioning and carrier aggregation (for communications). |
| R4-2218997 | OPPO | Proposal-1: Deprioritize PRS aggregation transmitted by the co-located TRPs. Proposal-2: For PRS measurement within gap, no additional impact on the existing CA/DC operation for communication is identified. FFS PRS measurement outside gap. |
| R4-2219186 | ZTE Corporation | - Proposal 1: For time difference between PRS/SRS CCs in intra-band contiguous CA, we do not need to use the existing MRTD value since the 3μs is too large to improve the positioning accuracy. - Proposal 2: For time difference between PRS/SRS CCs in intra-band contiguous CA, we need to wait for the concrete value from RF session. |
| R4-2219467 | Ericsson | Observation 1: No feasibility issue has been identified during the SI phase for PRS/SRS bandwidth aggregation in RRC_INACTIVE. Proposal 1: PRS resources in different carriers/PFLs from different, but co-located, TRPs can be aggregated for MC positioning measurements. Proposal 2: MC positioning measurements should not impact the ongoing CA/DC operation for communication. Proposal 3: Evaluate MCPC and its impact on RRM when MC positioning measurement is done within MG during WI phase. Proposal 4: Evaluate MCPC and its impact on RRM when MC positioning measurement is done outside of the MG during WI phase. Proposal 5: Evaluate how to extend Rel-16/Rel-17 gapless PRS measurement restrictions to MC positioning measurements during WI phase. Proposal 6: Based on the evaluations done and the observations made during the SI phase, bandwidth aggregation for positioning is feasible. |
| R4-2219552 | Huawei, HiSilicon | Proposal 1: RAN4 to focus on same TRP and de-prioritize collocated TRPs for PRS/SRS CA. Proposal 2: Discuss the impact of MC positioning measurement on the CA/DC for communication when both are configured in parallel during the WI phase. Proposal 3: Consider the following Text Proposals as inputs from RRM perspective to TR 38.859. |
| R4-2219771 | Nokia, Nokia Shanghai Bell | Proposal 1: In the feasibility study consider only the case of transmission of PRS resources in different PFLs by the same ARP of a TRP. Proposal 2: For the feasibility study, assume no concurrent CA/DC operation. Proposal 3: For the feasibility study, assume MG based PRS measurements. Proposal 4: Whether the existing Rel-16/Rel-17 PRS measurement restrictions for PRS measurement can be extended to MC positioning measurements should be discussed in the WI phase. Proposal 5: Impact of switching CCs, such as introducing a switching delay, is investigated in the WI phase. Proposal 6: MC positioning measurements can be performed only on the activated CC’s configured for positioning which is investigated in the WI phase. Proposal 7: RRM impacts for specifying impairments margin including group delay calibration error, phase coherency errors, and phase center offsets, based on RF session outcome, are investigated in the WI phase. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Sub-topic 1-1: Conditions/assumptions for PRS/SRS bandwidth aggregation

Sub-topic description:

- The WF in R4-2217257 approved at RAN4#104bis-e contain agreements on several conditions under which the PRS/SRS bandwidth aggregation is feasible.

Open issues and candidate options before f2f meeting:

- The WF in R4-2217257 approved at RAN4#104bis-e contains the following open issue related to co-location of carriers for the PRS/SRS bandwidth aggregation:

- PRS resources in different PFLs to be aggregated for MC positioning measurements, shall be transmitted by the same TRP or FFS whether can also by transmitted by the co-located TRPs.

  - PRS resources to be aggregated from different PFLs should be associated with a common Antenna Reference Point (ARP) or

  - If PRS resources in different PFLs are transmitted from different antennas, then the antennas shall be physical close to each other.

Issue 1-1-1:  Numerology across PFLs for PRS and SRS

In the WF in R4-2217257 the following was agreed at RAN4#104bis-e:

- For PRS bandwidth aggregation, a common numerology is required across all intra-band contiguous PFLs to be aggregated.

- Proposals

  - Option 1: QC

    - For PRS bandwidth aggregation, a common numerology is required across all intra-band contiguous PFLs to be aggregated.

    - For SRS bandwidth aggregation, a common numerology is required across all intra-band contiguous SRS resources to be aggregated.

- Recommended WF

  - Further discuss the proposal.

Issue 1-1-2:  PRS and SRS BWs of PFLs

In the WF in R4-2217257 the following was agreed at RAN4#104bis-e:

- PRS resources to be aggregated for MC positioning measurements from different PFLs can have different bandwidths (i.e. different number of PRS RBs).

- Proposals

  - Option 1: QC

    - PRS resources from different PFLs to be aggregated can have different bandwidths (i.e. different number of RBs)

    - SRS resources in different carriers to be aggregated can have different bandwidths (i.e. different number of RBs)

- Recommended WF

  - Further discuss the proposal.

Issue 1-1-3:  Proximity of PFLs in frequency and time domains

In the WF in R4-2217257 the following was agreed at RAN4#104bis-e:

- To study the RRM impact, prioritize the aggregation of PRS or SRS transmitted in the same slot and in the same symbols from the intra-band contiguous PFLs.

- Proposals

  - Option 1: QC

    - PRS resources from different PFLs to be aggregated are transmitted in the same slot and in the same symbols

    - SRS resources in different carriers to be aggregated are transmitted in the same slot and in the same symbols

- Recommended WF

  - Further discuss the proposal.

Issue 1-1-4:  Co-location of PFLs

- Proposals

  - Option 1: QC, OPPO, HW, Nokia

    - RAN4 to focus on same TRP and de-prioritize collocated TRPs for PRS/SRS CA.

    - Option 1A: QC, Nokia

      - PRS resources from different PFLs to be aggregated are transmitted by the same TRP and associated with a common Antenna Reference Point (ARP)

  - Option 2: CATT, E///

    - PRS resources in different PFLs to be aggregated for MC positioning measurements can be transmitted by the co-located TRPs.

- Recommended WF

  - Further discuss the proposal.

Issue 1-1-5:  Activation/deactivation of PFLs

- Proposals

  - Option 1: Nokia

    - MC positioning measurements can be performed only on the activated CC’s configured for positioning which is investigated in the WI phase.

- Recommended WF

  - Further discuss the proposal.

### Sub-topic 1-2: Impact of CA/DC on PRS/SRS bandwidth aggregation

Sub-topic description

- In the WF in R4-2217257 the following was agreed at RAN4#104bis-e:

  - Multicarrier positioning capability (MCPC) (e.g. number of intra-band contiguous PFLs) is to be defined during the WI.

Open issues and candidate options before f2f meeting:

- The WF in R4-2217257 approved at RAN4#104bis-e contains the following open issue related to impact of CA/DC on the PRS/SRS bandwidth aggregation:

  - FFS: Impact of MC positioning measurement on the carrier aggregation/dual connectivity (CA/DC) for communication when both are configured in parallel:

Issue 1-2-1: Impact on CA/DC operation

- Proposals

  - Option 1: CATT, E///

    - MC positioning measurements should not impact the ongoing CA/DC operation for communication.

  - Option 2: HW

    - Discuss the impact of MC positioning measurement on the CA/DC for communication when both are configured in parallel during the WI phase.

  - Option 3: QC

    - If support for PRS/SRS bandwidth aggregation is specified in Rel-18, RAN4 understands that requirements for PRS/SRS bandwidth aggregation will be specified in the WI phase subject to new UE processing capabilities defined by RAN1 and including limits on concurrence between multi-carrier/layer processing for positioning and carrier aggregation (for communications).

  - Option 4: OPPO

    - For PRS measurement within gap, no additional impact on the existing CA/DC operation for communication is identified. FFS PRS measurement outside gap.

  - Option 5: Nokia

    - For the feasibility study, assume no concurrent CA/DC operation.

- Recommended WF

  - Further discuss the proposals.

### Sub-topic 1-3: PRS/SRS bandwidth aggregation capabilities

Sub-topic description

- UE capability related to PRS/SRS bandwidth aggregation

Open issues and candidate options before f2f meeting:

- No previous open issue.

Issue 1-3-1: Relation with CA/DC capabilities

- Proposals

  - Option 1: CATT

    - The capability for the number of carriers in MC positioning measurement is a separate capability with the supported number of carriers in CA/DC communication.

- Recommended WF

  - Further discuss the proposal.

### Sub-topic 1-4: Applicable positioning methods using PRS/SRS bandwidth aggregation

Sub-topic description

- Positioning methods for which requirements with PRS/SRS bandwidth aggregation are to be defined.

Open issues and candidate options before f2f meeting:

- No previous open issue.

Issue 1-4-1: Supported positioning methods

- Proposals

  - Option 1: QC

    - If support for PRS/SRS bandwidth aggregation is specified in Rel-18, RAN4 will define corresponding requirements for DL-TDOA and multi-RTT.

- Recommended WF

  - Further discuss the proposal.

### Sub-topic 1-5: RRM requirements for PRS/SRS bandwidth aggregation

Sub-topic description

- Positioning methods for which requirements with PRS/SRS bandwidth aggregation are to be defined.

Open issues and candidate options before f2f meeting:

- No previous open issue.

Issue 1-5-1: MC positioning measurement within MG

- Proposals

  - Option 1: E///

    - Evaluate MCPC and its impact on RRM when MC positioning measurement is done within MG during WI phase.

  - Option 2: Nokia

    - For the feasibility study, assume MG based PRS measurements.

- Recommended WF

  - Further discuss the proposal.

Issue 1-5-2: MC positioning measurement outside MG

- Proposals

  - Option 1: E///

    - Evaluate MCPC and its impact on RRM when MC positioning measurement is done outside of the MG during WI phase.

    - Evaluate how to extend Rel-16/Rel-17 gapless PRS measurement restrictions to MC positioning measurements during WI phase.

- Recommended WF

  - Further discuss the proposal.

Issue 1-5-3: PRS measurement restrictions for gapless MC positioning measurement

- Proposals

  - Option 1: E///, Nokia

    - Evaluate how to extend Rel-16/Rel-17 gapless PRS measurement restrictions to MC positioning measurements during WI phase.

- Recommended WF

  - Further discuss the proposal.

Issue 1-5-4: Impact of switching delay of PFL

- Proposals

  - Option 1: Nokia

    - Impact of switching CCs, such as introducing a switching delay, is investigated in the WI phase.

- Recommended WF

  - Further discuss the proposal.

Issue 1-5-5: Impairments and margins

- Proposals

  - Option 1: Nokia

    - RRM impacts for specifying impairments margin including group delay calibration error, phase coherency errors, and phase center offsets, based on RF session outcome, are investigated in the WI phase.

- Recommended WF

  - Further discuss the proposal.

### Sub-topic 1-6: MRTD and Time offset

Sub-topic description

- In the WF in R4-2217257 the following was agreed at RAN4#104bis-e:

  - RRM impact of possible timing error between PRS/SRS from different PFLs in single RF chain (Tx/Rx) architecture if defined by RF session will be considered in MC positioning requirements during the WI.

  - No further discussion needed on impact of existing MRTD/MTTD on MC positioning measurement.

  - RRM impact of possible frequency offset between PRS/SRS from different PFLs in single RF chain (Tx/Rx) architecture if defined by RF session will be considered in MC positioning requirements during the WI.

Open issues and candidate options before f2f meeting:

  - No open issues

Issue 1-6-1: MRTD

- Proposals

  - Option 1: ZTE

    - For time difference between PRS/SRS CCs in intra-band contiguous CA, we do not need to use the existing MRTD value since the 3μs is too large to improve the positioning accuracy.

- Recommended WF

  - According to the past agreement no further discussion is needed on MRTD.

Issue 1-6-2: Time offset between PFLs

- Proposals

  - Option 1: ZTE

    - For time difference between PRS/SRS CCs in intra-band contiguous CA, we need to wait for the concrete value from RF session.

- Recommended WF

  - Already captured by previous agreement.

### Sub-topic 1-7: Capturing overall agreements

Issue 1-7-1: Text proposal to TR 38.859:

- Proposals

  - Option 1: HW

    - Consider the following Text Proposals as inputs from RRM perspective to TR 38.859.

      - TP in R4-2219552

- Recommended WF

  - Further discuss the text proposal in R4-2219552.

Issue 1-7-2: Overall conclusion:

- Proposals

  - Option 1: E///

    - Based on the evaluations done and the observations made during the SI phase, bandwidth aggregation for positioning is feasible.

- Recommended WF

  - Further discuss the proposal.

# Topic #2: NR Carrier Phase Measurements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2219468 | Ericsson | Observation 1: Framework for carrier phase measurement is yet to be concluded by RAN1. Proposal 1: Based on observation 1, RAN4 should uphold its previous agreement on RRM impact of carrier phase measurement for positioning. |
| R4-2219553 | Huawei, HiSilicon | Proposal 1: RAN4 to conclude that no feasibility issue is identified from RRM perspective for carrier phase measurement. Proposal 2: Single carrier PRS/SRS transmission is prioritized for the RRM impacts of carrier phase measurement. Proposal 3: RAN4 to conclude that no feasibility issue is identified from RRM perspective for carrier phase measurement in RRC_INACTIVE. Proposal 4: Consider the following Text Proposals as inputs from RRM perspective to TR 38.859. |
| R4-2219772 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to consider RRM impacts for NR carrier phase measurements for single CC/PFL scenario during feasibility study and for multiple CCs/PFLs scenario during the WI phase. Proposal 2: Carrier phase measurements for multiple CCs/PFLs are assumed to be performed in TDM manner as starting point. Proposal 3: RAN4 to investigate support of NR carrier phase measurements for RRC_CONNECTED and RRC_INACTIVE states and to analyze feasibility on high level for RRC_IDLE state in the current study. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions..

### Sub-topic 2-1: RAN4 study on carrier phase measurements

Sub-topic description:

- In the WF in R4-2217257 the following was agreed at RAN4#104bis-e:

  - RAN4 wait for conclusive RAN1 outcome on carrier phase measurements before starting RAN4 feasibility study on carrier phase measurement aspects

Open issues and candidate options before f2f meeting:

Issue 2-1-1: When to initiate RAN4 study carrier phase measurements?

- Proposals

  - Option 1: E///

    - Based on observation 1, RAN4 should uphold its previous agreement on RRM impact of carrier phase measurement for positioning.

  - Option 2: HW

    - RAN4 to conclude that no feasibility issue is identified from RRM perspective for carrier phase measurement.

- Recommended WF

  - Further discuss the proposal

### Sub-topic 2-2: Conditions/assumptions for carrier phase measurements

Sub-topic description

- In the WF in R4-2217257 the following was agreed at RAN4#104bis-e:

  - If carrier phase measurement is considered feasible based on RAN1 outcome, then further discuss if single carrier PRS/SRS transmission can be prioritized for the RRM impact study.

Open issues and candidate options before f2f meeting:

- Discussion on number of carriers

Issue 2-2-1: Number of carriers

- Proposals

  - Option 1: Nokia

    - RAN4 to consider RRM impacts for NR carrier phase measurements for single CC/PFL scenario during feasibility study and for multiple CCs/PFLs scenario during the WI phase.

  - Option 2: HW

    - Single carrier PRS/SRS transmission is prioritized for the RRM impacts of carrier phase measurement.

- Recommended WF

  - Further discuss the proposal

Issue 2-2-2: Handling multiple carriers

- Proposals

  - Option 1: Nokia

    - Carrier phase measurements for multiple CCs/PFLs are assumed to be performed in TDM manner as starting point.

- Recommended WF

  - Further discuss the proposal

### Sub-topic 2-3: Applicable RRC state for carrier phase measurements

Sub-topic description

- In the WF in R4-2217257 the following was agreed at RAN4#104bis-e:

  - If carrier phase measurement is considered feasible based on RAN1 outcome, then further investigate if carrier phase measurement is also supported in RRC_INACTIVE.

Open issues and candidate options before f2f meeting:

- Discussion on applicability in RRC_INACTIVE

Issue 2-3-1: Carrier phase measurements in RRC_CONNECTED and RRC_INACTIVE

- Proposals

  - Option 1: Nokia

    - RAN4 to investigate support of NR carrier phase measurements for RRC_CONNECTED and RRC_INACTIVE states and to analyze feasibility on high level for RRC_IDLE state in the current study.

  - Option 2: HW

    - RAN4 to conclude that no feasibility issue is identified from RRM perspective for carrier phase measurement in RRC_INACTIVE.

- Recommended WF

  - Further discuss the proposals

### Sub-topic 2-4: Capturing overall agreements

Issue 2-4-1: Text proposal to TR 38.859:

- Proposals

  - Option 1: HW

    - Consider the following Text Proposals as inputs from RRM perspective to TR 38.859.

      - TP in R4-2219553

- Recommended WF

  - Further discuss the text proposal in R4-2219553.