# 3GPP TSG-RAN WG4 Meeting # 105                                 R4-2220223

# Toulouse, France, November 14 – November 18, 2022

# Source: 	ZTE Corporation

# Title: 	Discussion on BS EMC enhancement R18

# Agenda item:	8.16.2

# Document for:	Approval

- Introduction

In RAN#95-e meeting, the new WI: BS/UE EMC Enhancements for NR and LTE was approved and the study phase of it will start in Q4 of 2022. In this document, we give some discussions on MSR BS test configuration simplification works.

- Discussions

2.1 Test configuration from TS37.113 and TS37.141

The first issue from WF[1] is to investigate that using existing TCs or creating a new TC, which is shown below.

- Investigate using existing TCs or creating a new TC

- Option 1: using existing TCs in TS 37.141 & TS 37.145.

- Option 2: creating new TC dedicated for EMC test, which is decoupled from RF conformance testing.

Firstly, we give a review of the test configuration tables below from TS 37.113.

Table 4.5-1: Test configurations for single-band Multi-RAT capable MSR BS (CS3-6)

| Capability Set | UTRA + E-UTRA. NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) (CS 3) |  |  | GSM+ UTRA (CS 4) | GSM + E-UTRA, NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) (CS 5) | GSM + UTRA + E-UTRA, (CS 6) | GSM+UTRA/ E-UTRA, UTRA+ E-UTRA, NB-IoT in-band, NB-IoT guard band (CS7) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC2 | BC2 | BC2 | BC2 |
| Emission tests | C: TC3a CNC: NTC3a C/NC: TC3a and NTC3a NI: TC16 NG: TC19 | C: TC3a CNC: NTC3a C/NC: TC3a and NTC3a NI: TC16 NG: TC19 | C: TC3b | C: TC4a CNC: NTC4a C/NC: TC4a and NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b and NTC4b NI: TC16 NG: TC19 | C: TC4c CNC: NTC4c C/NC: TC4c and NTC4c NI: TC16 NG: TC19 | C: (TC4a, TC3a) (Note 4), TC4b CNC: (NTC4a, NTC3a) (Note 4), NTC4b C/NC: (TC4a, NTC4a,TC3a, NTC3a) (Note 4), TC4b,NTC4b NI: TC15,(TC16) (Note 4) NG: TC18, (TC19) (Note 4) |
| Immunity tests | C: TC3a CNC: NTC3a C/NC: TC3a, NTC3a NI: TC16 NG: TC19 | C: TC3a CNC: NTC3a C/NC: TC3a, NTC3a NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC5a CNC: NTC5a C/NC: TC5a, NTC5a | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 | C: TC5b CNC: NTC5c, C/NC: TC5b, NTC5c | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 |
| NOTE 1: Void NOTE 2: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NI" shall not be used for testing. NOTE 3: The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NG" shall not be used for testing. NOTE 4: For Band 3, the test configuration is only applicable if UTRA is declared to be supported in Band 3. |  |  |  |  |  |  |  |

Table 4.5-1a: Test configurations for single-band Multi-RAT capable MSR BS (CS9-13)

| Capability Set | GSM+NB-IoT standalone (CS 9) | UTRA + NB-IoT standalone (CS 10) |  | E-UTRA + NB-IoT standalone (CS 11) |  |  | GSM+UTRA+NB-IoT standalone (CS 12) | GSM+ E-UTRA+ NB-IoT standalone (CS 13) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC2 | BC1 | BC2 | BC1 | BC2 | BC3 | BC2 | BC2 |
| Emission tests | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Immunity tests | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |

Table 4.5-1b: Test configurations for single-band Multi-RAT capable MSR BS (CS14-17)

| Capability Set | UTRA + E-UTRA + NB-IoT standalone (CS 14) |  |  | GSM + UTRA + E-UTRA + NB-IoT standalone (CS 15) | NR + E-UTRA NB-IoT in-band NB-IoT guard band (CS 16) |  | NR + NB-IoT standalone + E-UTRA NB-IoT in-band NB-IoT guard band (CS 17) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC2 | BC1 and BC2 | BC3 | BC1 and BC2 | BC3 |
| Emission tests | TC14 | TC14 | TC14 | TC13 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 |
| Immunity tests | TC14 | TC14 | TC14 | TC13 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 |

Table 4.5-2: Test configurations for single-band Single-RAT multi-carrier capable BS

| Capability Set | UTRA (MC) capable BS (CS1) |  |  | E-UTRA (MC) capable BS (CS2) NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) |  |  | NB-IoT (MC) capable BS (CS8) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC1 | BC2 | BC3 | BC1 | BC2 |
| Emission tests | C: TC1a CNC: NTC1a C/NC: TC1a and NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a and NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2 and NTC2 | C: TC2 CNC: NTC2 C/NC: TC2 and NTC2 | C: TC2 CNC: NTC2 C/NC: TC2 and NTC2 | TC8 | TC8 |
| Immunity tests | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | TC8 | TC8 |
| NOTE 1: Void NOTE 2: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NI" shall not be used for testing. NOTE 3: The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NG" shall not be used for testing. |  |  |  |  |  |  |  |  |

Table 4.5-3: Test configurations for multi-band capable BS (CS1-CS7 and CS16)

| Capability Set | Multi-band testing |  |
| --- | --- | --- |
| BS test case | BC1/BC2 | BC3 |
| Emission tests | TC7b | TC7b |
| Immunity tests (Note) | TC7b | TC7b |

From the table above we can find the test configurations for each CSs and the detailed information for each TC is illustrated in TS37.141 clause 4.8.

Observation 1: For most of CSs, all of the RATs in it are used with corresponding TC.

CS15 is one exception of observation. CS15 and CS13 share the same test configuration TC13. This is shown in Table 1 below.

Table 1: Test configuration for CS13 and CS15

| GSM+ E-UTRA+ NB-IoT standalone (CS 13) | GSM + UTRA + E-UTRA + NB-IoT standalone (CS 15) |
| --- | --- |
| BC2 | BC2 |
| TC13 | TC13 |
| TC13 | TC13 |

Observation 2: The test configuration for both CS15 (GSM+UTRA+E-UTRA+NB-IoT standalone) and CS13(GSM+E-UTRA+NB-IoT standalone) is TC13.

The detailed configuration of TC13 is described in TS37.141 clause 4.8.13.1, which is shown below.

#### 4.8.13.1	TC13 generation

TC13 is constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum Base Station RF Bandwidth.

-	Place a NB-IoT carrier at the upper edge and a GSM carrier at the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	Place two 5 MHz E-UTRA carriers in the middle of the Base Station RF Bandwidth. If the BS does not support 5 MHz channel BW use the narrowest supported BW, if only one carrier is supported or two carriers do not fit place only one carrier.

-	For transmitter tests, alternately add NB-IoT carriers at the upper edge and GSM carriers at the lower edge using 600 kHz spacing until the Base Station RF Bandwidth is filled or the total number of supported carriers is reached.

-	For transmitter tests, add additional E-UTRA carriers of the same bandwidth as the already allocated E-UTRA carriers in the middle if possible.

As we can see from the instruction, there is no UTRA carriers in TC13 generation.

Observation 3: TC13 that sets up NB-IoT, GSM and E-UTRA carriers are used to test CS15 (GSM+UTRA+E-UTRA+NB-IoT standalone). The UTRA carrier is not required in CS15 test configuration.

From the observation 1,2 and 3, current test configuration will include all of the RATs in the CSs except CS15. This makes the EMC test configuration complicated and repeated. However, those TC are mainly used for RF test, each RAT would have different behaviour in RF test especially at near field. But most of EMC tests are far field test, which would not have much influence on individual RAT behaviour. Therefore, one simplification method is to use worse RAT to instead other RAT in test configuration.

In our understanding, the first issue from WF[1] mainly depends on whether or not test all RATs in the CS. i.e. Set up all of the A, B and C RATs for CSx(A+B+C). There are two options:

Option 1: No, some RATs can be covered by other RATs with worse behaviour.

Option 2: Yes, then new TCs that include all RATs should be introduced.

For option 1, as discussed above we think it is feasible to use worse RATs cover other RATs. However if the agreement is option 2, we think it is no need to do the simplification work or rather introduce new TCs. Therefore option 1 is our preference.

Proposal 1: Whether or not test all RATs in the CS? i.e. Set up all of the A, B and C RATs for CSx (A+B+C).

(Recommended) option 1: No, some RATs can be covered by other RATs with worse behaviour.

Option 2: Yes, then new TCs that include all RATs should be introduced.

2.2 Analyzing Capability Set in TS37.113

In TS37.113 the RATs in CS1, CS4, CS6, CS10, CS12 are analyzed and listed in Table 2 below:

Table 2: RATs in CS1,4,6,10 and 12

| CS1 | CS4 | CS6 | CS10 | CS12 |
| --- | --- | --- | --- | --- |
| UTRA(MC) capable BS | GSM+UTRA | GSM+UTRA+E-UTRA | UTRA+NB-IoT standalone | GSM+UTRA+NB-IoT standalone |

Observation 4: There is no E-UTRA in CS1, CS4, CS10 and CS12, but they have UTRA.

Observation 5: There is no NB-IoT in CS4 and CS6.

For CS1, CS4, CS10 and CS12 BS EMC test, E-UTRA carries cannot be set up in test configuration, because E-UTRA is not included in the function of such BS. So as to CS4 and CS6 BS, NB-IoT should not be set during the EMC test. Therefore in MSR BS EMC test, the test configuration can not set up the RAT that are not belongs to the BS function.

Proposal 2: There is no need to use NB-IoT to cover GSM or E-UTRA to cover UTRA for the CSs that does not have NB-IoT or E-UTRA function, even if they are the worst case. The MSR BS can not set up the RATs that are outside of its function.

Proposal 3:The future TC simplification works can not set up the RAT carriers that are not exist in the MSR BS function.

- Conclusion

In this paper, we first discuss and analysis the test configuration in TS37.113 and TS37.141, then provide some suggestion for future simplification work. The conclusions are shown below:

Observation 1: For most of CSs, all of the RATs in it are used with corresponding TC.

Observation 2: The test configuration for both CS15 (GSM+UTRA+E-UTRA+NB-IoT standalone) and CS13(GSM+E-UTRA+NB-IoT standalone) is TC13.

Observation 3: TC13 that sets up NB-IoT, GSM and E-UTRA carriers are used to test CS15 (GSM+UTRA+E-UTRA+NB-IoT standalone), The UTRA carrier is not required in CS15 test configuration.

Observation 4: There is no E-UTRA in CS1, CS4, CS10 and CS12, but they have UTRA.

Observation 5: There is no NB-IoT in CS4 and CS6.

Proposal 1: Whether or not test all RATs in the CS? i.e. Set up all of the A, B and C RATs for CSx (A+B+C).

(Recommended) option 1: No, some RATs can be covered by other RATs with worse behaviour.

Option 2: Yes, then new TCs that include all RATs should be introduced.

Proposal 2: There is no need to use NB-IoT to cover GSM or E-UTRA to cover UTRA for the CSs that does not have NB-IoT or E-UTRA function, even if they are the worst case. The MSR BS can not set up the RATs that are outside of its function.

Proposal 3:The future TC simplification works can not set up the RAT carriers that are not exist in the MSR BS function.

- Reference

- R4-2217462 WF on principles for BS EMC enhancements

- TS37.113

- TS37.141