3GPP TSG-RAN WG4 #105	R4-2220224

November 14th ‒ 18th, 2022

Toulouse, FR

Agenda item:	8.23.5

Source: 	Qualcomm Incorporated

Title: 	Ad-hoc meeting minutes for NTN Ka band definition

Document for:	Discussion

# Introduction

This document contains meeting minutes and tentative agreements reached during an ad-hoc meeting for the NTN Ka band definition held on November 14, 2022.

# Discussion

The moderator summary lists the following topics for discussion.

Issue 1-1-1: Band definition principles

- Proposals

  - Option 1: to define separate bands for FSS and MSS to enable some terminal access/barring mechanism. (R4-2219378/P1)

  - Option 2: do not define separate bands for FSS and MSS

- Recommended WF

  - TBA

Discussion:

Ericsson:  RAN4 should not specify fixed VSAT terminals in FSS spectrum.  Should at least consider the MSS spectrum.  Other bands for ESIM can be further discussed.

Intel:  It is only FSS spectrum that is available for NTN.  Is there also MSS available?

Inmarsat:  If we consider ESIM, we must consider FSS.  MSS is outside the scope of the WID.

Ericsson:  Agree if we consider ESIM, then we should consider FSS.  But what about fixed VSAT?

Inmarsat:  Fixed VSAT is also FSS.

Ericsson:  Fixed VSAT is out of scope because there is no mobility.  RAN4 should only consider mobile operation.

ZTE:  Fixed VSAT and ESIM are in FSS.  Mobile Earth Station (MES) is used in MSS.  Having both bands can be acceptable to us.

Hughes:  In R16, we defined VSAT for both fixed and moving platform. Cannot separate VSAT and ESIM.  Last 500 MHz of the band (MSS portion) cannot be used for mobile.

Hughes:  ESIM is only for GSO, not NGSO.  So not having fixed VSAT means that we cannot have NGSO.

Hispasat:  Ka band is traditionally used for FSS.  VSAT have been used here.  Fixed VSAT and ESIM are both in scope.  Don’t know why we are discussing MSS.

Ericsson:  Fixed device in fixed spectrum is out of scope of RAN4.

Chair:  Is this restriction in RAN4’s terms of reference?

Stellar:  Everything is FSS in this band.

ZTE:  ESIM is just an ITU term.  Prefer moving VSAT since ESIM might not imply a small antenna.

ESA:  VSAT antenna size might not be small.  Just its aperture.

Chair:  FSS spectrum should be included in 3GPP band definition for the purpose of enabling ESIM/moving VSAT moveable NTN terminal with priority.  [Fixed VSAT in FSS allocation – Companies to check RAN4 ToR to see if there is a restriction on fixed device in fixed spectrum]

Issue 1-1-2: Number of Ka-bands to be defined

- Proposals

  - Option 1: Several NTN Ka-bands

    - To address the diversity of spectrum allocation to ESIM by national regulators, RAN4 should consider specifying several “NTN Ka-bands”. Each band would then be applicable to the corresponding regulator’s country (R4-2218489/P4, R4-2218488/P4)

  - Option 2: One NTN Ka-band

- Recommended WF

  - TBA

Discussion:

Chair:  Is Option 1 agreeable?  No dissenting views.

ZTE:  How do we define the bands in the absence of regulatory rules.  When do we start the work?

Nokia:  Same as ZTE.  We originally proposed one band to just start with one.  If we do multiple bands in parallel, we need to have regulatory rules and prioritization.

Hughes:  Can agree to option 1, but would like to simplify the applicability instead of on country-by-country basis.

Thales:  How about this?  “To address the diversity of spectrum allocation to ESIM by national regulators, RAN4 should consider specifying several “NTN Ka-bands”

Nokia:  Ok with this, but where should we start?

Chair:  Define several NTN Ka-bands to address the diversity of spectrum allocation to ESIM by national regulators, RAN4 should consider specifying several “NTN Ka-bands”

Open issue:  How do we start?  Should we have an upper limit on the number of Ka bands in Rel-18?

Issue 1-1-3: Potential NTN band ranges

- Proposals

  - Option 1: to define MSS band (DL: 19.7-20.2GHz and UL: 29.5-30GHz) for Ka-band firstly and further discuss how to define other bands(s); (R4-2219378/P2, R4-2219083/P1, R4-2218489/P2, R4-2218488/P3)

    - DL: 19.7-20.2GHz (n512)

    - UL: 29.5-30GHz (n512)

  - Option 2: (R4-2219076/P2 & P1)

    - DL:

      - Option a: 17.7-20.2 GHz (n512);

      - Option b: 17.7-20.2 GHz (n512, n511, n510);

    - UL:

      - Option a: 27.5-30.0 GHz (n512);

      - Option b: 27.5-30.0 GHz (n512), 28.35-30.0 GHz (n511), 27.5-28.35 GHz (n510)

Note:

    - For the Ka-band Downlink NTN band definition, define one NTN band covering the full harmonized Ka-band space-to-earth range of 17.7-20.2 GHz for Regions 1, 2 and 3.

    - For the Ka-band Uplink NTN band definition, consider the following options:

      - Option a: Define one NTN band covering the full harmonized Ka-band earth-to-space range of 27.5-30.0 GHz, with specific Network Signaling (NS) to address regional-specific requirements and restrictions.

      - Option b: Define three NTN bands for Ka-band Uplink.

        - One band covering the full harmonized Ka-band earth-to-space range of 27.5-30.0 GHz applicable for Region 1, Region 3 and Region 2 countries except the United States

        - Additional two bands for the US/FCC market, and countries deploying the same assignments:

          - one for the 27.5-28.35 GHz range,

          - one for the 28.35-30.0 GHz range.

Example of Option b

- Recommended WF

  - TBA

Discussion:

Inmarsat:  Cannot accept option 1 because MSS should not be referenced.

Hispasat:  Cannot accept option 1 because it focuses on the 500 MHz as a priority.  We prefer to focus on the whole band.

Ericsson:  Cannot agree on option 2 because we don’t know which regulations apply to which bands.  Option 1 allocation applies to Japan for example.

Verizon:  Can accept option 2, but would like to consider FCC regulatory requirements first.  We also need to consider the coex requirements to the existing TN 28 GHz.

Chair:  Option 2 with consideration of US/FCC regulations as a starting point for n511 [and CEPT for n512 as a starting point].

Could not agree to the above.

# Conclusion

Ad-hoc chair recommends a way forward on NTN Ka band definition led by Thales.

# Reference

- RP-2220138, “Topic summary for [105][312] NR_NTN_enh_Part1,” Thales