3GPP TSG-RAN WG4 Meeting #105	R4-2220242

Toulouse, France, 14 ‒ 18 Nov 2022

Title: 	WF for IoT over NTN SAN RF requirements

Agenda Item:	     9.5.9

Source: 	ZTE

Document for:	Approval

Agreement

Issue 1-3:  UE ACLR/ACS requirements

- Agreement: reuse the TN NB-IoT UE ACLR/ACS requirement for IoT over NTN UE

Issue 1-4:  SAN ACLR/ACS requirements

- Agreement: the NR NTN SAN ACLR/ACS requirement for IoT over NTN SAN

Issue 2-1 IoT level for NB-IoT over NTN

- Agreement:

- For GEO: IoT level as 20dBc

- Tentative agreement for LEO: IoT level as [31dBc], further discuss and refine the value not precluded in maintenance phase

Issue 2-2 IoT level for MTC over NTN

Agreement:

- LEO: Reuse IoT 15dB for MTC over NTN

Issue 2-3 In channel selectivity requirement for MTC over NTN

Agreement:

  - Stick to the previous agreement.

| In-channel selectivity | 7.8 | Not needed [Not needed as no requirement for NB-IoT SA in TS36.104.] | For GEO and LEO, to reuse ICS level as 12 dB for GEO, 24dB for LEO used for NR over NTN in Rel-17. Reuse the same FRC A1-4 and SNR for 1.4MHz LTE BW; Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO Reuse the implementation margin as 2dB; |
| --- | --- | --- | --- |