3GPP TSG-RAN WG4 Meeting # 105	R4-2220249

Toulouse, France, November 14 – November 18, 2022

Agenda item:			8.4.3

Source:	Huawei, HiSilicon

Title:	WF for the feasibility study on FR2 multi-band RIB

Document for:	Approval

Way forward

Issue 3-1: RF chain and general consideration

- Agreement:

  - Further study DPD aspect for multi-band supporting in FR2.

Issue 3-2: Frequency ranges/groups

- Agreement:

  - Rel-18 Study Item focus on the multi-band RIB within the same frequency group meanwhile the study on multi-band RIB across different groups not precluded.

Issue 3-4: Antenna array

- Agreement:

  - Further study the impact of supporting multi-bands with separate antenna array used for each band

  - RAN4 have common observation as following:

    - MB BS supporting about 26G+40G has possible solution (e.g. stacked patches) from antenna implementation perspective. The performance impact needs to be further studied.

    - It is possible to implement diplexer below multi-band mmWave array antenna elements to divide each signal of different frequency group’s bands.

Issue 3-5: Phase shifters

- Agreement:

  - RAN4 reached below common observation

    - Using single band phase shifters prior to a multi-band PA to steer each band separately is a possible approach and should not differentiate the multi-band requirements compared to other possible approaches of using single band or multi-band phase shifters.

Work split for TP drafting

TPs on feasibility study for future meetings, the following work split is proposed.

| Clauses | Company |
| --- | --- |
| 5.1 General | Ericsson |
| 5.2.1 RF Front end | Huawei |
| 5.2.2 DPD | Nokia |
| 5.2.3 Phase shifter | ZTE |
| 5.3 Antenna array | Huawei |
| 5.4 Other | Ericsson |