3GPP TSG-RAN WG4 Meeting #105											R4-2220321

Toulouse, France, November 14 – November 18, 2022

Title: 	WF on Rel-17 NR extension to 71GHz RRM Performance requirements

Agenda Item:	6.3.7

Source: 	Qualcomm

Document for:	Approval

# Topic #1: General

Issue 1-1-1: CCA modelling in test cases

Agreement:

- For DL CCA model in FR2-2 test cases:

  - TE generates a uniform random variable p from the range [0,1].

  - If p > PCCA_DL,

    - TE picks one SSB/SMTC occasion out of a group of each 12 consecutive SSB/SMTC occasions based on a fixed pattern, where one SSB/SMTC occasion is equivalent to one SSB burst Set.

    - TE models CCA failure in this SSB/SMTC occasion. Note that other 11 SSB/SMTC occasions shall be transmitted by the TE

    - Whole SSB occasion group is considered as unavailable to the UE

  - If p < PCCA_DL,

    - TE transmit 12 consecutive SSB/SMTC occasions

    - Whole SSB occasion group is considered as available to the UE

Issue 1-1-2: PCCA_DL

Agreement:

- PCCA_DL is equal to [93] %

Issue 1-1-3: Rate of correct tests observed during repeated tests

Agreement:

- The rate of correct tests observed during repeated tests shall be at least 90%

# Topic #3: Test-cases

Tentative agreement:

- All test-cases with agreed/endorsed draft CRs shall not be dropped

- All low priority test cases with draft CRs not agreed shall be dropped

- All non-CCA related test cases will be added to corresponding sections in Clause A.7

- All CCA related test cases will be added to Clause A.15

  - Existing non-CCA test cases under Clause A.15 shall be modified to CCA test-cases

## 2.1	Test cases for RRM performance requirements without CCA

| Group of requirements | Test cases | Sub-test | 120 kHz SCS | 480/960 kHz SCS | Company | Status |
| --- | --- | --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 | intra-frequency | Yes | Yes | Ericsson | Submit to A.7 |
|  |  | inter-frequency | Yes | yes | Ericsson | Submit to A.7 |
| HO (delay and interruptions) | FR2-2->FR2-2 | intra-frequency, unknown target cell | Yes | Yes | Huawei | Available |
|  |  | inter-frequency, unknown target cell | Yes | Low priority | Huawei | Available |
|  | FR1->FR2-2 | unknown target cell | Yes | Yes | Huawei | Available |
| RRC Re-establishment | FR2-2->FR2-2 | intra frequency | Yes | Low priority | CATT | Endorsed |
|  |  | inter frequency | Yes | Yes | CATT | Endorsed |
|  |  | intra frequency without serving cell timing | Yes | Yes | CATT | Endorsed |
| Random access | 4-step RACH | Contention based | Reuse FR2-1 | Yes | Nokia | Endorsed |
|  |  | Non-contention based | Reuse FR2-1 | Yes | Nokia | Endorsed |
| Timing | UE Transmit Timing |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes | Huawei | Available |
|  | Timing advance adjustment accuracy |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes | Huawei | Available |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2->FR2-2 | Reuse FR2-1 | Yes | Huawei | Available |
|  | RRC-based active BWP switch |  | Reuse FR2-1 | Yes | Huawei | Available |
|  | RRC-based Active BWP Switch on multiple CCs |  | Reuse FR2-1 | Yes | Huawei | Available |
| Radio link monitoring | FR2-2 PCell, SSB-based | Out-of-sync, in non-DRX | Yes | Yes | Ericsson | Endorsed |
|  |  | In-sync, in non-DRX mode | Yes | Low priority | Ericsson | Endorsed |
|  |  | Out-of-sync, in DRX mode | Yes | Low priority | Ericsson | Endorsed |
|  |  | In-sync, in DRX mode | Yes | Yes | Ericsson | Endorsed |
|  | FR2-2 PCell, CSI-RS-based | Out-of-sync, in non-DRX | Yes | Low priority | Ericsson | Endorsed |
|  |  | In-sync, in non-DRX mode | Yes | Yes | Ericsson | Endorsed |
|  |  | Out-of-sync, in DRX mode | Yes | Yes | Ericsson | Endorsed |
|  |  | In-sync, in DRX mode | Yes | Low priority | Ericsson | Endorsed |
|  | Scheduling restrictions |  | Yes | Yes | Ericsson | Endorsed |
| Beam failure detection and link recovery | FR2-2 PCell | SSB-based in non-DRX mode | Yes | Yes | Nokia | Endorsed |
|  |  | SSB-based in DRX mode | Yes | Low priority | Nokia | Endorsed |
|  |  | CSI-RS-based in non-DRX mode | Yes | Low priority | CATT | Endorsed |
|  |  | CSI-RS -based in non-DRX mode | Yes | Yes | CATT | Endorsed |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Yes | Yes | CATT | Endorsed |
| SCell activation/deactivation delay | SCell in FR2 | intra-band in non-DRX | Yes | Yes | Ericsson | Submit to A.7 |
|  | FR1+FR2 inter-band with target SCell in FR2 |  | Yes | Yes | Ericsson | Submit to A.7 |
| PSCell addition and release delay | Known PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | No | Qualcomm | Submitted |
|  | Unknown PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes | Qualcomm | Submitted |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 | Reuse FR2-1 | Yes | Nokia | Endorsed |
|  | RRC based | PCell in FR2-2 | Reuse FR2-1 | Low priority | Nokia | Endorsed |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes | Yes | vivo | Endorsed |
|  |  | under DRX | Yes | Low priority | vivo | Endorsed |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Yes | Low priority | vivo | Endorsed |
|  |  | under DRX | Yes | Yes | vivo | Endorsed |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes | Yes | vivo | Available |
|  |  | PCell in FR2-2, DRX is used | Yes | Low Priority | vivo | Available |
|  |  | PCell in FR1, DRX is not used | Yes | Low Priority | vivo | Endorsed |
|  |  | PCell in FR1, DRX is used | Yes | Yes | vivo | Endorsed |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Yes | Low Priority | vivo | Available |
|  |  | PCell in FR2-2, DRX is used | Yes | Yes | vivo | Available |
|  |  | PCell in FR1, DRX is not used | Yes | Yes | vivo | Endorsed |
|  |  | PCell in FR1, DRX is used | Yes | Low Priority | vivo | Endorsed |
| L1-RSRP measurement for beam reporting | SSB based | DRX is used | Yes | Yes | Qualcomm |  |
|  | CSI-RS based | DRX not used | Yes | Yes | Qualcomm |  |

## 2.2	Test cases for RRM performance requirements with CCA

| Group of requirements | Test cases | Sub-test | Needed / Not needed | Company | Status |
| --- | --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 (CCA) | intra-frequency | Yes, at least with 120 kHz and 480 kHz | Ericsson |  |
|  |  | inter-frequency | Yes, at least with 120 kHz and 480 kHz | Ericsson |  |
| HO (delay and interruptions) | FR2-2->FR2-2 (CCA) | intra-frequency, unknown target cell | Yes | Huawei | Submitted |
|  | FR1->FR2-2 (CCA) | unknown target cell | Yes | Huawei | Submitted |
| RRC Re-establishment | FR2-2->FR2-2 (CCA) | inter frequency | Yes | Nokia | Submitted |
|  |  | intra frequency without serving cell timing | Yes | Nokia | Submitted |
| Random access | 4-step RACH | Contention based | Yes | Nokia | Submitted |
|  |  | Non-contention based | Yes | Nokia | Submitted |
| Radio link monitoring | FR2-2 Pcell (CCA), SSB-based | Out-of-sync, in non-DRX | Yes | Ericsson | Submitted |
|  |  | In-sync, in DRX mode | Yes | Ericsson | Submitted |
| Beam failure detection and link recovery | FR2-2 PCell (CCA) | SSB-based in non-DRX mode | Yes | CATT | Submitted |
|  |  | CSI-RS -based in non-DRX mode | Yes | CATT | Submitted |
| SCell activation/deactivation delay | SCell in FR2 (CCA) | intra-band in non-DRX | Yes | CATT | Submitted |
|  |  | inter-band in non-DRX | Low priority | CATT | Submitted |
|  | FR1+FR2 (CCA)inter-band with target SCell in FR2 |  | Yes | CATT | Submitted |
| PSCell addition and release delay | Unknown PSCell |  | Yes | Huawei | Submitted |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 (CCA) | Yes | Huawei | Submitted |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes | vivo | Submitted |
|  |  | under DRX | Yes | vivo | Submitted |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes | vivo | Submitted |
|  |  | PCell in FR1, DRX is used | Yes | vivo | Submitted |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is used | Yes | vivo | Submitted |
|  |  | PCell in FR1, DRX is not used | Yes | vivo | Submitted |