3GPP TSG-RAN WG4 Meeting # 105	R4-2220328

# Toulouse, France, November 14 – November 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on CG-SDT test cases in FR2 for RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Perf |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce RedCap UE test case for NR SDT in FR2 based on the latest CR R4-2220345 under SDT WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RedCap UE test case for NR SDT in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test requirements are defined for NR SDT of RedCap UE in FR2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.17.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

### A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap

Editor’s Note: the contents in the clause will be updated based on further agreement on the RedCap specific test configurations and the updates from latest FR2 SDT CR in R4-2220345.

#### A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap

##### A.17.2.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly performs TA validation for CG-SDT. This test will partly verify the TA validation requirements in clause 5.2B.2.1.

Supported test configurations are shown in table A.17.2.X.1.1-1. The test parameters are given in Tables A.17.2.X.1.1-2 and A.17.2.X.1.1-3.

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of 5 successive time periods, with time duration of T1 to T5 respectively. Figure A.17.2.X.1.1-1 shows the variation of the RSRP level in the active cell during the test.

Time duration T1 starts at time point TA and ends at time point TB. During T1 test equipment sets the transmit power to P0.

Time duration T2 starts at time point TB and ends at time point TD. At time point TC which is W1 after time point TB, test equipment sends RRCRelease message with CG-SDT configuration to UE and UE enters RRC_INACTIVE. During T2 test equipment sets the transmit power to P1.

Time duration T3 starts at time point TD and ends at time point TE. During T3 test equipment sets the transmit power to P0.

Time duration T4 starts at time point TE and ends at time point TI. Test equipment makes UL data arrival at UE lower layer at time point TF which is W2 after time point TE, and the details to configure UL data arrival is left to RAN5. After time point TF, test equipment observes whether UE transmits with CG-SDT within 640ms + Z after TF. The time point when UE transmits with CG-SDT is denoted as TG. At time point TH (FFS how TH is determined), the second RRCRelease is sent to UE. During T4 test equipment sets the transmit power to P2.

Time duration T5 starts at time point TJ and ends at time point FFS. During T5 test equipment sets the transmit power to P3. Test equipment triggers UL data arrival at UE lower layer at time point TJ, and the details to configure UL data arrival is left to RAN5.

Table A.17.2.X.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 120 kHz, data SCS 120KHz, BW 100 MHz |

Table A.17.2.X.1.1-2: General test parameters for TA validation for CG-SDT in FR2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 DD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 kHz |
| PRACH Configuration | Config 1 |  | Table A.3.8.3.4 |
| OCNG parameters |  |  | OP.5 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| DRX |  | s | 1.28 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | [8] |
| cg-SDT-TimeAlignmentTime |  |  | infinity |
| CG-SDT resource period |  | ms | 40 |
| T1 |  | s | [TBD] |
| T2 |  | s | [TBD] |
| T3 |  | s | [TBD] |
| T4 |  | s | [TBD] |
| T5 |  | s | [TBD] |

Table A.17.2.X.1.1-3: Cell specific test parameters TA validation for CG-SDT in FR2

| Parameter |  |  |  | Unit |  | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1, | T2 | T3 | T4 | T5 |
| AoA setup |  |  |  |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 4 |  |  |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  | dB |  |  |  |  |  |
|  |  | Config 1 |  | dBm/15kHz |  | [-98] |  |  |  |
|  |  | Config 1 |  | dBm/SCS |  | [-98] |  |  |  |
|  |  | Config 1 |  | dB | [0] | [10.35] | [0] | [15.25] | [6] |
|  |  | Config 1 |  | dB | [0] | [10.35] | [0] | [15.25] | [6] |
| SS-RSRP |  | Config 1 |  | dBm/SCS |  |  |  |  |  |
| Io |  | Config 1 |  | dBm/95.04 MHz |  |  |  |  |  |
| Propagation condition |  |  |  |  |  | AWGN |  |  |  |
|  |  |  | Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |

Figure A.17.2.X.1.1-1: RSRP variation for TA validation for CG-SDT

##### A.17.2.1.1.2	Test Requirements

The UE behaviour in each test during time durations shall be as follows:

During T4, UE shall transmit UL data with CG-SDT within 640ms + Z after time point TF.

During T5, UE shall not transmit UL data with CG-SDT.

The rate of correct events observed during repeated tests shall be at least 90%.

### <End of Change 1>