3GPP TSG-RAN WG4 Meeting # 105 										   R4-2220395

Toulouse, France, 14th-18th, Nov., 2022

Title:	WF on RLM/BFD relaxation for ePowSav

Agenda item:			10.2.2

Source:	vivo

Document for:	Approval

Backgroud

RAN2 LS R2-2209130 informed the following, RAN2 agreements and asked RAN4 to provide feedback if any:

- RLM/BFD relaxation and SCG deactivation with bfd-and-RLM configuration is true can be configured simultaneously. (Case 1) - In this case 1, UE will initiate UAI for the relaxation state report of RLM/BFD measurements for SCG, and this UAI for SCG RLM/BFD relaxation is reported over MCG. - RLM/BFD relaxation and SCG deactivation with bfd-and-RLM configuration is false can be configured simultaneously. (Case 2) - In this case 2, BFD and RLM is not operating, and thus BFD and RLM relaxation and the associated reporting can also be considered non-operational (regardless configuration).

Way Forward on RLM and BFD relaxation for deactivated PSCell

Sub-topic 2-1: Clarification on the RRM requirements for case 1

Issue 2-1-1: Whether to define RRM requirements for case 1?

- Agreements:

  - Option 1: No

    - The release 17 relaxed RLM/BFD requirements do not apply for SCG deactivated cells.

    - RAN4 not to define the relaxed RLM/BFD requirements for PSCell/SCells in deactivated SCG in Rel-17.

    - Send LS to RAN2 to inform the RAN4’s decision.