3GPP TSG-RAN WG4 Meeting #105-e	R4-2220397

Toulouse, France, November 14th – November 18th, 2022

Title: 	WF on NR FR2 HST RRM Core requirements impact

Agenda Item:	8.12.6

Source: 	Samsung

Document for:	Approval

# Sub-topic 2-1: RRM impact by multi-panel simultaneous reception

## Issue 2-1-1: Deployment scenario for FR2 HST multi-panel simultaneous reception

Agreements:

  - Candidate scenarios to identify RRM impact from multi-panel simultaneous reception:

    - Scenario #1: Bi-directional deployment in Scenario-A and Scenario-B

    - Note: Uni-directional deployment is not considered for further studies

Way Forward:

- It is noted that RF session still keep the possibility of multi-panel simultaneous reception in uni-directional deployment.

## Issue 2-1-2: RF chains for FR2 HST multi-panel simultaneous reception

Way forward:

- FFS the definition of simultaneous multi-panel operation in HST FR2.

- FFS HST FR2 specific issues in simultaneous multi-panel operation compared with Rel-18 NR FR2 multi-Rx chain DL reception WI

  - Candidate HST FR2 specific issues to be considered for the next meeting

    - Measurements

    - UE capability

    - Transmission scheme with mDCI or sDCI

    - UE architecture

    - Other issues are not precluded

Agreements:

- Rel-18 FR2 PC6 UE can be configured with multiple carriers even with multi-RX chains enabled.

- The configuration options are FFS:

  - Option 1: Rel-18 FR2 PC6 UE can be configured with multiple carriers even with multi-RX chains enabled, but multi-Rx chain is enabled only on one of the component carriers

  - Option 2: Rel-18 FR2 PC6 UE can be configured with multi-RX chains enabled on more than on component carrier.

## Issue 2-1-3: MRTD for FR2 HST multi-panel simultaneous reception

Agreements:

- For Rel-18 PC6 UE supporting simultaneous multi-panel operation, FFS MRTD of signals received on two panels on the same carrier can be extended to the value higher than the length of CP

  - FFS whether this UE behaviour/requirement depends on UE capability or not.

  - FFS whether depends on the deployment (e.g., Ds)

## Issue 2-1-4: Transmission Scheme for FR2 HST multi-panel simultaneous reception

Way forward:

- Transmission Scheme for FR2 HST multi-panel simultaneous reception:

  - RAN4 at least consider NC JT scheme in HST FR2 Enhanced deployments

  - FFS other schemes.

## Issue 2-1-5: UL and DL beam switching for FR2 HST multi-panel simultaneous reception

Way forward:

- FFS whether enhancements are needed in uplink spatial relation switch delay requirement.

  - Option 1: Keep current uplink spatial relation switch delay requirement

  - Other options are not precluded

    - Companies are encouraged to clarify the enhancement if other options are preferred.

# Sub-topic 2-2: RRM impact by CA

## Issue 2-2-1: Impacted RRM requirement for CA

Agreement:

- RAN4 shall not specify requirements for deactivated SCell in FR2 HST scenario.

Way forward:

- Whether basic CA requirements should be further enhanced can be discussed after more clarification is achieved.

  - Option 1 (Nokia):

    - RAN4, firstly, to specify CA RRM requirements in HST FR2 scenarios only for open space deployment and for PC6 UEs capable of single-panel reception, i.e., by specifying CA capability based on Rel-17 PC6 UEs.

    - RAN4 to discuss what is the minimal set of UE functionalities need for the support of CA in HST FR2 scenario and defined requirements only for those.

  - Other options are not precluded

- For Rel-18 FR2 PC6, by introducing CA operation, RRM impacts are expected at least for:

  - Measurement period for Inter-frequency measurement

  - L1-RSRP measurement requirements

- For Rel-18 FR2 PC6, the necessity of the following requirements shall be FFS:

  - PSS/SSS detection, Time index detection for Intra-frequency and inter-frequency measurements

  - FFS IDLE mode in inter-frequency measurement requirements

  - Beam failure recovery on SCell

  - TCI state switch in SCell.

  - Active SCell measurement requirement (measurement period)

## Issue 2-2-2: Measurement enhancement for CA

Way forward:

- FFS whether enhancement on CA also need to consider tunnel deployment scenario and multi-RX capabilities

  - FFS RX beam scaling factor for tunnel scenario

## Issue 2-2-3: Network signalling for Rel-18 FR2 HST CA Scenario

Way forward:

- HST signaling for CA enhancement:

  - Option 1: Reuse PCell signaling (i.e., Rel-17 HST FR2 flag with CA configuration from NW)

    - Option 1a (Nokia): Reuse PCell signalling partially, i.e., highSpeedMeasFlagFR2-r17 for HST FR2 scenario indication on SCell

  - Option 2 (Ericsson, Nokia): Define new signaling