3GPP TSG-RAN WG4 Meeting # 105												R4-2220402

Toulouse, France, November 14th – November 18th, 2022

Agenda item:			8.8.3.5

Source:	Moderator (Ericsson)

Title:	WF for FR2_multiRx_RRM_part3

Document for:	Approval

# Introduction

For this topic, moderator summary is captured in R4-2220063. Numbering of issues and subtopics are same as in moderator summary.

Background on the terminology:

Single DCI:

•	One DCI for two TCI states (PDSCH single DCI)

•	One MAC CE for two TCI states (PDCCH SFN)

Multiple DCI:

•	Two DCI one for each TCI state (PDSCH multiple DCI)

•	Two MAC CE one for each TCI state (PDCCH non-SFN)

# Agreements and way forward

## Sub-topic 1-1: General scope

Issue 1-1-1: Regarding group-based reporting and simultaneous reception with different QCL type-D

- Agreement

  - group based reporting is one of the means to enable simultaneous reception

- Candidate proposals for further study

  - Option 1: Simultaneous reception is not necessarily based on group-based reception

  - Option 2: TCI switch requirements are to specify dual TCI state switching requirements for multi-Rx (not necessary to limit to only simultaneous reception)

Issue 1-1-2: As per RAN1 agreements, group-based reporting is supported for single DCI only. If group-based reporting is a prerequisite, this AI can focus on single DCI case only.

- Candidate proposals for further study

  - Companies to check this RAN1 agreement and more discussion is needed during next meeting.

Issue 1-1-3: Definition of Dual TCI state switch

  - Dual TCI state switching refers to the case when two active TCI states with QCL type D for simultaneous reception of PDCCH/PDSCH are involved before and/or after TCI switching command.

  - In other words, following are considered as dual TCI state switching to discuss the requirements

    - Single DCI to Dual TCI

    - Dual TCI to Dual TCI

    - FFS: Dual TCI to Single TCI

## Sub-topic 1-2: sDCI based TCI state switching requirements.

Issue 1-2-1: TCI state switch indication method for PDCCH and PDSCH

  - PDCCH non-SFN

    - Only single PDCCH is supported. So Single MAC CE should be used

  - FFS: PDCCH SFN

    - One MAC CE is used to indicate both TCI states

  - PDSCH

    - For both SFN and non-SFN, only one DCI code point is indicated using single DCI and the code point may indicate one or two TCI states

## Sub-topic 1-4: known and unknown conditions.

Issue 1-4-1: Known and unknown states combination

- Dual TCI state switching requirements are defined for simultaneous reception. It is FFS whether we need to define any new requirements for non-simultaneous reception. The dual TCI states known, or unknown are FFS.

  - Option A: known and known

  - Option B: Known and unknown

  - Option C: unknown and unknown

For all other issues listed in moderator summary, there was no consensus reached in this meeting. Companies are encouraged to bring further analysis to next meeting