# 3GPP TSG-RAN WG4 Meeting # 105                                                     R4-2220484

# Toulouse, France, November 14 – November 18, 2022

Source:	ZTE Corporation

Title:	TP for TR 38.894 to include simultaneous Rx/Tx CA_n39-n41

Agenda item:			7.26.2

Document for:	Approval

1. Introduction

A text proposal for TR 38.894 [1] to add the include simultaneous Rx/Tx CA_n39-n41.

There were some discussions in last meeting, for cross-band isolation MSD, there are some proposals in [2]. However, during the discussion [3], some new values are proposed by the companies. We summarized in table 1.

Table 1. cross-band isolation MSD values in [2] and [3]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD |  |  |  |  | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |  |  |  |
|  |  |  |  |  |  |  |  | [2] | [3] | [4] |  | Average |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.6 | 3.4 | 3.2 | 3.0 | 3.3 | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2546 | 100 | 2.6 | 2.5 | 2.0 | 1.5 | 2.2 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.2 | 1.1 | 1.5 | 2.4 | 1.6 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1900 | 40 | 0.9 | 0.8 | 1.5 | 2.3 | 1.4 | >ACLR2 |

It can be seen that the proposed values from companies are close, and we propose to use the average values as above.

For the delta Tib,c/Rib,c values, the proposals on the values [3] from companies are the same.

2. Reference

[1] R4-2216659	TR skeleton for TR 38.xxx Simultaneous RxTx inter-band combination in Rel-18

[2] R4-2215899,  RF requirements on CA_n39-n41 supporting simultaneous Rx/Tx, ZTE

[3] R4-2217770, Email discussion summary for [104-bis-e][117] LTE_NR_Other_WI, Moderator(Huawei)

[4] R4-2220097, Topic summary for [105][117] LTE_NR_Other_WI,	Moderator (Huawei, HiSilicon)

2. Text Proposal

---Start of changes---

# 5.x	CA_n39A-n41A

## 5.x.1	Status of the band combination

### 5.x.1.1	Operating bands for CA

The operating bands for CA_n39-n41 are specified in Table 5.x.1.1-1.

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.x-1) |
| --- | --- |
| CA_n39-n41 | n39, n41 |

For CA_n39-n41, the current spec only considers scenarios without simultaneous Rx/Tx.

### 5.x.1.2	Maximum output power for inter-band CA

The UE Power Class for uplink CA_n39A-n41A are specified in Table 5.x.1.2-1.

Table 5.x.1.2-1 UE Power Class for uplink CA_n39A-n41A

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n39A-n41A | 23 | +2/-3 |

## 5.x.2	MSD analysis

Table 5.x.2-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n39-n41.

Table 5.x.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |

Based on above table, there is no harmonic issue for the band combination of n39 and n41.

Table 5.x.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |

Based on above table, there are UL3/DL4 order harmonic mixing issue for the band combination of n39 and n41, i.e.  3rd harmonic of band n41 UL may fall into 4th harmonic of band n39 DL.

## 5.x.3	Requirements for simultaneous Rx/Tx

For CA_n39-n41 supporting simultaneous Rx/Tx, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n39-n41 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n39-n41 | 0.2 | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

The cross band isolation MSD for CA_n39-n41 supporting simultaneous Rx/Tx are defined in table 5.x.3-3.

Table 5.x.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2546 | 100 | 2.2 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.6 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1900 | 40 | 1.4 | >ACLR2 |

The harmonic mixing MSD requirements for CA_n39-n41 supporting simultaneous Rx/Tx are FFS.

---End of changes---