3GPP TSG-RAN WG4 Meeting # 105														R4-2220736

Toulouse, France, November 14th – November 18th, 2022

Title: 	WF on Even Further RRM enhancement for NR and MR-DC – Part 2

Agenda Item:	8.9.4

Source: 	OPPO

Document for:	Approval

Sub-topic 1-1: General and scope

Issue 1-1: Applicability of the existing requirement for FR1+FR1 NR-DC

Agreement:

- For R16/17 and later requirement not listed in the WID,

  - not applicable to FR1-FR1 NR-DC, i.e., no UE requirement applies. Any extension of scope needs to be discussed in RAN-P.

- For R15 requirement not listed in the WID,

- not precluded in FR1-FR1 NR-DC and subject to WI stage discussion.

Sub-topic 1-2: PSCell addition/release delay and conditional PSCell addition delay

Issue 1-2: UE preparation time for FR1-FR1 NR-DC conditional Pscell additional delay

Agreement:

- TUE_preparation for FR1-FR1 NR-DC could keep the legacy value (10ms)

Sub-topic 1-3: Scheduling availability

Issue 1-3-1: Scheduling availability for RLM/BFD/CBD/L1-RSRP measurements

Agreement:

- For RLM/BFD/CBD/L1-RSRP when FR1+FR1 NR-DC is performed, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to RLM/BFD/CBD/L1-RSRP performed on FR1 serving PCell or PSCell in different bands

Issue 1-3-2: How to capture scheduling availability for RLM/BFD/CBD/L1-RSRP measurements for FR1-FR1 NR-DC in TS 38.133

Way forward:

- Discuss in CR stage

| Scheduling availability requirements for L1 measurement | Option 1: if specified in section of FR1-FR1 NR-DC, the impacted clauses are: | Option 2: if specified in existing section of FR1, the impacted clauses are |
| --- | --- | --- |
| RLM | 8.1.7.4 for all NR-DC cases (Note: the title may need updated) | 8.1.7.1, 8.1.7.2 for FR1 |
| BFD | 8.5.7.4 for all NR-DC cases (Note: the title may need updated) | 8.5.7.1, 8.5.7.2 for FR1 |
| CBD | 8.5.8.4 for all NR-DC cases (Note: the title may need updated) | 8.5.8.1, 8.5.8.2 for FR1 |
| L1-RSRP measurement | New section (as no dedicated clause for NR-DC case in current spec) | 9.5.6.1, 9.5.6.2 for FR1 |

Issue 1-3-3: Scheduling availability for intra-frequency measurement without MG

Agreement:

- The existing scheduling availability requirement for FR1 inter-band CA can be reused

- No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

- FFS whether to explicitly update the spec for FR1-FR1 NR-DC

- For UE who does not support simultaneousRxTxInterBandCA, FFS scheduling availability requirement of UE performing measurements on symbols colliding with PUSCH/PUCCH/SRS on FR1 for FR1-FR1 NR-DC scenario.

Issue 1-3-4: Scheduling availability for inter-frequency measurement without MG

Agreement:

- No need to introduce scheduling availability requirement of UE performing measurements in TDD bands for FR1-FR1 NR-DC scenario unless such requirement is introduced for FR1 inter-band CA.

- For UE who supports simultaneousRxDataSSB-DiffNumerology-Inter-r16, no need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

Sub-topic 1-4: Measurement availability

Issue 1-4-1: Measurement restriction

Way forward:

- FFS whether there is scenario that network configures mixed numerology on two CCs, while UE doesn’t support simultaneous reception for two CCs.

Issue 1-4-2: Measurement restriction requirements

Agreement:

- There is no measurement restriction for FR1-FR1 NR-DC scenario.

Sub-topic 1-5: CSSF within gap

Issue 1-5: CSSF within gap

Agreement:

- For CSSF within gap, update the grouping rule for FR1+FR1 NR-DC:

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is zero and the UE is configured with per UE gaps:

- 	FR1 intrafrequency measurement objects of MCG belong to group A

-      FR1 intrafrequency measurement objects of SCG belong to group B

Sub-topic 1-6: HO with PSCell

Issue 1-6: Additional uncertainty delay

Agreement:

- Not define HO with PSCell requirements for collision between PCell RACH and PSCell RACH occasion in FR1+FR1 NR-DC

Sub-topic 1-7: SCG activation/deactivation

Way forward:

- FFS:

  - Option 1 (Apple, MTK): For RACH-less based SCG activation for FR1+FR1 NR-DC,

    - if RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell.

    - there are no requirements if FR1 PSCell is unknown (i.e. follow same logic as FR1-FR2 NR-DC scenario).

  - Option 2 (Ericsson): For RACH-less based SCG activation for FR1+FR1 NR-DC,

    - if RLM and BFD are configured and TCI state is known, Tsearch = 0 ms

  - Option 3 (OPPO, vivo, Nokia): Hold on the discussion for Tsearch for RACH-less SCG activation after there are some progress or update in R17 SCG activation delay requirement.