<<START OF CHANGES>>

## 4.3	Specification Suffix Information

The following suffixes are defined at 2nd level for clauses 5, 6, and 7, as shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| A | Cat-M1 |
| B | NB1, NB2 |

The suffixes shall apply as defined in clause 4.2.

## 4.4	RF requirements in later releases

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

## 5.2	Operating bands

E-UTRA satellite access is designed to operate in the operating bands defined in Table 5.2-1.

Table 5.2-1 E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

## 5.2A	Operating bands for UE category M1

UE category M1 is designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1 in both half duplex FDD mode and full-duplex FDD mode.

## 5.2B	Operating bands for category NB1 and NB2

Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1.

Category NB1 and NB2 UE operate in HD-FDD duplex mode.

## 5.3	Channel bandwidth

## This clause is reserved.

### 5.3A	Channel bandwidth for category M1

The requirements in present document are specified for the channel bandwidth listed in Table 5.3A-1.

Table 5.3A-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

Figure 5.3A-1 shows the relation between the Channel bandwidth (BWChannel) and the Transmission bandwidth configuration (NRB). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Transmission

Center subcarrier (corresponds to DC in baseband) is not transmitted in downlink

Active Resource Blocks

Resource block

Transmission bandwidth configuration [NRB]

bandwidth [RB]

Channel bandwidth [MHz]

Figure 5.3A-1: Definition of channel bandwidth and transmission bandwidth configuration for one E-UTRA carrier

## 5.3B	Channel bandwidth for category NB1 and NB2

For category NB1 and NB2, requirements in present document are specified for the channel bandwidth listed in Table 5.3B-1.

Table 5.3B-1: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Figure 5.3B-1 shows the relation between the Category NB1/NB2 channel bandwidth (BWChannel) and the Category NB1 /NB2 transmission bandwidth configuration (Ntone). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Figure 5.3B-1 Definition of Channel Bandwidth and Transmission Bandwidth configuration

## 5.4	Channel arrangement

This clause is reserved.

## 5.4A	Channel arrangement for category M1

### 5.4A.1	Channel spacing

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent E-UTRA carriers is defined as following:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective E-UTRA carriers. The channel spacing can be adjusted to optimize performance in a particular deployment scenario.

### 5.4A.2	Channel raster, carrier frequency and EARFCN

The global frequency raster is defined for all frequencies. The granularity of the global frequency raster is 100 kHz, which means that the carrier centre frequency must be an integer multiple of 100 kHz. For each operating band, a subset of frequencies from the global frequency raster are applicable and forms a channel raster with a granularity ΔFRaster.

The carrier frequency in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in Table 5.4A.2-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in Table 5.4.2-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

The applicable channel raster and EARFCNs for each operating band are specified in Table 5.4A.2-1.

For operating bands with a channel raster of 100 kHz, every EARFCN within the operating band shall be applicable for the channel raster, and the step size for the channel raster in Table 5.4A.2-1 is given as <1>. The broadcast parameter earfcn-LSB defined in [TS36.331] may be used to assist the UE in synchronizing to the cell.

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 –<1>- 229375 | 1980 | 261844 | 261844 –<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 –<1>- 229075 | 1626.5 | 261504 | 261504 –<1>- 261843 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

### 5.4A.3	TX–RX frequency separation

The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.4A.3-1 for the TX and RX channel bandwidth defined in Table 5.3A.1-1.

Table 5.4A.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz |
| 255 | -101.5 MHz |

## 5.4B	Channel arrangement for category NB1 and NB2

### 5.4B.1	Channel spacing

Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz.

### 5.4B.2	Channel raster, carrier frequency and EARFCN

The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1.

The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 –262143. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

NOTE 1: Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification.

NOTE 2:	For the carrier including NPSS/NSSS for stand-alone operation, MDL = -0.5.

Editor’s note: FFS on MDL offsets values for non anchor carrier for DL.  The outcome for the MDL issue may mean revisiting some of the above text as necessary.

### 5.4B.3	TX–RX frequency separation

For UE category NB1/NB2 operation in stand-alone mode, the default TX-RX frequency separation shall be as specified in Table 5.4A.3-1 for the NB-IoT TX and RX channel bandwidth defined in Table 5.3B-1.

# 6	Transmitter characteristics

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

## 6.2	Transmit power

This clause is reserved.

## 6.2A	Transmit power for category M1

### 6.2A.1	UE maximum output power for category M1

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2A.1-1: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | +/-2 | 20 | +/-2 |
| 255 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.3.1-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: Applicable for category M1 HD-FDD UE |  |  |  |  |  |  |

The period of measurement shall be at least as defined in Table 6.2A.1-2.

Table 6.2A.1-2: Measurement period for UE maximum output power

| TTI pattern | Minimum measurement period |
| --- | --- |
| Subframe | 1ms |
| Slot | 7OS |
| Subslot | 2OS, 3OS |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

For each supported frequency band, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the IE P-Max as defined in TS 36.331 is not provided; or

-	if the IE P-Max as defined in TS 36.331 is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2A.4;

-	else (i.e the IE P-Max as defined in TS 36.331 is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2A.4;

### 6.2A.2	UE maximum output power reduction for category M1

Editor’s note: <Depends on outcome of SEM/ACLR discussion>

### 6.2.2B	UE maximum output power reduction for category NB1 and NB2

Editor’s note: <Depends on outcome of SEM/ACLR discussion>

### 6.2A.3	UE additional maximum output power reduction for category M1 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Editor’s note: < Further discussion is needed for outcome.>

### 6.2.3B	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

Editor’s note: < Further discussion is needed for outcome.>

### 6.2A.4	Configured transmitted Power for category M1

The configured transmitted power requirements in clause 6.2.5 of TS 36.101 shall apply, wherein

-	The Maximum output power requirements are specified in subclause 6.2A.1

-	The MPR requirements are specified in subclause 6.2A.2

-	The A-MPR requirements are specified in subclause 6.2A.3.

### 6.2.4B	Configured transmitted Power for category NB1 and NB2

For category M1 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 shall apply, wherein

-	The Maximum output power requirements are specified in subclause 6.2.1B

-	The MPR requirements are specified in subclause 6.2.2B

-	The A-MPR requirements are specified in subclause 6.2.3B.

## 6.2B	Transmit power for category NB1 and NB2

### 6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

For each supported frequency band, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the IE P-Max as defined in TS 36.331 is not provided; or

-	if the IE P-Max as defined in TS 36.331 is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2B.4;

-	else (i.e the IE P-Max as defined in TS 36.331 is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2B.4;

### 6.2B.2	UE maximum output power reduction for category NB1 and NB2

Editor’s note: <Depends on outcome of SEM/ACLR discussion>

### 6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

Editor’s note: < Further discussion is needed for outcome.>

### 6.2B.4	Configured transmitted Power for category NB1 and NB2

For category M1 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 shall apply, wherein

-	The Maximum output power requirements are specified in subclause 6.2B.1

-	The MPR requirements are specified in subclause 6.2B.2

-	The A-MPR requirements are specified in subclause 6.2B.3.

## 6.3	Output power dynamics

This clause is reserved.

## 6.3A	Output power dynamics for category M1

### 6.3A.1	UE Minimum output power for category M1

The minimum controlled output power of the UE is defined as the broadband transmit power of the UE, i.e. the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value.

The minimum output power is defined as the mean power in one sub-frame (1ms). The minimum output power shall not exceed the values specified in Table 6.3.2.1-1 of TS 36.101.

### 6.3.1B	UE Minimum output power for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.3.2F of TS 36.101 shall apply.

### 6.3A.2	Transmit OFF power for category M1

Transmit OFF power is defined as the mean power when the transmitter is OFF. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the UE is not considered to be OFF.

The transmit OFF power is defined as the mean power in a duration of at least one sub-frame (1ms) excluding any transient periods. The transmit OFF power shall not exceed the values specified in Table 6.3.3.1-1 of TS 36.101.

### 6.3.2B	Transmit OFF power for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.3.3F of TS 36.101 shall apply.

### 6.3A.3	ON/OFF time mask for category M1

The requirements for transmit ON/OFF time mask defined in clause 6.3.4 of TS 36.101 shall apply.

### 6.3.3B	ON/OFF time mask for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.3.4F of TS 36.101 shall apply.

### 6.3A.4	Power control for category M1

For category M1 UE, the requirements in clause 6.3.5E of TS 36.101 shall apply, wherein

-	The Maximum output power requirements are specified in subclause 6.2A.1

-	The Minimum output power requirements are specified in subclause 6.3A.1.

-	The requirements for configured transmitted power are specified in subclause 6.2A.4.

### 6.3.4B	Power Control for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.3.5F of TS 36.101 shall apply, wherein

-	The Maximum output power requirements are specified in subclause 6.2.1B

-	The Minimum output power requirements are specified in subclause 6.3.1B

-	The requirements for configured transmitted power are specified in subclause 6.2.4B.

## 6.3B	Output power dynamics for category NB1 and NB2

### 6.3B.1	UE Minimum output power for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.3.2F of TS 36.101 shall apply.

### 6.3B.2	Transmit OFF power for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.3.3F of TS 36.101 shall apply.

### 6.3B.3	ON/OFF time mask for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.3.4F of TS 36.101 shall apply.

### 6.3.4B	Power Control for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.3.5F of TS 36.101 shall apply, wherein

-	The Maximum output power requirements are specified in subclause 6.2B.1

-	The Minimum output power requirements are specified in subclause 6.3B.1

-	The requirements for configured transmitted power are specified in subclause 6.2B.4.

## 6.4	Transmit signal quality

This clause is reserved.

## 6.4A	Transmit signal quality for Category M1

### 6.4A.1	Frequency error for UE category M1

Table 6.4.1A-1: Frequency error requirement for HD-FDD UE category M1

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

Editor’s note: < Reuse 0.1 and 0.2 ppm requirements of frequency error and further discuss the condition where the requirements are applied. Further context will be added. Context consistent with 38.101-5 is considered. >

### 6.4.1B	Frequency error for UE category NB1 and NB2

Table 6.4.1B-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

Editor’s note: < Reuse 0.1 and 0.2 ppm requirements of frequency error and further discuss the condition where the requirements are applied. Further context will be added. Context consistent with 38.101-5 is considered. >

### 6.4A.2	Transmit modulation quality for category M1

Transmit modulation quality defines the modulation quality for expected in-channel RF transmissions from the UE. The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage

-	In-band emissions for the non-allocated RB

All the parameters defined in subclause 6.4A.2 are defined using the measurement methodology specified in clause Annex F of TS 36.101.

For category M1 UE, the requirements in clause 6.5.2E of TS 36.101 shall apply, and only QPSK and 16QAM in UL shall be applicable.

## 6.4B	Transmit signal quality for Category NB1 and NB2

### 6.4B.1	Frequency error for UE category NB1 and NB2

Table 6.4.1B-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

Editor’s note: < Reuse 0.1 and 0.2 ppm requirements of frequency error and further discuss the condition where the requirements are applied. Further context will be added. Context consistent with 38.101-5 is considered. >

### 6.4B.2	Transmit modulation quality for Category NB1 and NB2

Transmit modulation quality requirements for Category NB1 and NB2 UEs for BPSK and QPSK modulation as specified in clause 6.5.2F of 36.101 are applicable.

## 6.5	Output RF spectrum emissions

The output UE transmitter spectrum consists of the three components; the emission within the occupied bandwidth (channel bandwidth), the Out Of Band (OOB) emissions and the far out spurious emission domain.

Figure 6.5-1: Transmitter RF spectrum

## 6.5A	Output RF spectrum emissions for category M1

### 6.5A.1	General

### The definitions in clause 6.5 shall apply.

### 6.5A.2	Occupied bandwidth for category M1

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the 1.4MHz channel bandwidth specified in Table 6.6.1-1 of TS 36.101

### 6.5.1B	Occupied bandwidth for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.6.1F of TS 36.101 shall apply.

### 6.5A.3	Out of band emission for category M1

#### 6.5A.3.1	General

The out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an Adjacent Channel Leakage power Ratio.

#### 6.5A.3.2	Spectrum emission mask

Editor’s note: <Assume TN as baseline, and reconfirm after coexistence verification. >

#### 6.5A.3.2.3	Additional Spectrum Emission Mask for category M1

The additional spectrum emission mask is not applicable.

#### 6.5A.3.4	Adjacent Channel Leakage Ratio for category M1

Editor’s note: < Wait for coexistence verification outcome. >

### 6.5A.4	Spurious emission for category M1

#### 6.5A.4.1	Minimum requirements

Editor’s note: <To be added>

#### 6.5A.4.2	Spurious emission band UE co-existence

Editor’s note: <Further input needed but consider NR NTN and existing E-UTRA TN as baselines.>

#### 6.5A.4.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious

emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a

specific deployment scenario as part of the cell handover/broadcast message.

Editor’s note: < Further input needed given that there are additional spurious emissions requirements for band 255. >

## 6.5B	Output RF spectrum emissions for category NB1 and NB2

### 6.5B.1	General

The definitions in clause 6.5 shall apply.

### 6.5B.2	Occupied bandwidth for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 6.6.1F of TS 36.101 shall apply.

### 6.5B.3	Out of band emission for category NB1 and NB2

#### 6.5B.3.1	General

The out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an Adjacent Channel Leakage power Ratio.

#### 6.5B.3.2	Spectrum emission mask

Editor’s note: <Assume TN as baseline, and reconfirm after coexistence verification.>

#### 6.5B.3.3	Additional Spectrum Emission Mask for category NB1 and NB2

The additional spectrum emission mask for category NB1 and NB2 is not applicable.

#### 6.5B.3.4	Adjacent Channel Leakage Ratio for category NB1 and NB2

Editor’s note: < Wait for coexistence verification outcome.>

### 6.5.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements inline with SM.329 [2] and E-UTRA operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

### 6.5.3A	Spurious emission for category M1

#### 6.5.3A.1	Minimum requirements

Editor’s note: <To be added>

#### 6.5.3A.2	Spurious emission band UE co-existence

Editor’s note: <Further input needed but consider NR NTN and existing E-UTRA TN as baselines.>

#### 6.5.3A.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious

emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a

specific deployment scenario as part of the cell handover/broadcast message.

Editor’s note: < Further input needed given that there are additional spurious emissions requirements for band 255. >

### 6.5B.4	Spurious emission for category NB1 and NB2

Editor’s note: <To be added>

## 6.6	Transmit intermodulation

This clause is reserved.

### 6.6A	Transmit intermodulation for category M1

For category M1 UE, Tx intermodulation requirements are not applicable.

### 6.6B	Transmit intermodulation for category NB1 and NB2

For category NB1 and NB2 UE, the Tx intermodulation requirements in clause 6.7.1F of TS 36.101 shall apply.

# 7	Receiver characteristics

## 7.1	General

The requirements in clause 7.1 of TS 36.101 shall apply.

## 7.2	Diversity characteristics

The requirements in clause 7 assume that the receiver is equipped with single Rx port.

## 7.3	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category M1, category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

This clause is reserved.

### 7.3A	Reference sensitivity power level for UE category M1

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category M1, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes [A.2.2 and A.3.2] (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex [A.5.1.1]) with parameters specified in Table 7.3A-1 and Table 7.3A-2 for category M1.

Table 7.3A-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 255 | [-102.7] | FDD |
| 256 | [-102.2] | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101. |  |  |

Table 7.3A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 255 | [-103.5] | HD-FDD |
| 256 | [-103] | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101. |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3A-1/Table 7.3A-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3A-3.

NOTE:	Table 7.3.1A-3 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex [G](informative).

Table 7.3A-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 255 | 61 | FDD and HD-FDD |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1 in TS 36.102). |  |  |

### 7.3B	Reference sensitivity power level for UE category NB1 and NB2

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 Annex A.3.2 with received signal level as specified in Table 7.3B.1-1. Requirement in Table 7.3B.1-1 applies for any uplink configuration.

Table 7.3B.1-1: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

## 7.4	Maximum input level

This clause is reserved.

### 7.4A	Maximum input level for category M1

This is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD as described in TS 36.101 Annex A.5.1.1) with parameters specified in Table 7.4A-1.

Table 7.4A-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -402 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101. NOTE 2: Reference measurement channel is TS 36.101 Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 of TS 36.101. |  |  |

### 7.4B	Maximum input level for category NB1 and NB2

This is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

Category NB1 and NB2 UE maximum input level requirement is -40 dBm. For this input level the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2 of TS 36.101.

## 7.5	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

This clause is reserved.

### 7.5A	Adjacent Channel Selectivity for category M1

Editor’s note: <Depends on outcome of coexistence verification>

### 7.5B	Adjacent Channel Selectivity for category NB1 and NB2

Editor’s note: <Depends on outcome of coexistence verification>

## 7.6	Blocking characteristics

This clause is reserved.

## 7.6A	Blocking characteristics for category M1

### 7.6A.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

### 7.6A.2	In-band blocking requirements for category M1

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 Annex A.5.1.1) with parameters specified in Tables 7.6A.2-1 and 7.6A.2-2. For operating bands with an unpaired DL part (as noted in Table 5.2A-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6A.2-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.6A.2-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

### 7.6.1B	In-band blocking requirements for category NB1 and NB2

For category NB1 and NB2 UE, the requirements in clause 7.6.1.1F of TS 36.101 shall apply.

### 7.6.2	Out-of-band blocking

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied.

### 7.6A.3	Out-of-band blocking requirements for category M1

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5A and subclause 7.6A.2 shall be applied.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 Annex A.5.1.1) with parameters specified in Tables 7.6A.3-1 and 7.6A.3-2.

Table 7.6A.3-1: Out-of-band blocking parameters for category M1 UE

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 1.4 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 with PCMAX_L as defined in subclause 6.2.5. |  |  |

Table 7.6A.3-2: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

For Table 7.6A.3-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration. For these exceptions the requirements of subclause 7.7A spurious response are applicable.

### 7.6A.4	Narrow band blocking for category M1

This requirement is measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 Annex A.5.1.1) with parameters specified in Table 7.6A.4-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6A.4-1: Narrow-band blocking

| Parameter | Unit | Channel Bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Pw | dBm | PREFSENS + channel-bandwidth specific value below |
|  |  | 22 |
| Puw (CW) | dBm | -55 |
| Fuw (offset for f = 15 kHz) | MHz | 0.9075 |
| Fuw (offset for f = 7.5 kHz) | MHz |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101. NOTE 2: Reference measurement channel is specified in TS 36.101 Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 of TS 36.101. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as PREFSENS for Pw. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. NOTE 5: For DL category M1 UE, the parameter, Pw, for all the channel bandwidth will be PREFSENS +22 dBm. |  |  |

## 7.6B	Blocking characteristics for category NB1 and NB2

### 7.6B.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

### 7.6B.2	In-band blocking requirements for category NB1 and NB2

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

For category NB1 and NB2 UE, the requirements in clause 7.6.1.1F of TS 36.101 shall apply.

### 7.6B.3	Out-of-band blocking requirements for category NB1 and NB2

For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5B and subclause 7.6B.2 shall be applied.

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 Annexes A.3.2 with parameters specified in Table 7.6B.3-1.

For Table 7.6B.3-1 in frequency range 1, 2 and 3, up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7B spurious response are applicable.

Table 7.6B.3-1: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2562 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 3: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz.” NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

### 7.6B.4	Narrow band blocking for category NB1 and NB2

For category NB1 and NB2 UE, this is not applicable.

### 7.6.3	Narrow band blocking

This requirement is measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

For category NB1 and NB2 UE, this is not applicable.

#### 7.6.3.1	Minimum requirements

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 Annex A.5.1.1) with parameters specified in Table 7.6.3.1-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part. For category NB1 and NB2 UE, this is not applicable.

Table 7.6.3.1-1: Narrow-band blocking

| Parameter | Unit | Channel Bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Pw | dBm | PREFSENS + channel-bandwidth specific value below |
|  |  | 22 |
| Puw (CW) | dBm | -55 |
| Fuw (offset for f = 15 kHz) | MHz | 0.9075 |
| Fuw (offset for f = 7.5 kHz) | MHz |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101. NOTE 2: Reference measurement channel is specified in TS 36.101 Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 of TS 36.101. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3.1A-1 should be used as PREFSENS for Pw. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. NOTE 5: For DL category M1 UE, the parameter, Pw, for all the channel bandwidth will be PREFSENS +22 dBm. |  |  |

## 7.7	Spurious response

This clause is reserved.

### 7.7A	Spurious response for category M1

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in subclause 7.6A.2 is not met.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 Annex A.5.1.1) with parameters specified in Tables 7.7A-1 and 7.7A-2.

Table 7.7A-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101. N OTE 2: Reference measurement channel is specified in TS 36.101 Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 of TS 36.101. NOTE 3: The REFSENS power level is specified in Table 7.3A-1. |  |  |

Table 7.7A-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

### 7.7B	Spurious response for category NB1 and NB2

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in subclause 7.6B.2 is not met.

For category NB1 and NB2 UE, the minimum requirements in clause 7.7.1F of TS 36.101 shall apply for b255.

## 7.8	Intermodulation characteristics

Intermodulation response rejection is a measure of the capability of the receiver to receiver a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

### 7.8A	Intermodulation characteristics for category M1

The definition in clause 7.8 shall apply. The wide band intermodulation requirement is defined following the same principles using modulated E-UTRA carrier and CW signal as interferer.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 Annex A.5.1.1) with parameters specified in Table 7.8A-1 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8.A-1: Wide band intermodulation

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 12 |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 |  | 1.4 |
| FInterferer 1 (Offset) | MHz | -BW/2 –2.1 / +BW/2+ 2.1 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101. NOTE 2: Reference measurement channel is specified in TS 36.101 Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 of TS 36.101. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in TS 36.101 Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 with set-up according to Annex C.3.1. NOTE 4: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3.1A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 5: For DL category M1 UE, the parameters for the applicable channel bandwidth apply, and BW refers to the corresponding channel bandwidth. |  |  |

### 7.8B	Intermodulation characteristics for category NB1 and NB2

For category NB1 and NB2 UE, the definition in clause 7.8 and the requirements in clause 7.8.1F of TS 36.101 shall apply.

## 7.9	Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

The spurious emissions requirements in clause 7.9.1 of TS 36.101 shall apply.

<<END OF CHANGES>>