3GPP TSG-RAN WG4 Meeting # 105		R4-2218011

Toulouse, France, November 14 –November 18, 2022

3GPP TSG-RAN WG2 Meeting #119bis-e		R2-2211061

# Online, October 10-19, 2022

Title:	LS on RAN2 agreements about L1/L2-triggered mobility (LTM)

Response to:

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN2

To:	RAN1, RAN3, RAN4

Cc:

Contact Person:	Li-Chuan TSENG

E-mail Address:	li-chuan.tseng@mediatek.com

1. Overall Description:

Regarding L1/L2-based inter-cell mobility (Objective #1) in Rel-18 NR further mobility enhancement, RAN2 made the following agreements and assumptions at RAN2#119bis meeting on the terminology, target performance enhancements, L1 measurements, beam indication, RRC, and dynamic cell switch:

- Terminology RAN2 to use “LTM” as term for the L1/L2-triggered mobility. Use the term “cell switch” for the procedure of triggering change of cells via the LTM feature Use the term “Subsequent” LTM for the case when cell switch between L1/L2 mobility candidates is done without RRC reconfiguration in between. Target performance enhancements No security update support in Rel-18 with L1/L2 based mobility. FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration. FFS if this need to be specified. For UE processing, the following (not exhaustive) is assumed to be performed after receiving the cell switch command: - MAC/RLC reset (when configured) - RF retuning (e.g. needed for inter-frequency), baseband retuning R2 assumes that the following items may be discussed by RAN1 and RAN4 (and may be scenario specific): - - Whether to perform DL synchronization to candidate/target cell before receiving the cell switch command. R2 assumes this is feasible at least for the case that the target cell is already an active serving cell. - - Whether to support of performing TRS tracking and CSI measurement of candidate/target cell before/by cell switch command L1L2 based mobility supports the following CA scenarios: - PCell change without SCell change - PCell change with SCell change Support NR-DC scenario in L1L2 based mobility, at least for the PSCell change without MN involvement case, i.e. intra-SN. - L1 measurements and beam indication RAN2 assumes that RAN1 will drive discussions on L1 measurement enhancements, if any. If RAN1 identifies the need for e.g. event reporting, filtering etc, RAN2 can then be involved if needed. Inter-freq L1L2 mobility: R2 Confirms that For L1L2 mobility inter-freq scenarios in general should be supported (including mobility to inter-frequency cell that is not a current serving cell), including the support of inter-frequency L1 measurements, if feasible by R4 and R1. RAN2 assumes that whether to use the unified TCI framework as the baseline for beam indication for L1L2 mobility is up to RAN1 (RAN2 observes that L1/L2 mobility need to support inter-freq cases). - RRC A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates. RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported. - Dynamic cell switching RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined). RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress. R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed.

2. Actions

To RAN1, RAN3, and RAN4 groups

ACTION: 	RAN2 kindly asks RAN1, RAN3, and RAN4 to take the above information into consideration in their work on L1/L2-triggered mobility.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #120		14 – 18 November 2022		Toulouse FR

TSG RAN WG2 Meeting #121		27 February – 3 March 2023		Athens GR