3GPP TSG-RAN WG4 Meeting # 105			R4-2218022

Toulouse, France, November 14 –November 18, 2022

3GPP TSG-RAN WG1 Meeting #110b-e 	 R1-2210719

e-Meeting, October 10th – 19th, 2022

Title:	Reply LS on active TCI state list for UL TCI

Response to:	R4-2214972/R1-2208354

Release:	Release 17

Work Item:	NR_feMIMO-Core

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact Person:

#### Name:	Mihai Enescu

E-mail Address:	mihai.enescu@nokia.com

1. Overall Description:

RAN1 would like to thank RAN4 for the question in LS in R4-2214972 on active TCI state list for UL TC.

RAN1 has the following reply to the RAN4 questions:

Question 1: What is the relationship between qd and active UL (or joint) TCI state list? How does network configure the RS resource indexes qd?

Answer 1: According to 38.213, “the RS index for obtaining the downlink pathloss estimate for PUSCH,

PUCCH, and SRS transmission is provided by PL-RS associated with or included in the indicated TCIState or

UL-TCIstate except for SRS transmission that is not provided followUnifiedTCIstateSRS”

[...]

”if followUnifiedTCIstateSRS is not provided for a SRS resource set and for a SRS resource from the SRS

resource set, [...] a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by

PL-RS associated with or included in the TCIState or UL-TCIState of an SRS resource with

lowestSRS-ResourceId in the SRS resource set”

Question 2: Is UE expected to maintain the pathloss RSs of all the pathloss reference RS in the active UL (or

joint) TCI list?

Answer 2: A UE is expected to maintain the pathloss RSs of all active UL (or joint) TCI states. But the total

number of different path-loss RSs of all active UL (or joint) TCI states is not expected to be larger than 4.

Question 3: What is the UE behavior when number of active UL (or joint) TCI states is larger than 4 if

corresponding pathloss RSs need to be maintained?

Answer 3: The UE is not required to maintain more than 4 PL-RS. The same PL-RS can be used in

more than one UL or Joint TCI states.

2. Actions:

To RAN4 group.

ACTION: 	RAN1 requests RAN4 to kindly take the above information into account.

3. Date of Next TSG-RAN WG1 Meeting:

RAN1#111	November 14th – November 18th, 2022		Toulouse, FR

RAN1#112	February 27th – March 03rd, 2023			Athens, GR